3GPP TSG-RAN WG3#125bis 	R3-245042

Hefei, China, 14-18 October 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	TP (BL CR TS 38.401) Inter-CU LTM Signalling

Document for:	Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper provides TP for the baseline procedure considering contributions in R3-245040 and R3-245041.

# 2	TP for TS 38.401

### 8.9.X	Inter-gNB-CU LTM Handover

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 8.2.1.X-1 shows the inter-gNB-CU LTM procedure for intra-NR.

Figure 8.9.X-1: Inter-gNB-CU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells.

2.	The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the source gNB-CU.

3.	The source gNB-CU determines to initiate LTM configuration.

4.	The source gNB-CU issues a HANDOVER REQUEST message(s) to the target gNB-CU(s) with necessary information to prepare the handover at the target side.

5.	Admission Control is performed by the target gNB-CU(s).

6.	The target gNB-CU sends a UE CONTEXT SETUP REQUEST message to the target candidate gNB-DU(s) for each candidate cell, containing one target candidate cell ID, the LTM configuration ID of the candidate cell, and LTM configuration ID mapping list. The target gNB-CU may request PRACH resources from the candidate gNB-DU(s). The target gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

7.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

8.	The target gNB(s) sends a HANDOVER REQUEST ACKNOWLEDGE message including configurations of LTM candidate cells to the source gNB-CU.

9.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other gNB-DU(s). The source gNB-CU may send the updated CSI resource configuration to the source gNB-DU.

10.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

11.	The source gNB-CU sends a LTM CONFIGURATION UPDATE message to update the target gNB-CU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells.

Editor’s note: Details are FFS.

12.	The target gNB-CU a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

13.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the candidate gNB-DU.

14.	The target gNB-CU responds with a LTM CONFIGURATION UPDATE AKCNOWLEGDE message.

Editor’s note: Details are FFS.

15.	The source gNB-CU generates the RRC Reconfiguration.

16.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

17.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

18.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

19.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

20.	Early synchronization to the target candidate cell(s) may be performed as specified in TS 38.300 [2].

21.The candidate gNB-DU sends the TA value, the associated CFRA resource information, the candidate cell ID source gNB-DU ID, and the Source gNB ID to the source gNB-DU in the DU-CU TA INFORMATION TRANSFER message to the target gNB-CU.

22.The target gNB-CU sends the TA INFORMATION TRANSFER message including the TA related information received from the candidate gNB-DU to the Source gNB-CU.

23.	The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

24.	The UE sends the L1 measurement result to the source gNB-DU.

25.	The source gNB-DU decides to execute LTM to a candidate target cell.

NOTE: The Source gNB may initiate early data forwarding at an earlier timing than LTM cell change being triggered.

26.	The source gNB-DU sends the Cell Switch command to the UE.

27.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the source gNB-CU to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID.

28.	The source gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-CU in the LTM CELL SWITCH NOTIFICATION message.

29.	The target gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

30.	The source gNB-CU sends a SN STATUS TRANSFER message to the target gNB-CU.

31.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

32.	The target gNB-DU sends an ACCESS SUCCESS message to the source gNB-CU with the target cell ID.

33.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

34.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

35.	The target gNB-CU sends a HANDOVER SUCCESS message to the source gNB-CU.

Editor’s note: Subsequent LTM and its associated signaling and messages between source gNB-CU, target gNB-CU and the target candidate gNB-DU(s) to support it are FFS.

36. Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

37.	The target gNB-CU may send the UE CONTEXT RELEASE message to the source gNB-CU to release the resources of prepared cells.

38.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

39.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.