3GPP TSG-RAN WG3 Meeting #125bis		R3-245057

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP to NTN BL CRs) Support of regenerative payload

Agenda Item:	14.3

Document for:	Discussion and Decision

Introduction

In Rel-19 NR NTN WID [1], Regenerative Payload with full gNB on board is to be supported, detail objectives could be found below:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue discuss the potential issues on support of regenerative payload for NR NTN, our views will be provided accordingly.

Discussion

NG connection management

The last RAN3 meeting has discussed how to manage NG connections when the on-board gNB enters or leaves the service area of the current AMF, and the candidates are as follows：

- Option 1: Legacy procedure, e.g. SCTP SHUTDOWN

- Option 2: NG Removal/Setup

- Option 3: NG suspend/resume

- Option 4: Both Op2 and Op3.

According to the discussion in the last RAN3 meeting, option 1 is workable without any specification impact, however, it’s not efficient. And RAN3 has discussed and agreed to specify the option 2, i.e. introduce NG Removal procedure. The TPs on support of NG Removal could be found in [4][5][6], RAN3 is requested to discuss and agree the TPs.

Proposal 1: Discuss and agree the TPs [4][5][6] to reflect the agreement on supporting of NG Removal procedure for NR NTN.

For the option 3, introduce a suspend/resume mechanism for NGAP management. When the on-board gNB coverage leaves the AMF service area, the NG connection is suspended, indicating that the current NG connection is unavailable, but the actual logical interface still exists. When the on-board gNB come back to the AMF service area, the NG connection could be resumed quickly. Due to the mobility of on board gNB, the TNL over the NG interface will frequently disconnect and re-connect. Solution#2 in TS 23.700-29 mentioned that:

The solution proposes to suspend the TNL between the RAN node and the CN if the RAN node attempts to disconnect the backhaul connection towards the AMF/MME in a PLMN. When the RAN node decides that it cannot serve the current PLMN any longer, it should indicate the AMF/MME to suspend the N2/S1-AP connection and store the context for resuming the N2/S1-AP connection in the future. Both the RAN node and the AMF/MME store the RAN node context information for resuming the N2/S1-AP connection. The RAN node and the 5GC should further adjust the heartbeat timer of the SCTP over N2/S1-AP interface to avoid heartbeat time out.

In this solution, on board gNB needs to store the non-UE context information for resuming NGAP connection. While some companies express concerns about limited storage space on satellites, we believe that the above non-UE context information is only the basic context of NGAP and the amount of data is not large. For example, in TN system, gNB can establish NGAP connections with all AMFs in one or more AMF sets and store the corresponding context. Therefore, there is no problem for on board gNB to store multiple NGAPs contexts for NGAP suspend/resume.

Observation 1: There is no problem for on board gNB to store multiple NGAP contexts for NGAP suspend/resume.

NG suspend and resume solution can provides a faster NG connection mechanism and reduce the frequent TNL disconnection and re-connection. Therefore, we think that both option 2 and option 3 can be supported, and the two NG Removal/Setup and NG suspend/resume can be used flexibly or can be used in combination. For example, if on board gNB may not return to the service area of an AMF for a long time in the future, it can choose NG Removal procedure. If on board gNB return to the service area of an AMF in a shorter period of time, it can choose NG suspend procedure.

It’s possible to reuse the RAN CONFIGURATION UPDATE procedure with necessary enhancement, e.g. add the suspend/resume indicator. To minimize the specification impact, it is also possible to define new procedures to support both NG removal and NG suspend/resume solution, which could using different code-points to indicate NG removal or NG suspend/resume.

As above, the corresponding stage 3 TP for 38.413 BL CR is provided in section 7. RAN3 is requested to discuss and agree the TP.

Proposal 2: Besides NG Removal/Setup, it is proposed to support NG suspend/resume for NR NTN NG management. RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection. Corresponding TP for 38.413 BL CR is provided in appendix 7.

Supported TAI list of the on-board gNB

In current NGAP specification, gNB provides the supported TA list information to the AMF. And based on the TA list of the UE registration area and the supported TA list of gNB, the AMF selects the appropriate gNB to send paging messages to the UE. For ground gNB, the coverage area of the Tracking Area remains fixed. Once the gNB is deployed onto a satellite, such as MEO or LEO satellites, the mobility of the on board gNB causes the coverage of the gNB to change over time, and the supported TAIs list also changes accordingly. Existing procedure .i.e. RAN CONFIGURATION UPDATE can be reused. But for traditional procedure, frequent updates of the supported TAIs list will cause a large number of NG interface signaling interactions.

Observation 2: Existing procedure i.e. RAN CONFIGURATION UPDATE can be reused to provide the supported TAI list of the on-board gNB to the AMF, but it will cause a large number of NG interface signaling interactions, which is not efficient.

SA2 has already discussed how to manage the dynamic list of supported TAIs. The solutions are as follows in current TR 23.700-29[3]:

- TA "timetable“ is calculated per RAN node by AMF based on satellite ephemeris (Solution #2)

- TA "timetable“ is provided to AMF per RAN node ID (Solution #4)

Observation 3: In the TR 23.700-47, both Signalling based solution and OAM based solution are captured for updating the supported TAI list of the on-board gNB.

In R17/18, we’ve ever discussed exchange of served cells and neighbor cell relations between the gNBs. At that time, we concluded that the served cell information and its neighbour relations could be left to OAM. Similar with the neighbor relationship issue, we slightly preferred to go for OAM based solution to reduce the frequency of interactions. Which means the information of supported TAIs is provided to AMF per RAN node ID by OAM configuration. In addition, it is proposed to send LS to SA2 and SA5 on OAM requirement for supported TAIs and the draft LS is provided in appendix 5.

Proposal 3: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. It’s proposed to send LS to SA2 and SA5 about our decision, the draft LS is provided in appendix 5.

Feeder link switch

The last RAN3 meeting has discussed two cases of feeder link switch, and the conclusions are as follows:

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

We will focus on the case, which the AMF and the on board have not actually changed, only the gateway has changed, so it can be considered that the logical connection of NG has not changed.

Observation 4: For the above case, the logical connection of NG between on-board gNB and the on ground AMF is not changed before and after feeder link switch.

And in the last RAN3 meeting, some companies mentioned how to solve the packet loss problem in hard feeder link switch scenarios. For the above problem, 5GC may not know the hard feeder link switch, and the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. But in our perspective, RAN3 can assume that both NG-RAN and CN know when the old feeder link is disconnected and the new feeder link is established. The activation and disconnection of the old and new TNLA can be based on the status of the feeder link. For example, CN can continue to use the old TNLA until the old feeder link and TNLA is disconnected, and then use the new TNLA after the new TNLA is successfully connected.

Observation 5: RAN3 assumes that both NG-RAN and CN know when the old feeder link is disconnected and the new feeder link is established.

For the above case, the UE is connected at the old gateway, and TNLA(s) are established with the AMF for signaling transmission of the control plane. When the on board gNB moves out of the connection range of the old gateway, it will establish a feeder link with the new NTN-GW. In the legacy procedures, TNLA could be added, removed and updated.

Before feeder link switch, it is necessary to establish a new TNLA for the NGAP for the new feeder link. The establishment of the new TNLA for the new feeder link can be pre-configured. But in our perspective, it may be need some enhancements for the NGAP signaling. It is necessary for AMF to indicate the usage of the TNLA, indicating that the TNLA is used for new feed link or feeder link switch.

Proposal 4: A new TNLA is needed for the new feeder link between on-board gNB and AMF, the AMF should indicate on-board gNB when the new TNLA is applied, e.g. indicate gNB the usage of a TNLA is for new feeder link.

Intra cell Inter AMF mobility

In previous RAN3 meeting, some companies mentioned that how satellite provides the continuous service link to the UE when inter-AMF mobility occurred due to the movement of a UE. Some companies proposed that the logical gNB on board the satellite providing the service link to UE is changed i.e. the same physical gNB implements two logical gNBs. In our opinion, it is not necessary to use two logical gNBs and we propose to use the legacy procedure .i.e. NG based HO to support it.

We assumed that AMF service consists with a list of TAC and the coverage of each TAC is clear to gNB. A single NTN cell may cover multiple TACs, especially those in different AMFs. When UE moves in intra cell, gNB can recognize the change of TAC according to the coarse location of UE and the change of AMF according to TAC, and then initiate the NG HO procedure. GNB sends Target ID set as the new TAC information to the source AMF via HANDOVER REQUIRED message, so that the source AMF can correctly route to the target AMF according to the TAC.

Observation 6: Existing NGAP HO procedure could be reused to support intra cell inter AMF HO, with the assumption that the gNB could deduce the mapping between TACs and AMFs.

However, in current specification, it’s not clear whether gNB knows the AMF service area. We understand that the mapping between TACs and AMFs service area can be pre-configured to gNB and AMF. We propose to send the LS to confirm our assumptions with SA2 and SA5. Accordingly, we provide a simple TP to BLCR for TS 38.300 on NTN OAM requirements.

Proposal 5: The mapping between TACs and AMF could be pre-configured to gNB and AMF.

Proposal 6: Discuss and agree the TP for BLCR on TS 38.300 in appendix 6, and send LS to SA2 and SA5 about the OAM requirement (draft LS in Appendix 5).

Other mobility aspects

2.5.1 Support of RRC_INACTIVE UE

In last RAN3 meeting, some companies mentioned that RAN3 needs to discuss how to support RRC_INACTIVE UE in NTN regenerative payload. The last meeting concluded as follows:

WA: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

According to WA, current mechanisms can be reused to support RRC_INACTIVE UE, depending on implementation and deployment. RAN3 does not expect enhancements to support RRC_INACTIVE UE and it is proposed to turn WA into agreement.

Proposal 7: On support of RRC_INACTIVE, it’s proposed to turn WA into agreement, no specification impact is expected from RAN3 perspective.

2.5.2 Location based HO over XnAP/NGAP

R18 has already enhanced the Xn interface to support Time based CHO. Handover with Time-based trigger condition and Location-based trigger condition are specified in Stage-2 TS 38.300. In the previous RAN3 meeting, some companies proposed to enhance location based HO. For inter-gNB mobility (between on board gNBs), it is proposed that RAN3 consider whether and how to better support location based HO via the Xn interface. For example, source NG-RAN can transfer the coarse UE location information or the condition of location based CHO to target NG-RAN in the Handover Request. Thus the target NG-RAN can decide when to reserve resources based on this information.

We can focus on XnAP first, and adopt it to NG if needed.

Proposal 8: For location based CHO, it is proposed to transfer some information from the source NG-RAN node towards the target NG-RAN node, e.g. the coarse location of the UE, for better resource handling.

Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

NG connection management

Proposal 1: Discuss and agree the TPs [4][5][6] to reflect the agreement on supporting of NG Removal procedure for NR NTN.

Observation 1: There is no problem for on board gNB to store multiple NGAP contexts for NGAP suspend/resume.

Proposal 2: Besides NG Removal/Setup, it is proposed to support NG suspend/resume for NR NTN NG management. RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection. Corresponding TP for 38.413 BL CR is provided in appendix 7.

OAM requirement for supported TAI list of a gNB on board

Observation 2: Existing procedure i.e. RAN CONFIGURATION UPDATE can be reused to provide the supported TAI list of the on-board gNB to the AMF, but it will cause a large number of NG interface signaling interactions, which is not efficient.

Observation 3: In the TR 23.700-47, both Signalling based solution and OAM based solution are captured for updating the supported TAI list of the on-board gNB.

Proposal 3: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. It’s proposed to send LS to SA2 and SA5 about our decision, the draft LS is provided in appendix 5.

Feeder link switch

Observation 4: For the above case, the logical connection of NG between on-board gNB and the on ground AMF is not changed before and after feeder link switch.

Observation 5: RAN3 assumes that both NG-RAN and CN know when the old feeder link is disconnected and the new feeder link is established.

Proposal 4: A new TNLA is needed for the new feeder link between on-board gNB and AMF, the AMF should indicate on-board gNB when the new TNLA is applied, e.g. indicate gNB the usage of a TNLA is for new feeder link.

Intra cell Inter AMF mobility

Observation 6: Existing NGAP HO procedure could be reused to support intra cell inter AMF HO, with the assumption that the gNB could deduce the mapping between TACs and AMFs.

Proposal 5: The mapping between TACs and AMF could be pre-configured to gNB and AMF.

Proposal 6: Discuss and agree the TP for BLCR on TS 38.300 in appendix 6, and send LS to SA2 and SA5 about the OAM requirement (draft LS in Appendix 5).

Other mobility aspects:

Proposal 7: On support of RRC_INACTIVE, it’s proposed to turn WA into agreement, no specification impact is expected from RAN3 perspective.

Proposal 8: For location based CHO, it is proposed to transfer some information from the source NG-RAN node towards the target NG-RAN node, e.g. the coarse location of the UE, for better resource handling.

Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3

- RAN3_125_agenda_20240823EOM

- TR 23.700-47 Study on integration of satellite components in the 5G architecture

- R3-245362  (TP for TS 38.410) Support for Regenerative Payload in NR NTN

- R3-245179  (TP for TS 38.413) Introduce NG Removal procedure

- R3-24xxxx (TP for TS 38.300) NG Removal – Stage 2 Description

- Draft LS to SA2, SA5 LS on OAM requirements

Title:	[Draft] LS on OAM requirements to support regenerative payload

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	CATT (to be RAN3)

To:	SA2, SA5

Cc:

Contact Person:

#### Name:	Jiancheng Sun

#### E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 discussed the potential issues on support of regenerative payload for NR NTN and identified the following candidate mechanisms, which need to be confirmed by SA2 and SA5:

- The mobility of the on board gNB causes the coverage of the gNB will changed with time, accordingly the supported TAIs list of the gNB will change with time. Frequent updates of the supported TAIs list using the legacy RAN Configuration Update procedure may cause a large number of NG interface signalling interactions.

RAN3 prefers to consider OAM based solution, the information of supported TAIs is provided to AMF per RAN node ID by pre-configuration or OAM configuration.

- RAN3 assumes the AMF service area is consist of a list of TAC, RAN could trigger inter AMF HO in case the UE moves across the TACs of the different AMFs.

RAN3 would like to confirm with SA2 and SA5 if the assumption is correct. If yes, the relationship between AMF and TACs should be configured to the gNB via OAM.

2. Actions:

To SA2, SA5:

ACTION: RAN3 respectfully requests SA2 and SA5 to check the RAN3 assumptions as above, and response accordingly.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#126	       Nov 18th – Nov 22nd	2024		Orlando, US

RAN3#127	       Feb. 17th – Feb. 21st  	2024		Athens, GR

- TP for BLCR on 38.300 OAM requirements

-----------------Start of the Changes-------------------

16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

-	The mapping between TAC(s) and AMF service area;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

-----------------End of the Changes-------------------

TP for BLCR on 38.413 Support of NG Suspend/Resume

-----------------Start of the Changes-------------------

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the NG Suspend Indicator IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information for suspending TNL of NG interface.

If the NG Resume Indicator IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information for resuming TNL of NG interface.

Editor’s Note: The procedure texts could be refined.

#### 8.7.2.3	Unsuccessful Operation

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation

If the AMF cannot accept the update, it shall respond with a RAN CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node shall wait at least for the indicated time before reinitiating the RAN Configuration Update procedure towards the same AMF.

-----------------Next change-------------------

#### 9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| NG Suspend Indicator | O |  | 9.3.2.X |  | YES | reject |
| NG Resume Indicator | O |  | 9.3.2.Y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

#### 9.2.6.5	RAN CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the AMF to acknowledge the NG-RAN node transfer of updated information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.6.6	RAN CONFIGURATION UPDATE FAILURE

This message is sent by the AMF to indicate RAN configuration update failure.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to Wait | O |  | 9.3.1.56 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Next change-------------------

#### 9.3.2.X	NG Suspend Indicator

This IE is used to provide the information for suspending TNL of NG interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG Suspend (FFS) |  |  |  |  |

Editor’s Note: The IE naming and content are FFS.

#### 9.3.2.Y	NG Resume Indicator

This IE is used to provide the information for resuming TNL of NG interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG Resume (FFS) |  |  |  |  |

Editor’s Note: The IE naming and content are FFS.

-----------------End of Change-------------------