3GPP TSG-RAN WG3 Meeting #125bis                             	  R3-245058

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP to TR 38.769) A-IoT architecture aspects

Agenda Item:	16.2

Document for:	Discussion and Decision

- Introduction

In the previous RAN3 meetings, we discussed the architecture aspects for A-IoT [1], corresponding agreements and open issues are as follows:

Two deployment system architecture figures for Topology1 and Topology2

Protocol stack, to be continued.

For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting. To be continued...

In this contribution, we will further discuss the architecture aspects on support of A-IoT, and provide the observations and proposals accordingly.

- Discussion

About the protocol stack for topology 1, we captured the following figure in the pCR the last meeting:

Figure 6.3.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

- Figure 6.3.1-2. Protocol Stack for Topology 1

    - Editor’s Note 1: Figure 6.3.1-2 serves as a starting point for further discussions.

For topology 1, the XXAP is terminated at an AIoT RAN node.

Editor’s Note 2: the signalling transport for XXAP is FFS.

Editor’s Note 3: The protocol stack does not detail how AIoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the AIoT CN may include AMF and AIoT related functions which is also up to SA2 decision.

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the AIoT RAN node functions are FFS.

The protocol stack as above is taken as starting point, however, the details are to be further discussed.

In above figure, protocol stack for AIoT Radio interface is pending to RAN2, to be further updated by RAN2.

Proposal 1: Protocol stack for AIoT Radio is pending to RAN2.

For the XX interface, what functionalities are included in AIoT CN (e.g. AMF, and or AIoT related functions) to be decided by SA2.  We understand the protocol stack is good enough from RAN3 perspective.

Proposal 2: Protocol stack for XX as specified in Figure 6.3.1-2 is good enough from RAN3 perspective.

XX-AP provides at least the following functions:

-	Inventory;

-	Command;

Editor’s Note: Further details of XXAP can be further discussed and confirmed.

Proposal 3: XX-AP provides at least Inventory and Command functions, more details could be further discussed and updated later.

For the topology 2, the logical system architecture is captured as below figure:

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an AIoT-enabled UE, and the AIoT RAN node function is located at an AIoT-enabled gNB.

The following definitions apply:

AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface.

AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

Figure 6.3.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between AIoT enabled UE and AIoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The AIoT CN could include AMF and AIoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The AIoT enabled gNB performs radio resource management for AIoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

However, there’re still some open issues pending to RAN2, SA2.

For EN1, the detail protocol stack between AIoT enabled UE and AIoT CN are pending to the final solutions, i.e. RRC based solution, NAS based solution, UP based solution. Details could be further updated in 6.3.2.1, the EN could be removed.

For EN2, similar to the topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface. The EN could be removed.

For EN3, what will be included in AIoT CN, AMF, and or AIoT related functions are pending to SA2. It’s good enough to use AIoT CN from RAN perspective, we can remove the EN directly.

For EN4, the definition of AIoT enabled gNB is clear, no need to further specify how to perform radio resource management.

Proposal 4: Remove the Editor’s notes for topology 2 architecture.

For the solutions for Topology 2, RRC based solution, NAS based solution, UP based solution, the protocol stack and corresponding details are provided in [3].

Proposal 5: Discuss and agree the TP for Topology 2 solutions in [3].

For Top 1 and Top 2, the XX interface between AIoT RAN and AIoT CN could be:

- Option 1: Reuse NGAP with necessary enhancements, e.g. supporting Inventory, Command procedures.

- Option 2: Define a new interface xx, between AIoT RAN and AIoT CN, direct connection.

- Option 3: Define a new interface xx, between AIoT RAN and AIoT CN, NRPPa like, routing via N2.

For the Option 1, no need to define the new interface, AIoT related signallings and IEs are introduced to the NGAP, not too much impact is foreseen.

For the option 2, AIoT RAN could directly connect to the “AIoT Function” in the CN (e.g. AIoTF), AMF is not involved for the topology 1.

The option 3 is similar to the option 2, the difference is the XX interface is routing via AMF, via the NGAP, just like the NRPPa, or SMF container.

As above, all of the 3 options are feasible. If the “AIoT function” in the 5GC is part of AMF, or tightly connected to AMF, option 1 seems better. If “AIoT Function” and AMF are deployed separately, option 2 or 3 could be considered. As the relationship between the AIoT Function (AIoTF) and AMF to be discussed and decided by SA2, which option to be taken need to be checked with SA2, whether and how the AMF is involved to support AIoT functions.

Proposal 6: Send LS to SA2 to check whether and how AMF is involved for A-IoT.  (Draft LS is in the Annex 6)

For the Topology 2, the intermediate node is a UE, NGAP is used to connect to the AMF for the UE, and some enhancement to the NGAP is needed, e.g. authorization of the UE reader.

Considering the commonalities of Topology 1 and Topology 2, it’s easier to use NGAP to connect between A-IoT RAN/AIoT enabled gNB and A-IoT CN.

Proposal 7: Considering the commonalities of Topology 1 and 2, it’s slightly preferred to use NGAP between A-IoT RAN node/AIoT-enabled gNB and AIoT CN, with necessary enhancement.

To allow the flexibility, the AIoT RAN node in Topology 1, or AIoT-enabled gNB in Topology 2 may also serve the legacy NR UEs. However, there’s no extra specification impact to support the legacy NR UEs over NR-Uu, NGAP.

Proposal 8: AIoT RAN node or AIoT-enabled gNB may also support the legacy NR UEs, without any extra impact to NR-Uu and NGAP.

Base on the discussion above, we provide the TP to TR 38.769 in section 5. It’s proposed to discuss and agree the TP to TR 38.769 on system architecture aspects to support A-IoT.

Proposal 9: Discuss and agree the TP to TR 38.769 (in section 5) on AIoT system architecture aspects.

- Conclusion

Based on the discussion in section 2, the following proposals are provided:

Proposal 1: Protocol stack for AIoT Radio is pending to RAN2.

Proposal 2: Protocol stack for XX as specified in Figure 6.3.1-2 is good enough from RAN3 perspective.

Proposal 3: XX-AP provides at least Inventory and Command functions, more details could be further discussed and updated later.

Proposal 4: Remove the Editor’s notes for topology 2 architecture.

Proposal 5: Discuss and agree the TP for Topology 2 solutions in [3].

Proposal 6: Send LS to SA2 to check whether and how AMF is involved for A-IoT.  (Draft LS is in the Annex 6)

Proposal 7: Considering the commonalities of Topology 1 and 2, it’s slightly preferred to use NGAP between A-IoT RAN node/AIoT-enabled gNB and AIoT CN, with necessary enhancement.

Proposal 8: AIoT RAN node or AIoT-enabled gNB may also support the legacy NR UEs, without any extra impact to NR-Uu and NGAP.

Proposal 9: Discuss and agree the TP to TR 38.769 (in section 5) on AIoT system architecture aspects.

- Reference

- RAN3#125 Chair Notes

- R3-243972 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR

- TP to TR 38.769 AIoT architecture aspects

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

## 6.3	RAN architecture aspects

Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for AIoT consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAN: hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and AIoT RAN node in topology 1 and between AIoT device and AIoT-enabled UE in topology 2.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN.

NOTE: the details of AIoT CN are subject to SA2.

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function that contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

### 6.3.1 Support of Topology 1

Figure 6.3.1-1 depicts a logical system architecture for topology 1, where the Common reader function and AIoT RAN node function are deployed within an AIoT RAN.

Figure 6.3.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.3.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

- Figure 6.3.1-2. Protocol Stack for Topology 1

For topology 1, the XXAP is terminated at an AIoT RAN node.

XX-AP provides the following functions:

-	Inventory;

-	Command;

Editor’s Note1: Details of XXAP can be further discussed.

Editor’s Note 2: Protocol stack and corresponding details for AIoT Radio is pending to RAN2.

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an AIoT-enabled UE, and the AIoT RAN node function is located at an AIoT-enabled gNB.

The following definitions apply:

AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface.

AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

Figure 6.3.2-1 Logical system architecture for topology 2

In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- Draft LS to SA2

Title:	[Draft] LS on Clarification of architecture aspects for Ambient IoT

Response to:	-

Release:	Rel-19

Work Item:	FS_AmbientIoT

Source:	CATT (to be RAN3)

To:	SA2

Cc:	RAN1, RAN2

Contact Person:	Jiancheng Sun

E-mail Address:	sunjiancheng (at) catt (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

While progressing the study on Ambient IoT, RAN3 discussed the system architectures on support of Ambient IoT. The system architecture for Topology 1 agreed by RAN3 is illustrated with the figure below:

Logical system architecture for topology 1

Logical system architecture for topology 2

In the above system architectures, the “XX” interface is used to connect between AIoT RAN/AIoT-enabled gNB and AIoT CN, three potential options are assumed for XX interface, as follows:

- Option 1: Reuse NGAP with necessary enhancements, e.g. supporting Inventory, Command procedures.

- Option 2: Define a new interface XX, between AIoT RAN and “AIoT Function” in the Core Network, direct connection, AMF is not involved.

- Option 3: Define a new interface XX, between AIoT RAN and “AIoT Function” in the Core Network, NRPPa like, routing via AMF, XXAP is carried by NGAP.

We see SA2 is also discussing the A-IoT overall architectures, RAN3 would like to ask SA2 the following questions:

Question 1: Whether and how AMF is involved to support A-IoT functions?

Question 2: Which option (as above) is preferred to support the “XX” interface between AIoT RAN node and AIoT CN?

2	Actions

To SA2:

ACTION: RAN3 kindly asks SA2 to provide the feedback to the above questions on Ambient IoT architecture aspects.

3	Date of next RAN3 meetings

RAN3#126	       Nov 18th – Nov 22nd	2024		Orlando, US

RAN3#127	       Feb. 17th – Feb. 21st  	2024		Athens, GR