3GPP TSG-RAN WG3 #125bis                                          R3-245060

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP to TR 38.769) Information exchange between AIoT CN and AIoT RAN

Agenda Item:	16.3

Document for:	Decision

Introduction

The following agreements and FFS were captured in last RAN3 meeting:

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

In this contribution, we will further analyze the information between AIoT CN and AIoT RAN, and base on the discussion, we will provide our observations and proposals accordingly.

Discussion

In last RAN3 meeting, the procedures for A-IoT Inventory have been captured in TR38.769. At RAN3#125bis, we also provide the candidate procedures for A-IoT inventory and command and command only cases in [1]. Here we continue discussing the information to be exchanged between AIoT CN and AIoT RAN for AIoT Inventory and Command procedures.

  - Inventory procedure

For AIoT Inventory procedure, we’ve agreed to use AIoT Device Identification to support inventory for a single device, a group of device, or all devices. Some other information elements are also mentioned by companies’ contributions which may be included in Inventory Request message, as below:

- Session ID or Task ID;

- Cause/Purpose of inventory;

- Reader id;

- Estimated Number of devices;

- Periodicity;

For Inventory Session ID or Task ID, it is used to support multiple inventory procedures in parallel. We are not sure whether this feature can be supported in AIoT radio interface, e.g. multiple inventory procedures are performed simultaneously in the AIoT radio interface. If not, only supporting parallel inventory in XX interface would increase the complexity in A-IoT RAN, e.g. AIoT RAN needs to hold the 2nd, 3rd inventory procedures, and perform the procedures one by one after the ongoing inventory procedure is completed which may cause extra delay for the AIoT CN to get the inventory report, especially for the 2nd, 3rd inventory.

Observation 1: Whether multiple inventory procedures could be performed simultaneously in the AIoT radio interface to be confirmed by RAN1/RAN2.

We see there’s no clear requirement and clear benefits to support multiple inventory procedures over XX interface. To support multiple inventory procedures over XXAP may require some kind of enhancement, e.g. AIoT RAN accepts the inventory request but notifies AIoT CN the wait time or some information about the pending inventory procedures, or else the AIoT CN may not know what’s happening in the AIoT RAN, why there’s no Inventory report in so long time. And considering long wait time of the inventory procedure, we may also need to consider whether to introduce an inventory cancellation procedure to allow AIoT CN cancel any requested inventory task.

Observation 2: No clear benefits to support multiple inventory procedures in XX interface in parallel.

Observation 3: To support multiple inventory procedures performed simultaneously over XXAP may require some kind of enhancement, e.g. indicate AIoT CN the wait time, the failure cause, may also need to introduce inventory cancellation procedure due to long execution time of the multiple inventory procedures over AIoT Radio interface.

As above, support of that is not an elementary function for AIoT, it’s a kind of enhancement, which should not be persuaded in Rel-19 as this is the 1st step to support AIoT. AIoT CN should not initiate another inventory procedure in case there’s still an on-going inventory procedure not completed towards an AIoT RAN. If one AIoT RAN connects to multiple AIoT CN, the 2nd inventory procedure could be failed with clear cause value.

Proposal 1: No need to introduce Task ID/Session ID for AIoT inventory procedure. AIoT CN should not initiate new inventory procedure(s) towards the AIoT RAN in case there’s on-going inventory procedure not completed. If another inventory request is received in XXAP during the on-going inventory procedure, AIoT RAN could fail the procedure with clear cause value.

For the cause/purpose of inventory, it may indicate the cause of triggering inventory procedure, for example, inventory for Command, inventory for locating, inventory for requesting device ID. Whether need to distinguish the cause/purpose of the inventory is pending to the behaviours to be taken in AIoT RAN. Currently, we do not see any necessary to distinguish the purpose and take different actions in AIoT RAN. As above, no need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

Proposal 2: No need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

For the Reader Id, AIoT CN could know relationship between reader and device via the first inventory procedure. Then for subsequent inventory procedure, AIoT CN is capable to select reader to perform inventory, so reader ID may need to be included in inventory message with optional present for Topology 1, if it holds multiple readers. However, how to define the Reader ID, a new ID, or reuse TRP ID should be further discussed.

For the topology 2, we understand one UE reader only holds one “reader”, no need to indicate the reader ID to the UE reader.

As above, Reader ID may be included in Inventory request for Topology 1, if an AIoT RAN holds more than one Readers. How Reader ID is defined is FFS, maybe we can reuse TRP ID.

Proposal 3: Reader ID may be included in inventory request for Topology 1, if an AIoT RAN holds more than one Readers. How Reader ID is defined is FFS, maybe TRP ID could be reused.

For Estimated Number of devices, if AIoT CN could provide estimated number of device for all devices inventory, AIoT RAN may allocate random access resource refer to the estimated number of device. But it is finally up to the conclusion of RAN2.

Proposal 4: Estimated Number of devices seems beneficial for AIoT radio resource management in AIoT RAN, details are pending to RAN2.

For Periodicity, it is used for AIoT RAN to trigger periodic inventory procedure by a single request from AIoT CN. For example, operator start a task to inventory warehouse once a day, it may be better for AIoT CN than for AIoT RAN to trigger inventory procedure every day because AIoT CN is more reliable. Thus, it seems not necessary to provide the periodicity information to AIoT RAN.

Proposal 5: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. Not necessary to introduce the Periodicity in XX message.

If AIoT RAN or AIoT CN decide that inventory is complete, an inventory complete message is necessary to inform the other side. First it is reasonable for AIoT CN to complete the inventory procedure, for example, based on predefined time duration, or operator decides to complete the ongoing the inventory procedure. As for AIoT RAN, whether it could check the completion of inventory is up to RAN2. RAN3 may wait for the conclusion of RAN2.

Proposal 6: It is proposed to introduce an inventory complete message in XX interface. AIoT CN may indicate AIoT RAN the completion of inventory procedure. Whether AIoT RAN could check the completion of inventory is up to RAN2.

In the last meeting, AIoT Device Identification was agreed to be used to trigger inventory request to a single device, a group of devices, or all devices.

Since there may be security issue when transferring Device info directly at AIoT radio interface, device info may be included or partially included in AIoT NAS PDU because AMF/AIoTF may apply security protection to NAS. But it is finally up to conclusion of SA2/SA3.

Proposal 7: How to include AIoT Device Identification in inventory request procedure is finally up to conclusion of SA2/SA3. The following way may be possible:

- including AIoT Device Identification explicitly in inventory request (as an XXAP IE);

- including AIoT Device Identification in the NAS PDU of the AIoT device;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

For the content of inventory response message, we think success or failure indication should be included. For session ID, we may first discuss whether supporting parallel inventory in XX interface. For reader ID, it should be included in report message, not response message.

Proposal 8: success or failure indication should be included in inventory response message.

For the content of Inventory report message, besides Device ID, Reader ID may be needed for Topology 1 to locating purpose, i.e. indicate AIoT CN the coarse location of the device. For Enrichment info (e.g., signal strength measurements by Reader, it needs further discussion.

Proposal 9: device ID and reader ID should be included in inventory report message.

  - Command procedure

Command request message may be sent from AIoT CN to AIoT RAN in inventory and command case and command only case.

As we discussed in [1], for inventory and command case, there are two solutions:

- AIoT CN triggers command request after inventory procedure to AIoT RAN.

- AIoT CN triggers only command request to AIoT RAN.

For the second solution, XXAP message only includes command request information which is same as the solution of command only case in XX interface.

Therefore, in XX interface, command request may be sent directly or after inventory procedure from AIoT CN to AIoT RAN. The solution is finally decided by SA2/RAN2.

Proposal 10: Command request may be sent directly or after inventory procedure from AIoT CN to AIoT RAN. The solution is finally decided by SA2/RAN2.

We have agreed command service is performed to a single device. For the issue whether command service could be triggered towards a group of device, we think it is possible. For example, triggering read/write operation to a group of device. As for command operation to all devices, we are not sure whether there is the scenario to read/write all devices.

So, device info in command request message may include a device or a group of device info.

Proposal 11: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XX interface for command request.

In case of writing device, the action “write” and related “data” is encapsulated as NAS PDU and NG-RAN do not decode NAS. In case of reading device, the action “read” is also encapsulated as NAS PDU. So, NAS info is necessary in command request.

Proposal 12: It is proposed to include NAS information in command request message.

For Reader ID, according to above discussion, AIoT CN is capable to select reader not only for inventory procedure but also for command procedure. Therefore, if reader ID is provided from AIoT CN to AIoT RAN in command request, AIoT RAN triggers the command procedure in the selected reader. Or else, AIoT RAN triggers command in all the Reader(s) it holds.

Proposal 13: It is proposed to include reader ID in command request message.

  - Device context in AIoT RAN

As discussed above, command request may be sent directly or after inventory procedure from AIoT CN to AIoT RAN. If command request triggered after inventory procedure, device context may be used for the following command procedure. For example, Radio resource allocated for the first inventory procedure could also be used for the following command procedure.

Proposal 14: For inventory only request or command only request procedure, there is no needed to setup device context. For inventory and command case, if command request triggered after inventory procedure, device context may be setup for the following procedure.

If device context is needed, the following information may be included in device context:

- Device XXAP ID.

- Device ID. How the Device ID is defined is up to SA2.

- Security information, if AS security is required. It is up to SA3.

- Radio resource. It is up to RAN2 to decide the requirement.

Device XXAP ID may be allocated by RAN when performing the first inventory procedure. Device XXAP ID is used to indicate device to support per device XXAP message. So, device XXAP ID should be kept in device context. Device context may be helpful for the following command or data transport procedure. Based on device XXAP ID in XXAP message, related device context, for example, radio resource, could be found to assist AIoT RAN in sending following command or data to device.

CN performs device ID verifying within the first inventory procedure. If Device ID can be kept in AIoT RAN, AIoT RAN can also take the responsible to verify the device ID afterwards. The benefit is decreasing the time duration of the following inventory procedure.

Security information may be included in device context, if AS security is required. It is up to SA3.

Proposal 15: The device context may include:

-   Device XXAP ID

-	Device ID

-	Security information (If AS security is required)

-	Radio resource (It is up to RAN2 to decide the requirement)

Conclusions

In this contribution, we mainly discussed the network signalling for topology 1. Based on the discussion above, the following proposals are provided:

Observation 1: Whether multiple inventory procedures could be performed simultaneously in the AIoT radio interface to be confirmed by RAN1/RAN2.

Observation 2: No clear benefits to support multiple inventory procedures in XX interface in parallel.

Observation 3: To support multiple inventory procedures performed simultaneously over XXAP may require some kind of enhancement, e.g. indicate AIoT CN the wait time, the failure cause, may also need to introduce inventory cancellation procedure due to long execution time of the multiple inventory procedures over AIoT Radio interface.

Proposal 1: No need to introduce Task ID/Session ID for AIoT inventory procedure. AIoT CN should not initiate new inventory procedure(s) towards the AIoT RAN in case there’s on-going inventory procedure not completed. If another inventory request is received in XXAP during the on-going inventory procedure, AIoT RAN could fail the procedure with clear cause value.

Proposal 2: No need to indicate AIoT RAN the cause/purpose of the Inventory procedure.

Proposal 3: Reader ID may be included in inventory request for Topology 1, if an AIoT RAN holds more than one Readers. How Reader ID is defined is FFS, maybe TRP ID could be reused.

Proposal 4: Estimated Number of devices seems beneficial for AIoT radio resource management in AIoT RAN, details are pending to RAN2.

Proposal 5: Periodically inventory, if needed, could be triggered by the AIoT server or AIoT CN. Not necessary to introduce the Periodicity in XX message.

Proposal 6: It is proposed to introduce an inventory complete message in XX interface. AIoT CN may indicate AIoT RAN the completion of inventory procedure. Whether AIoT RAN could check the completion of inventory is up to RAN2.

Proposal 7: How to include AIoT Device Identification in inventory request procedure is finally up to conclusion of SA2/SA3. The following way may be possible:

- including AIoT Device Identification explicitly in inventory request (as an XXAP IE);

- including AIoT Device Identification in the NAS PDU of the AIoT device;

- partially including AIoT Device Identification in AIoT NAS PDU;

- Both 1) and 2) at the same message, or both 1) and 3) at the same message.

Proposal 8: success or failure indication should be included in inventory response message.

Proposal 9: device ID and reader ID should be included in inventory report message.

Proposal 10: command request may be sent directly or after inventory procedure from AIoT CN to AIoT RAN. The solution is finally decided by SA2/RAN2.

Proposal 11: The scenario of triggering command to a group of devices may be valid, so it is proposed to use AIoT Device Identification to indicate a device or a group of device in XX interface for command request.

Proposal 12: It is proposed to include NAS information in command request message.

Proposal 13: It is proposed to include reader ID in command request message.

Proposal 14: For inventory only request or command only request procedure, there is no needed to setup device context. For inventory and command case, if command request triggered after inventory procedure, device context may be setup for the following procedure.

Proposal 15: The device context may include:

-   Device XXAP ID

-	Device ID

-	Security information (If AS security is required)

-	Radio resource (It is up to RAN2 to decide the requirement)

Reference

[1] R3-245059 (TP for TR 38.769) Procedures for AIoT Command, CATT

TP for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

6.5	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

- 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

- 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(2)	Estimated Number of devices (for all device inventory)

Editor’s Note: Further check with RAN2 on AIoT radio resource allocation.

(3)   Service type (indicate inventory or command request for agnostic of the type of request)

Editor’s Note: If Inventory and command procedure is performed by Inventory procedure and Command procedure separately by AIoT CN, the service type is not needed in the Inventory Request.

(4)   Reader ID

For Topology 1, Reader ID may need to be included in the inventory request, if an AIoT RAN node holds more than one Readers. The Reader ID may be included in the Inventoy report to represent for the location of an AIoT device.

Editor’s Note: Definition of Reader ID to be further discussed, e.g. whether TRP ID could be reused.

Multiple individual A-IoT Device IDs (one ID per device) and related Reader ID can be provided to the A-IoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

- 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device.

Editor’s Note 1: it is FFS for command on all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)

The Command Request from the AIoT CN to the AIoT RAN, may include the following:

- AIoT Device Identification (to find a single device or a group of devices)

Note 1:	The definition of this identification is out of RAN3 scope.

- Reader ID

For Topology 1, Reader ID may need to be included in the Command request to send the command to the AIoT device in the indicate Reader(s). The Reader ID may be included in the Command Response to represent for the location of an AIoT device.

----------------End of the Changes----------------