3GPP TSG-RAN WG3 Meeting #125bis                         	R3-245061

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	(TP for TR 38.879) Locating of A-IoT device

Agenda Item:	16.4

Document for:	Discussion and Decision

- Introduction

In the previous RAN3 meeting, we discussed the support of “positioning” or “locating” of the A-IoT device, some agreements or common understandings are achieved as below:

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

FFS for other granularities.

In this contribution, we will further discuss the support of locating aspects for A-IoT, and provide the observations and proposals accordingly.

- Discussion

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Before we discuss the issue, we should first clarify what the “reader” will be like.

To our understanding, in topology 1, a “reader” looks like a TRP, an antenna, a beam, or a cell of a gNB, which is used to serve the AIoT devices in a specific area. To make the deployment more flexible, maybe the reader is co-located with the gNB, maybe not, and it’s reasonable to support multiple “readers” per A-IoT RAN node, and it’s assumed the “readers” are stationary. How the readers are connected to the gNB is out of RAN3 scope, as shown in the figure below:

Figure 1: one A-IoT RAN node holds one or multiple readers

As above, one AIoT RAN node may holds one or multiple readers (up to deployment), they are mainly used for Inventory and Command procedures, not specifically for locating purpose (i.e., not consider using multiple readers to get higher accuracy device location).

Observation 1: In Topology 1, a reader looks like a TRP, an antenna, a beam, or a cell of a gNB.

Observation 2: In Topology 1, it’s assumed the BS reader(s) are stationary.

Observation 3: In Topology 1, how the “reader(s)” are connected to the A-IoT RAN node is up to implementation, out of 3GPP.

Proposal 1: In Topology 1, one A-IoT RAN node may hold one or multiple readers, they are to be used for Inventory, Command, not specifically for locating purpose.

RAN3 should discuss whether to define “Reader ID” for AIoT locating and or other functions. To our understanding, it’s not necessary to define a new “Reader ID” for AIoT, the TRP ID and related information could be reused to represent a Reader instead of defining a new “ Reader ID”.

Proposal 2: No need to define “Reader ID”, TRP ID and related information could be reused to represent a “Reader” of AIoT RAN.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

For Topology 1, it’s assumed the A-IoT RAN node knows the location of the readers(e.g. TRPs) it holds, and the service area of each reader(TRP).

If Reader/TRP ID is only known by AIoT RAN, AIoT RAN could deduce the GNSS location of the device according to the reader/TRP where a device responses the Inventory or Command over AIoT Radio, and provide the GNSS location of the device and probably the Location uncertainty.

If Reader ID/TRP ID and related info are known by the AIoT CN, AIoT RAN only needs to provide the Reader/TRP ID where a device responses in the Inventory/Command procedure. In this case, the TRP ID and related information could be provided to AIoT CN via signalling or OAM configuration, no matter it’s LMF or new AIoT functions.

Proposal 3: RAN3 should discuss and decide which option to go:

- Option 1: AIoT RAN provides device location (e.g. GNSS location, location uncertainty) to AIoT CN;

- Option 2: AIoT RAN provides TRP ID to represent for the location of AIoT device.

For Topology 2, it’s assumed that one intermediate UE holds only one Reader, AIoT CN knows the UE is a reader during UE registration procedure (e.g. according to the UE capability). The Inventory/Command response is reported to AIoT CN via the UE specific signalling, e.g. the NAS of the UE.

Observation 4: In Topology 2, intermediate UE is registered to the network as UE Reader, and one intermediate UE holds only one reader.

As above, the CN is able to know the mapping between the AIoT devices and the UE reader(s), it knows the reader granularity location of an AIoT device without extra information.

Figure 2: Example of Top2, mapping between UE readers and AIoT devices.

Proposal 4: In Topology 2,  UE specific signalling will be used to carry the inventory/command signallings. AIoT CN knows the reader granularity location of an AIoT device without any additional information.

For Topology 2, the UE reader may be stationary, maybe moveable, how the CN could get the location of the UE reader need to be discussed.

Observation 5: In Topology 2, UE reader may be stationary, maybe moveable.

If UE reader stationary, the UE location could be pre-configured/OAM configured to the AIoT CN. If the UE reader is moveable, OAM based solution is not feasible. No matter whether UE is stationary or moveable, AIoT CN could obtain the location of the UE reader via LCS.

Proposal 5: In Topology 2, AIoT CN could obtain the location of the UE reader via OAM configuration or via LCS.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

In the SID [2], the objective shows:

- Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

This is RAN3-led objective, the key intension is not to introduce new Positioning mechanism, just considering how to report the device location via existing signalling or with minimum specification impact.

The understand the finer granularity of AIoT locating/positioning solutions could be further worked in the further release to satisfy the use cases which requires higher locating accuracy, e.g. 1~3m.

Proposal 6: According to the SID, locating an Ambient device besides the “reader ID granularity” is acceptable, no need to consider finer granularity in this release.

Base on the discussion above, we could remove the Editor’s Notes 1~4 could be removed. We provide the TP accordingly in the annex. RAN3 is requested to discuss and agree the TP for AIoT locating in section 5.

Proposal 7: RAN3 is requested to discuss and agree the TP for AIoT locating in section 5.

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Observation 1: In Topology 1, a reader looks like a TRP, an antenna, a beam, or a cell of a gNB.

Observation 2: In Topology 1, it’s assumed the BS reader(s) are stationary.

Observation 3: In Topology 1, how the “reader(s)” are connected to the A-IoT RAN node is up to implementation, out of 3GPP.

Proposal 1: In Topology 1, one A-IoT RAN node may hold one or multiple readers, they are to be used for Inventory, Command, not specifically for locating purpose.

Proposal 2: No need to define “Reader ID”, TRP ID and related information could be reused to represent a “Reader” of AIoT RAN.

Proposal 3: RAN3 should discuss and decide which option to go:

- Option 1: AIoT RAN provides device location (e.g. GNSS location, location uncertainty) to AIoT CN;

- Option 2: AIoT RAN provides TRP ID to represent for the location of AIoT device.

Observation 4: In Topology 2, intermediate UE is registered to the network as UE Reader, and one intermediate UE holds only one reader.

Proposal 4: In Topology 2,  UE specific signalling will be used to carry the inventory/command signallings. AIoT CN knows the reader granularity location of an AIoT device without any additional information.

Observation 5: In Topology 2, UE reader may be stationary, maybe moveable.

Proposal 5: In Topology 2, AIoT CN could obtain the location of the UE reader via OAM configuration or via LCS.

Proposal 6: According to the SID, locating an Ambient device besides the “reader ID granularity” is acceptable, no need to consider finer granularity in this release.

Proposal 7: RAN3 is requested to discuss and agree the TP for AIoT locating in section 5.

- Reference

- RAN3#125 Chair Notes

- RP-234058 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- TR 22.840 Study on Ambient power-enabled Internet of Things

- TR 38.848 Study on Ambient IoT (Internet of Things) in RAN

- TP for TR 38.879 AIoT Locating

<<<<<<<<<<<<<<<<<<<<<< Begin of the Changes >>>>>>>>>>>>>>>>>>>>>>>>

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, an AIoT RAN node may hold one or multiple readers. The readers are TRP like, and assumed to be stationary. AIoT RAN node ID and probably TRP ID can be considered as a location of AIoT device. Two possible ways to report the location of AIoT device to AIoT CN:

- AIoT RAN provides location of AIoT device(e.g. GNSS location, location uncertainty) to AIoT CN;

- AIoT RAN provides TRP ID to represent for the location of AIoT device, where TRP ID and related information could be obtained via signalling or OAM configuration.

Note: In Topology 1, one A-IoT RAN node may hold one or multiple readers, they are to be used for Inventory, Command procedures, not specifically for locating purpose.

For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device. AIoT CN could deduce the location of the AIoT device via the mapping between UE specific signalling and Inventory report, or command response of AIoT device. AIoT CN could obtain the location of the UE reader via OAM configuration or via LCS.

<<<<<<<<<<<<<<<<<<<<<< End of the Changes >>>>>>>>>>>>>>>>>>>>>>>>