3GPP TSG-RAN WG3 Meeting #125bis                             R3-245065

Hefei, China, 14th – 18th, October, 2024

Source:	CATT

Title:	Consideration on SDT signalling optimization for partial context transfer

Agenda Item:	8.3

Document for:	Discussion and Decision

Introduction

In the Rel-17 SDT, handling of DL non-SDT in case of SDT without anchor relocation is relied on RRCRelease + subsequent paging to trigger another RRC Resume as legacy behaviors designed for RRC_INACTIVE. However, this is not so efficient, in Rel-18, RAN3 further discussed and suggest RAN2 to further consider an optimization to let UE trigger RRC Resume Request automatically after receives RRCRelease.

In the LS [1], RAN2 confirms the Uu-based solution for non-SDT handling in case of SDT with partial context transfer case is supported. Accordingly, RAN3 completed the Uu-based solution and endorsed the corresponding CRs.

Beyond the Uu-based solution, some companies in RAN3 also wonders if feasible to define a network based solution for non-SDT handling in case of SDT with partial context transfer case, which provide another option for the NG-RAN, especially when the UE does not support the Uu-based solution. RAN3 discussed but could not decide whether there’s any extra impact to the UE behaviours, therefore, RAN3 sent an LS to RAN2 [2] to check the NR-Uu impact.

In RAN3#125bis, RAN2 replied the LS in [3]. In this contribution, we further discuss the feasibility of the network based solution, taking the information of the RAN2 LS into account.

Discussion

In the RAN3#123bis, RAN3 discussed and agreed the Uu-based solution and endorsed the CRs accordingly. And RAN3 discussed a network-based solution, i.e., for UEs not supporting the Uu-based solution as defined by RAN2. However, it’s not clear to RAN3 whether there’s any extra impact to the UE behaviours, that’s why RAN3 sent the LS to RAN2 to further check.

RAN3 would like to thank RAN2 for the reply LS on SDT signalling optimization for partial context transfer (R3-240015 / R2-2313759).

RAN3 has endorsed related CRs for TS 38.300 and TS 38.401, to adopt RAN2 solution based on RRCRelease with resume indication solution.

In addition, RAN3 has discussed a network-based solution, i.e., for UEs not supporting the RAN2 solution above, where the last serving gNB during the ongoing SDT procedure without anchor relocation, upon, e.g., receiving a DL-non SDT data, further relocates the UE context to the receiving gNB by sending an XnAP RETRIEVE UE CONTEXT RESPONSE message, and then the receiving gNB sends the RRCSetup message to the UE (as in TS 38.331, Figure 5.3.13.1-2) with ongoing SDT in RRC_INACTIVE state.

RAN3 assumes that the network-based solution would have impact on Xn signalling only, but would like to confirm with RAN2 whether this network-based solution has any RAN2 impact or issue.

Following the LS, RAN2 discussed the issue a few meetings, not easy to reach the consensus on whether there’s any impact to the UE behaviours, whether there’s any issue to support network based solution. Finally, RAN2 replied the LS to RAN3 in [3]:

RAN2 would like to thank RAN3 for the reply LS on SDT signalling optimization for partial context transfer in R2-2404123 (R3-242198).

With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that:

- If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost.

- If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again

In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

In the existing specification, it is not clearly specified how to handle the RRCSetup during SDT transmission procedure.

According to the reply LS [3], RAN2 would like to inform RAN3 that:

- If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost.

- If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again

Observation 1: When UE receives RRCSetup during SDT, the UE will discard any pending UL data.

Observation 2: When UE receives RRCSetup during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again.

In the Reply LS [3], RAN2 also pointed out the potential “MAC reset issue”, as below:

In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

According to existing specification, MAC is not reset when UE receives the RRCSetup in RRC_INACTIVE. Following the legacy UE behavior as above, and not introducing any extra enhancement to NR Uu, MAC is not reset during SDT without anchor relocation case.

Observation 3: MAC is not reset when UE receives RRCSetup in RRC_INACTIVE for normal RRC Resume case.

We understand no issue to continue the MAC processes between the UE and the receiving gNB before and after RRCSetup for SDT without anchor relocation case.

Observation 4: No issue to continue the MAC processes between the UE and the receiving gNB before and after RRCSetup for SDT without anchor relocation case.

As above, it’s possible to support network based solution, i.e. fallback to new RRC connection in case of non-SDT data is received during SDT without anchor relocation. However, the pending DL non-SDT data received in the last serving gNB, and the pending UL SDT data in the UE, if any, will be lost. If needed, the lost data need to be resent via the upper layer.

Proposal 1: On handling of DL non-SDT data during SDT without anchor relocation, it’s feasible to support network based solution, i.e. fallback to new RRC connection without any extra impact to UE behaviours.

Conclusion

In this contribution, we further discussed the SDT signalling optimization for partial context transfer according to the RAN2 LS reply. Based on the discussion, the following observations and proposal are provided:

Observation 1: When UE receives RRCSetup during SDT, the UE will discard any pending UL data.

Observation 2: When UE receives RRCSetup during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again.

Observation 3: MAC is not reset when UE receives RRCSetup in RRC_INACTIVE for normal RRC Resume case.

Observation 4: No issue to continue the MAC processes between the UE and the receiving gNB before and after RRCSetup for SDT without anchor relocation case.

Proposal 1: On handling of DL non-SDT data during SDT without anchor relocation, it’s feasible to support network based solution, i.e. fallback to new RRC connection without any extra impact to UE behaviours.

Reference

- R3-240015 / R2-2313759 Reply LS on SDT signalling optimization for partial context transfer (RAN2 to RAN3)

- R3-242198 Reply LS on SDT signalling optimization for partial context transfer (RAN3 to RAN2)

- R3-245012 Reply LS on SDT signalling optimization for partial context transfer (RAN2 to RAN3)