3GPP TSG-RAN3 Meeting #125bis                                                                R3-245068

# Hefei, China , Oct 14th  – 18th , 2024

Agenda Item:	19.2

Source: 	Qualcomm Inc.

Title: 	Signaling design to enable NR SBFD operation.

Document for:	Discussion/Decision

# Background

Following are the WID objectives for NR SBFD.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of time location of SBFD subbands in SIB is not precluded

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

    - Indication of frequency domain location of SBFD subbands in SIB is not precluded

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Followings are assumed based on TR 38.858

    - SBFD at the gNB side

    - Half duplex operation at the UE side

    - FR1 and FR2-1

    - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs

    - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

    - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

    - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

    - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - Information exchange among gNBs, including [RAN3]

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

    - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

      - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

      - PUSCH resource mapping, i.e., rate-matching around the muted REs

      - UCI resource determination in symbols with muted REs

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

      - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

      - At least aperiodic reporting

      - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - UCI bits generation

      - Priority rules for multiple CSI reporting

      - CLI measurement accuracy requirement

    - Note: Without dedicated optimization for dynamic/flexible TDD.

- Specify BS RF requirements for SBFD operation at gNB [RAN4]

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

- Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

- Notes: Following are assumed for UL resource muting

  - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting

  - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212

  - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH

  - UL resource muting applies only for UEs in RRC_CONNECTED mode

  - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol

  - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols

  - No changes on TBS determination for PUSCH

  - Check impact on transmit signal quality/MPR requirement, if any, with RAN4

  - This feature is subject to UE capability

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

  - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting

  - For the new reporting quantities, at least wideband reporting is supported

  - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we will discuss various RAN3 related signalling design proposals to enable successful NR SBFD operation.

# Sub Band (Non-Overlapping) Full Duplex (SBFD) Signalling design framework

## Introduction

In NR TDD, common frequency is used for both DL and DL transmission between gNB and UE. In time domain, common frequency resources are shared between DL and DL. Cell specific NR TDD configuration is provided to UEs via SIB1 in TDD-UL-DL-ConfigCommon IE. UE specific NR TDD configuration will be configured to UEs in dedicated RRC signalling messages using TDD-UL-DL-ConfigDedicated IE to enable dynamic TDD mode of operation.

In NR TDD configuration, in a given OFDM symbol in a given slot, entire spectrum is used either for DL transmission or UL reception from gNB perspective. In DL heavy configuration, there will be limited number of UL transmission opportunities for UE UL transmission. In order to improve UL capacity, UL coverage by aggregating a number of UL slots/symbols for transmission and to improve UL transmission latency, based on R18 RAN1 study item conclusion, it is beneficial to use DL and flexible symbols for UL transmission by UEs in configured UL sub bands. This means, in a given TDD DL/Flexible symbol in a given slot, it is possible for gNB to transmit in DL Sub-bands and receive from UEs in UL sub-band. RAN1 agreed to have a single UL sub band in a given DL an/or flexible symbols and is shown in below figure 1.

Figure 1:

In time domain, a slot can consist of both SBFD symbols and non-SBFD symbols.

RAN1 Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

FFS: Details when two TDD-UL-DL patterns are configured

Figure 2:

For semi-static indication of SBFD subband time location,

RAN1 Agreement:

When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period.

- SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon

- The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

- referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

Figure 3: SBFD symbols per each TDD pattern

From above description, similar to NR TDD, it is evident that there exist SBFD configuration (similar to NR TDD configuration) and cross link interference (CLI) may happen for the case of Inter-gNB and Inter-UE scenarios.

In next sections, we will describe the signalling enhancements needed to for exchanging SBFD configuration between gNBs and signalling enhancements needed to handle Inter-gNB CLI and Inter-UE CLI scenarios.

## Signalling to exchange SFBD Configuration

In case of NR TDD, during F1 Setup and GNB-DU CONFIGURATION UPDATE Procedure DU provides TDD configuration of NR Carrier Freq. In Xn Setup and NG-RAN NODE CONFIGURATION UPDATE signalling messages each gNB/CU-CP exchange its serving cell configuration.

As described in above section, SBFD duplex mode is configured for NR TDD carriers only. In time domain, consecutive slots and symbols can be configured as SBFD symbols and within SBFD symbols, in frequency domain certain number of PRBs will be configured as UL Subband and remaining PRBs as DL Subband.

Following is RAN1 agreement for time domain configuration for SBFD operation:

RAN1#118 Agreement:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

Starting slot index

Ending slot index

Ending Symbol index

Starting Symbol index

Figure 4:

In Frequency domain, the location of each of the SBFD UL sub band and the SBFD DL subband is based on RIV-indication, configuring the starting PRB and width of the sub band.

RAN1 discussed about frequency domain configuration and following is working assumption.

RAN1#118 : Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Figure 5:

RAN2 briefly discussed about SBFD configuration and following is agreement:

RAN2#127 Agreement:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In case of NR TDD, during F1 Setup and GNB-DU CONFIGURATION UPDATE Procedure DU provides TDD configuration of NR Carrier Freq. In Xn Setup and NG-RAN NODE CONFIGURATION UPDATE signalling messages each gNB/CU-CP exchange its serving cell configuration.

Similarly, RAN3 has to specify SBFD configuration exchange between gNBs and between CU-DU. For detailed IE structure, RAN3 has to wait for RAN1/RAN2 discussions and then RAN3 can specify stage-3 IEs accordingly.

Introduce SBFD configuration within TDD carrier configuration in frequency and time domain including

- SCS specific SBFD configuration for each SCS configuration per cell (SCS-SpecificCarrierList)

- In frequency domain , SBFD subband starting PRB and number of PRBs (for SBFD UL subband and for SBFD DL subband).

- In time domain , enable configuration of SBFD symbols within a TDD-UL-DL pattern period including

  - A starting slot index , A starting symbol index within the starting slot

  - An ending slot index , An ending symbol index within the ending slot

SBFD configuration is included  in Served Cell Information NR IE in following Xn and F1 messages and actual IE details FFS based on further RAN1/2 discussion:

- Xn Setup procedure,

- NG-RAN NODE CONFIGURATION UPDATE,

- F1 Setup ,

- GNB-DU CONFIGURATION UPDATE.

During NR SON/MDT discussions, in both F1 and Xn interface, Served Cell Information-> NR PRACH Configuration-> NR PRACH Configuration List IE was introduced to avoid PRACH collision between neighbouring gNB cells. For SBFD, PRACH configuration enhancements are actively being discussed in both RAN1 and RAN2.

There are 2 RACH configuration options under discussion for SBFD configuration:

RAN1#118 Agreement :

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

SBFD

SBFD

RO

DL

FL

SBFD

RO

DL

UL

RO

Non-SBFD

FL

SBFD

RO

Figure 6: RACH configuration Option 1 (Additional RO in red, Legacy RO in orange)

Based on RAN1/RAN2 progress, RAN3 has to discuss changes to NR RACH configuration to be included in both F1 and Xn signaling messages.

Thus, we propose

RAN3 to wait for RAN1/RAN2 conclusion about SBFD RACH configuration discussion and then introduce SBFD NR RACH configuration related IEs in both F1 and Xn signalling messages.

## Inter gNB - Cross Link Interference (CLI)

When SBFD mode is configured by gNBs, victim gNB’s UL reception in UL subband may be interfered by another neighbor aggressor gNB’s DL transmission in DL subband(s) in SBFD symbols. This type of inter-gNB interference is analogous to R16 RIM (Remote Interference Management) due to dynamic/flexible TDD configuration.

Figure 7

In order to enable interference measurement by each gNB, either SSB and/or NZP CSI-RS can be used by each gNB for interference measurement. This requires each gNB has to provide its serving cell SSB and/or NZP CSI-RS configuration to neighbouring gNBs. For interference detection, each gNB based on their implementation has perform SSB and/or NZP CSI-RS measurements by taking SFBD configuration and SSB/NZP CSI-RS configuration into account.

For both FR1 and FR2-1, it is possible for victim gNB to perform interference measurements based on SSB and/or NZP CSI-RS configuration.

In case of FR1, typically beams are wide, victim gNB can measure received power of SSB/NZP-CSI-RS and report to the aggressor gNB. In order for aggressor gNB to take implementation based scheduling action to supress its interference to other victim gNBs, it is desirable for victim gNB to provide receive channel measurement (RSRP, RSRQ and/or RSSI value) results to aggressor gNB per cell level. Just victim gNB providing a one bit indicating to inform aggressor gNB about CLI interference is not sufficient for aggressor gNB to determine how severe is interference to other victim gNBs and what appropriate action to take.

FR 2 reciprocity based beam measurements are depicted in below figure 8.

In case of FR2-1, SSB and NZP CSI-RS beams are much more granular compared to FR1 beams. Victim gNBs are expected to measure aggressor gNBs SSB beams, and/or NZP CSI-RS beams and select strongest DL beams causing interference for reporting to the aggressor gNBs. For SSB based measurements, SSB beam index will be reported and for NZP CSI-RS beam measurements (each NZP CSI-RS Resource corresponds to a CSI-RS beam and CSI-RS Resource Indicator (CRI)), CRI value representing a NZP CSI-RS beam will be reported along with victim gNB measured interference value. When and how often victim gNB provides interference measurements to aggressor gNBs (i.e., periodically or certain event based when interference level is about certain implementation based threasholds) is left upto to victim gNB implementation.

Based on interference measurement reports received by aggressor gNB from victim gNBs, it is left upto aggressor gNB implementation what action to take to mitigate inter-gNB interference. For example, aggressor gNB may use various scheduling techniques including TDM, FDM or Spacial domain schemes like beam pairing or beam nulling for interference mitigation.

RAN1 also discussed about potential inter-gNB signalling mechanisms necessary to handle inter-gNB CLI. RAN1  sent an LS [2] to RAN3 with the following information.

RAN1#118 Agreement

Send an LS to RAN3 with the following information (cc: RAN2)

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

Based on above discussion, we have following proposals:

gNB A and gNB B exchange SSB configuration, NZP-CSI-RS-Resource and NZP-CSI-RS-ResourceSet (as measurement RS) per each cell. Stage-3 details FFS.

Based on gNB implementation, each gNB measures neighbor gNBs SSBs and/or NZP-CSI-RS-ResourceSet to detect Inter-gNB CLI .

For FR1 case (beam nulling) : In Xn interface, victim gNB indicates to aggressor gNB about CLI mitigation request based on victim gNB measurement of SSB and/or NZP-CSI-RS  transmitted by aggressor gNB.

Victim gNB indicated CLI request includes radio channel metrics including RSRP, RSRQ, RSSI for measured RS per cell level granularity and it is left upto victim gNB implementation when to trigger and whether to report periodically or event based.

The specific action taken by aggressor gNB to mitigate inter-gNB interference is left upto aggressor gNB implementation. i.e aggressor gNB may use TDM, FDM or spatial scheduling mechanisms to mitigate interference.

For FR2-1 case (strongest beam measured by victim gNB) : In Xn interface, victim gNB indicates to aggressor gNB about strongest beam indication for beam specific interference mitigation by aggressor gNB .

For NZP CSI-RS based measurements by victim gNB, strongest beam(s) indication is indicated by CRI value (CSI-RS Resource Indicator value). CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. For each CRI, victim gNB reports radio channel measurements per beam level and cell level granularity.

For SSB based measurements by victim gNB, strong SSB beam(s) are indicated by SSB Index value along with measured radio channel metrics per each cell level.

RAN3 to further discuss F1 signalling enhancements to exchange cross node interference information exchange between CU and DU.

## Inter-UE Cross Link Interference (CLI)

When a aggressor UE is transmitting in SBFD symbol of UL subband to its serving cell, this may cause interference to neighbouring near by Victim UEs receiving in the downlink sub-band(s) of SBFD symbol as shown in below figure. Here the assumption is that not all UEs will have same SBFD UL configuration. i.e One UEs SBFD UL subband  may fall into SBFD DL subband of another UE and thus causing inter-UE interference.

In another scenario, UEs may have aligned DL/UL subband configuration, in which case, inter-UE CLI may happen due to inter-subband leakage from UL subband of aggressor UE to DL subband of victim UE.

Figure 9

Inter-UE CLI could happen between Aggressor UE and Victim UE either within same cell or across the cell as shown in above figure.

From network perspective, following are scenarios where Inter-UE CLI is expected to occur.

- Intra gNB intra-DU (Intra Cell and Inter Cell)

- Intra gNB inter-DU  (Inter Cell)

- Inter gNB inter-DU (Inter Cell)

To enable discussion, first RAN3 has to agree scenarios to be considered for Inter-UE CLI. Thus, we propose

RAN3 agree following scenarios for Inter-UE CLI.

- Intra gNB inter-DU (both Intra Cell and Inter Cell) ,

- Inter gNB inter-DU (Inter Cell),

- Inter gNB inter DU (Inter Cell)

In R16 Inter-UE CLI for dynamic/flexible TDD scenario, only intended Intended TDD DL-UL Configuration NR IE was exchanged between 2 gNBs in Xn Setup and NG-RAN NODE CONFIGURATION UPDATE procedures. F1 Setup and GNB-DU CONFIGURATION UPDATE signalling messages were enhanced to support exchange of Intended TDD DL-UL Configuration NR IE between CU and DU.

Victim UEs will be configured with SRS resource configuration(s) and/or CLI RSSI resources of Aggressor UEs to measure SRS-RSRP and CLI-RSSI for reporting to its serving gNB cells.

Observation 1: Victim UEs will be configured with SRS resource configuration(s) and/or CLI RSSI resources of Aggressor UEs to measure SRS-RSRP and CLI-RSSI for reporting to its serving gNB cells.

Victim UE CLI-RSSI measurement in SBFD DL subband is measuring leakage of UL transmission in UL subband into DL subband. Because of this, victim UE measured CLI-RSSI is supposed to be relatively weak. To address this leakage issue, it is beneficial for Serving gNB to mute SBFD DL transmission in DL subband on symbols overlapping with CLI-RSSI resource of aggressor UEs. It would be highly beneficial to mute SBFD DL transmission from neighboring gNBs. Otherwise, CLI-RSSI measurement may also capture received power from DL transmission of neighboring gNBs

Observation 2: Victim UE CLI-RSSI measurement in SBFD DL subband is measuring leakage of UL transmission in UL subband into DL subband, which results in victim UE measured CLI-RSSI will be of low quality.

Observation 3: It is beneficial for Serving gNB to mute SBFD DL transmission in DL subband on symbols overlapping with CLI-RSSI resource of aggressor UEs. Otherwise, CLI-RSSI measurement may also capture received power from DL transmission of neighboring gNBs.

Given this background, we believe it is beneficial if gNBs can align configuration of CLI-RSSI resource and associated DL muting in time domain by exchanging SRS and CLI RSSI resources between gNBs and between DUs of same gNB.

But actual SRS configuration/CLI RSSI resource configuration provided to victim UEs for cross link interference measurement was not exchanged between nodes and left to RAN OAM configuration. Issue with OAM configuration approach is there is no support for Inter-vendor support (typically OAM approach does not provide inter-operability via signalling and is limited to intra vendor limitation). From specification perspective, it is essential to support interoperability between different gNBs of different RAN vendors.

Observation 4: For inter-UE CLI case, lack of inter-gNB signalling support for SRS resource configuration(s) and/or CLI RSSI resources exchange results in “no support” for Inter vendor operability and each gNB will not be able to determine  which SBFD DL symbols to mute based on neighbor cells SRS/CLI RSSI resource configuration.

Thus, in order to support SBFD Inter-UE CLI measurement configuration, we propose RAN3 to specify UL SRS configurations (cell specific SRS configuration and UE specific SRS configuration list) and CLI RSSI resources exchange between gNBs via Xn signalling.

For Intra gNB and Inter gNB case, RAN3 to discuss signaling enhancements for exchange of UL cell specific and UE specific SRS configuration(s) and CLI RSSI resources.

In R16 Inter-UE CLI for dynamic TDD scenario, L3 measurement configuration and reporting was used.  For R19 SBFD, in order to support fast and bursty interference measurement and reporting, RAN1/RAN2 will be working on L1 measurement configuration and reporting. It is possible that both L3 and L1 based measurement configuration and reporting are expected to co-exist, RAN3 need to discuss CU-DU interactions for co-existence of L3 and L1 based measurement configurations.

Thus, we propose

RAN3 to discuss CU-DU interaction for L1 based inter-UE CLI measurement configuration and trigger mechanism, which node decides about inter-UE L1 measurement configuration.

RAN3 to discuss co-existence between L3 and L1 based inter-UE CLI measurement configuration and trigger mechanism

.

# Summary

Based on above discussion, we have the following observations and proposals for SBFD signalling enhancements.

SBFD configuration exchange proposals:

Proposal 1.	Introduce SBFD configuration within TDD carrier configuration in frequency and time domain including

-	SCS specific SBFD configuration for each SCS configuration per cell (SCS-SpecificCarrierList)

-	In frequency domain , SBFD subband starting PRB and number of PRBs (for SBFD UL subband and for SBFD DL subband).

-	In time domain , enable configuration of SBFD symbols within a TDD-UL-DL pattern period including

o	“A starting slot index , A starting symbol index within the starting slot

o	An ending slot index , An ending symbol index within the ending slot

Proposal 2.	SBFD configuration is included  in Served Cell Information NR IE in following Xn and F1 messages and actual IE details FFS based on further RAN1/2 discussion:

-	Xn Setup procedure,

-	NG-RAN NODE CONFIGURATION UPDATE,

-	F1 Setup ,

-	GNB-DU CONFIGURATION UPDATE.

Inter-gNB CLI proposals:

Proposal 3.	RAN3 to wait for RAN1/RAN2 conclusion about SBFD RACH configuration discussion and then introduce SBFD NR RACH configuration related IEs in both F1 and Xn signalling messages.

Proposal 4.	gNB A and gNB B exchange SSB configuration, NZP-CSI-RS-Resource and NZP-CSI-RS-ResourceSet (as measurement RS) per each cell. Stage-3 details FFS.

Proposal 5.	Based on gNB implementation, each gNB measures neighbor gNBs SSBs and/or NZP-CSI-RS-ResourceSet to detect Inter-gNB CLI .

Proposal 6.	For FR1 case (beam pairing) : In Xn interface, victim gNB indicates to aggressor gNB about CLI mitigation request based on victim gNB measurement of SSB and/or NZP-CSI-RS  transmitted by aggressor gNB.

Proposal 7.	Victim gNB indicated CLI request includes radio channel metrics including RSRP, RSRQ, RSSI for measured RS per cell level granularity and it is left upto victim gNB implementation when to trigger and whether to report periodically or event based.

Proposal 8.	The specific action taken by aggressor gNB to mitigate inter-gNB interference is left upto aggressor gNB implementation. i.e aggressor gNB may use TDM, FDM or spatial scheduling mechanisms to mitigate interference.

Proposal 9.	For FR2-1 case (strongest beam measured by victim gNB) : In Xn interface, victim gNB indicates to aggressor gNB about strongest beam indication for beam specific interference mitigation by aggressor gNB .

Proposal 10.	For NZP CSI-RS based measurements by victim gNB, strongest beam(s) indication is indicated by CRI value (CSI-RS Resource Indicator value). CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. For each CRI, victim gNB reports radio channel measurements per beam level and cell level granularity.

Proposal 11.	For SSB based measurements by victim gNB, strong SSB beam(s) are indicated by SSB Index value along with measured radio channel metrics per each cell level.

Proposal 12.	RAN3 to further discuss F1 signalling enhancements to exchange cross node interference information exchange between CU and DU.

Inter-UE CLI proposals:

Observation 1: Victim UEs will be configured with SRS resource configuration(s) and/or CLI RSSI resources of Aggressor UEs to measure SRS-RSRP and CLI-RSSI for reporting to its serving gNB cells.

Observation 2: Victim UE CLI-RSSI measurement in SBFD DL subband is measuring leakage of UL transmission in UL subband into DL subband, which results in victim UE measured CLI-RSSI will be of low quality.

Observation 3: It is beneficial for Serving gNB to mute SBFD DL transmission in DL subband on symbols overlapping with CLI-RSSI resource of aggressor UEs. Otherwise, CLI-RSSI measurement may also capture received power from DL transmission of neighboring gNBs.

Observation 4: For inter-UE CLI case, lack of inter-gNB signalling support for SRS resource configuration(s) and/or CLI RSSI resources exchange results in “no support” for Inter vendor operability and each gNB will not be able to determine which SBFD DL symbols to mute based on neighbor cells SRS/CLI RSSI resource configuration.

Proposal 13.	RAN3 agree following scenarios for Inter-UE CLI.

-	Intra gNB inter-DU (both Intra Cell and Inter Cell) ,

-	Inter gNB inter-DU (Inter Cell),

-	Inter gNB inter DU (Inter Cell)

Proposal 14.	For Intra gNB and Inter gNB case, RAN3 to discuss signaling enhancements for exchange of UL cell specific and UE specific SRS configuration(s) and CLI RSSI resources.

Proposal 15.	RAN3 to discuss CU-DU interaction for L1 based inter-UE CLI measurement configuration and trigger mechanism, which node decides about inter-UE L1 measurement configuration.

Proposal 16.	RAN3 to discuss co-existence between L3 and L1 based inter-UE CLI measurement configuration and trigger mechanism

# References

# [1] RP-241614: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] R1-2407533 : LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation