3GPP TSG-RAN WG3 Meeting #125-bis	R3-245070

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) A-IoT RAN Architecture aspects

Agenda Item:	16.2

Source:	Huawei

Document for:	other

# Introduction

In RAN3#125 meeting, the logical system architectures for Topo1 and Topo2, and the protocol stack of Topo 1 were captured in [1].

In this contribution, we further discuss the leftover RAN Architecture aspects.

# Discussion

## XX interface

In [1], the description of XX interface is:

XX interface: Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Considering the following open issue:

For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting. To be continued...

It is needed to update the definition of XX interface to cover both topo 1 and topo 2.

Proposal 1: Change the description of XX interface to: Interface between the A-IoT RAN and the A-IoT CN in topology 1, and between A-IoT-enabled gNB and the A-IoT CN in topology 2, on which certain A-IoT specific functions are performed.

For Topology 1, there is an FFS about signalling transport for XXAP:

Editor’s Note 2: the signalling transport for XXAP is FFS.

Based on the previous discussion, there is a common understanding to support the data transport over XXAP, and it is regarded by majority companies as control plane transmission, therefore the signalling transport for XXAP should be SCTP/IP, which follows the tranditional way of RAN3 interfaces.

Proposal 2: In Topology 1, the signalling transport for XXAP is SCTP/IP.

Besides, there are other two Editor’s notes related to XX interface:

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone. Editor’s Note 2: In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Please note that there are also editor’s notes for T1 and T2 said that “the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.”, from our view, in case the AIoT RAN node connects with the new A-IoT CN node via AMF, the NG interface has to be used, in case the AIoT RAN node directly connects with a new A-IoT CN node (in T1, and in T2 RRC based solution), it is also possible to reuse NG interface via introducing a new set of procedures over NGAP.

Comparing with introducing a brand-new interface (and create lots of new specificaitons), there are more advantages to use NG interface between A-IoT RAN node and A-IoT CN.

Proposal 3: In Topology 1, the XX interface is the existing NG interface with set of new AIoT functionalities.

Proposal 4: In Topology 2, the XX interface is the the existing NG interface in case of NAS/UP based solution, or the existing NG interface with set of new AIoT functionalities in case of RRC based solution.

## “Control of A-IoT radio resources” by the A-IoT RAN node function

The following editor’s notes are related to radio resource control:

A-IoT RAN node function: A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device. Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

And then in the sections about topology 2, there are another three editor’s notes related radio resource control:

Editor’s Note 4: The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms. Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS. Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

There is a common understanding that the mentioned A-IoT radio resource control does not include the R2D scheduling and D2R scheduling which are controlled by the common reader function. Therefore, this discussion is only about radio resource control/allocation by the A-IoT RAN node function, not related to the R2D scheduling and D2R scheduling.

In Topology 1, it is straight forward that the A-IoT RAN uses the available resources based on OAM configuration.

In Topology 2, the A-IoT RAN node function is located in the A-IoT-enabled gNB, assuming available resource is known by the A-IoT-enabled gNB based on OAM configuration as well, it seems that the unclear part is how to allocate resource from the A-IoT-enabled gNB to the A-IoT-enabled UE.

Considering the following RAN2 agreement, how to allocate A-IoT radio resource by the A-IoT-enabled gNB is out of RAN3 scope.

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

Proposal 5: The A-IoT-enabled UE uses the A-IoT radio resources allocated by the A-IoT-enabled gNB. How to allocate A-IoT radio resource by the A-IoT-enabled gNB to the A-IoT-enabled UE is out of RAN3 scope.

## How many “readers” within one A-IoT RAN?

Considering that an A-IoT RAN node may support multiple TRPs, each TRP within the A-IoT RAN node can be a RAN reader, and thus an A-IoT RAN node can be regarded as multiple RAN Readers. In previous meetings, this seems to be a common understanding but it was never captured.

It is also noticed that it was mentioned during the discussion of devicing location (agenda item 16.4), but was commented that this should be determined in the Architecture discussion, therefore, we would like to propose to have an agreement on it.

Proposal 6: in Topology 1, the A-IoT RAN consists of one or multiple Readers.

## Editor’s Notes resolution

There are quite a few other Editor’s notes in RAN3 part of the TR, in this section, we would like to resolve these Editor’s Notes.

From our point of view, in section 6.4, the following Editor’s Notes should be removed:

| Section | Editor’s Note | Reason to remove |
| --- | --- | --- |
| 6.4 | Editor’s note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary. | In other sections of other WGs, the similar ENs to record the objective within the SID have already been removed |
| 6.4 | Editor’s Note 2: What functionalities are hosted by the 5GS for A-IoT is TBD. | Nothing to be done by RAN3. |
| 6.4 | A-IoT device: Equipment with characteristics outlined e.g. in TS 22.369 [10] and TR 38.848 [2]. Editor’s Note 3: Further details FFS, if any. | Nothing to be done by RAN3. |
| 6.4 | A-IoT RAN: Hosts certain functions for A-IoT as part of the functional split between RAN and CN. Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2. A-IoT CN: Hosts certain functions for A-IoT as of the functional split between RAN and CN. NOTE: the details of A-IoT CN are subject to SA2. Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2. | Already been coverd by existing discussion, no need to have these two Editor’s notes. |
| 6.4 | A-IoT radio: Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2. Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2. | Nothing to be done by RAN3. |
| 6.4 | Common reader function: A function that communicates with the A-IoT device by means of A-IoT radio. Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2. | Nothing to be done by RAN3. |

Proposal 7: Remove the Editor’s Notes 1, 2, 3, 4, 5, 6 and 8 in section 6.4.

## Comparision of the Topology 2 solutions

In table 1, we try to compare the three solutions of Topology 2 from different perspectives:

Table 1: Comparison of the Topology 2 solutions

|  |  | RRC based solution | NAS based solution | UP based solution |
| --- | --- | --- | --- | --- |
| UE Reader authorization |  | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. |
| A-IoT radio resource control at A-IoT-enabled gNB |  | Need to support, details up to RAN1/2. | Need to support, details up to RAN1/2. | Need to support, details up to RAN1/2. |
| Awareness of A-IoT services by the A-IoT-enabled gNB |  | Yes | No | No |
| A-IoT procedures exchanging |  | XXAP + A-IoT-enabled UE’s RRC | A-IoT-enabled UE’s NAS | A-IoT-enabled UE’s PDU session Need new layer above PDU layer. |
| WGs impact | RAN2 | High | Low | Unclear which group to define the new layer above PDU layer. |
|  | RAN3 | Low (Resue XXAP Topo1) | Low |  |
|  | CT1 | Low | High |  |
|  | SA2 | Medium | Medium | Medium |

Proposal 8: For Topology 2 solutions, capture the comparsion of them into BL pCR.

# Conclusion

In this contribution, we discussed the leftover RAN Architecture aspects, and got the following proposals:

About XX interface

Proposal 1: Change the description of XX interface to: Interface between the A-IoT RAN and the A-IoT CN in topology 1, and between A-IoT-enabled gNB and the A-IoT CN in topology 2, on which certain A-IoT specific functions are performed.

Proposal 2: In Topology 1, the signalling transport for XXAP is SCTP/IP.

Proposal 3: In Topology 1, the XX interface is the existing NG interface with set of new AIoT functionalities.

Proposal 4: In Topology 2, the XX interface is the the existing NG interface in case of NAS/UP based solution, or the existing NG interface with set of new AIoT functionalities in case of RRC based solution.

About A-IoT radio resource control

Proposal 5: The A-IoT-enabled UE uses the A-IoT radio resources allocated by the A-IoT-enabled gNB. How to allocate A-IoT radio resource by the A-IoT-enabled gNB to the A-IoT-enabled UE is out of RAN3 scope.

How many “readers” within one A-IoT RAN?

Proposal 6: in Topology 1, the A-IoT RAN consists of one or multiple Readers.

About editor’s notes

Proposal 7: Remove the Editor’s Notes 1, 2, 3, 4, 5, 6 and 8 in section 6.4.

About T2 solution Comparision

Proposal 8: For Topology 2 solutions, capture the comparsion of them into BL pCR.

The corresponding TP is provided in section 5.

# Reference

[1] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

[2] RP-240826 Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

# Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g. in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN in topology 1, and between A-IoT-enabled gNB and the A-IoT CN in topology 2, on which certain A-IoT specific functions are performed.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

A-IoT RAN node function:	A function that contains e.g. the control of the A-IoT radio resources used towards the A-IoT device.

NOTE x: “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function may need further discussion.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN. The A-IoT RAN consists of one or multiple Readers.

Figure 6.4.1-1 Logical system architecture for topology 1

In Topology 1, the XX interface is the existing NG interface with set of new A-IoT functionailities.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

For topology 1, the XXAP is terminated at an A-IoT RAN node.

NOTE: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision.

Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the A-IoT RAN node functions are FFS.

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function, which is able to communicate with the A-IoT enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

NOTE: 	Figure 6.3.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

In Topology 2 RRC based solution, the XX interface is the the existing NG interface in case of NAS/UP based solution, or the existing NG interface with set of new AIoT functionalities in case of RRC based solution.

NOTE x:	The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision.

The A-IoT-enabled UE uses the A-IoT radio resources allocated by the A-IoT-enabled gNB.

NOTE y: How to allocate A-IoT radio resource by the A-IoT-enabled gNB is out of RAN3 scope.

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 2: Depiction and further details of the options above are FFS

#### 6.4.2.x	Comparision on the Solutions for Topology 2

Table 6.4.2.2-1 illustrates the comparision on the Solutions for Topology 2 listed in clause 6.4.2.1.

Table 1: Comparison of the Topology 2 solutions

|  |  | RRC based solution | NAS based solution | UP based solution |
| --- | --- | --- | --- | --- |
| UE Reader authorization |  | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. | Performed by A-IoT CN and inform to the A-IoT-enabled gNB. |
| A-IoT radio resource control at A-IoT-enabled gNB |  | Need to support, details up to RAN1/2. | Need to support, details up to RAN1/2. | Need to support, details up to RAN1/2. |
| Awareness of A-IoT services by the A-IoT-enabled gNB |  | Yes | No | No |
| A-IoT procedures exchanging |  | XXAP + A-IoT-enabled UE’s RRC | A-IoT-enabled UE’s NAS | A-IoT-enabled UE’s PDU session Need new layer above PDU layer. |
| WGs impact | RAN2 | High | Low | Unclear which group to define the new layer above PDU layer. |
|  | RAN3 | Low (Resue XXAP Topo1) | Low |  |
|  | CT1 | Low | High |  |
|  | SA2 | Medium | Medium | Medium |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>