3GPP TSG-RAN WG3 Meeting #125-bis	R3-245071

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions

Agenda Item:	16.2

Source:	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi

Document for:	other

# Introduction

In last meeting, the following were listed as solutions for topology 2:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

The detailed protocol stack was not captured due to limit of discussion/time, considering all the previous discussions, in this paper, the details of these solutions are provided.

# Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1-1: RRC based solution of Topology 2

Solution 2: NAS based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport procedures over NGAP.

Figure 6.4.2.1-2: NAS based solution of Topology 2

Solution 3: UP based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU Session, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Figure 6.4.2.1-3: UP based solution of Topology 2

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>