3GPP TSG-RAN WG3 Meeting #125-bis	R3-245072

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) A-IoT Signalling and Procedures for Topology 1

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#125 meeting, the candidate procedures for A-IoT Inventory for Topology 1 and Topology 2 were captured in [1].

In this paper, we futher discuss the leftover aspects about A-IoT signalling and procedure for Topology 1.

# 2	Discussion

## 2.1	Information carried in Inventory Request from A-IoT CN

In RAN3#124 meeting, there were two items captured in the pCR, i.e. A-IoT Device Identification, and Scope of inventory request. And there is an Editor’s note about the A-IoT Device Identification:

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

  - In the RAN2#127 meeting, regarding the Inventory, there were some agreements, as follows:

Paging Triggers/Retransmission Agreements: - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices Visibility of AIOT Information Agreements: 1 At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

- Based on above RAN2 agreements, we can see that there is a need for the A-IoT RAN to store the A-IoT Device Identification, in order to allow the paging repetition over the A-IoT radio. Besides, based on currently discussion in SA2, RAN2 and RAN3, there is no need for the A-IoT RAN to interpret/process it.

- Proposal 1: The A-IoT RAN needs to store the A-IoT Device Identification in order to allow the paging repetition over the A-IoT radio, but there is no need for the A-IoT RAN to interpret/process it.

Proposal 2: To support RAN2 agreement, the Inventory Request from A-IoT CN should be able to include “targeted for one or more than one device”, and “approximate number of target devices”.

Considering that one A-IoT CN may connect with multiple Application servers, different Application servers may trigger Inventory procedures within a short period, and one A-IoT RAN may connect with multiple A-IoT CNs, different A-IoT CNs may trigger Inventory procedures within a short period as well. To be more robust, it is needed to include Inventory Session ID in the Inventory Request message from A-IoT CN, to enable the A-IoT RAN to distinguish different requests from A-IoT CN(s). While from A-IoT CN point of view, upon receiving the Inventory Response/Failure/Report message from the A-IoT RAN, it also needs to get the Session ID to distinguish the Inventory procedures.

Proposal 3: Includes the Inventory Session ID in the Inventory Request/Response/Failure/Report messages.

## 2.2	Completion of the Inventory procedure in Topology 1

In the overall Inventory procedure in Topology 1, there are one Inventory Request procedure and one or multiple Inventory Report procedure(s) over XXAP. In our view, when there is no further device performing random access and no further UL Data (Device ID) to be reported, there are two opions for the A-IoT RAN to indicate to the A-IoT CN that the overall Inventory procedure has been completed:

- Option 1: introduce a Complete Indication IE in the INVENTORY REPORT message

- Option 2: introduce an Inventory Complete procedure

Comparing of these two options, option 1 requires the A-IoT RAN to buffer the received data before sending the last INVENTORY REPORT message, option 2 allows both A-IoT RAN and A-IoT CN to trigger the completion of the procedure, therefore, we slightly prefer option 2. Note that after the Inventory Complete procedure, any stored context of the completed inventory service at the A-IoT RAN should be released.

Proposal 4:  Introduce an Inventory Complete procedure to support the completion of the inventory service.

## 2.3	Command in Topology 1

  - There are two Editor’s notes about Command:

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)

  - In the RAN2#127 meeting, there are some agreements, as follows:

Visibility of AIOT Information Agreements: 1 At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional 2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

- From above RAN2 agreements, we can see that there is a need for A-IoT RAN to differentiate command and inventory.

- Observation 1: There is a need for the A-IoT RAN to to differentiate command and inventory.

- In the RAN2#126 meeting, “inventory and command” case was agreed as the baseline. In this baseline, network first performs “paging” over A-IoT radio, the device sends its device ID to the network, and then the network sends command data to the device. In our view, the network should send DL data to the single device after the security checking at the A-IoT CN, therefore, the following command procedure should be adopted.

- Proposal 5: Support Command Transmission after Inventory procedure for command operation.

Figure 2. Command transmission after Inventory procedure in Topology 1

- To support the “command after inventory” solution, from RAN3 perspective, similar to NGAP DL NAS Transport and UL NAS Transport procedures, we need to introduce DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command Response.

  - Proposal 6: Introduce XXAP DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command response over XX interface.

## 2.4	Context management over CN-RAN interface

  - Device association establishment

As shown in the Figure 2, upon receiving the Command indication IE in the Inventory Request, the A-IoT RAN can allocate RAN Device XXAP ID for the device and provide it to the A-IoT CN in the Inventory Report, then the A-IoT CN allocates the CN Device XXAP ID to establish the XXAP device association. The XXAP device association could be used for e.g., parallel transactions for multiple devices.

- Proposal 7. To support command operation, it is needed to establish the RAN/CN Device XXAP ID to establish XXAP device association.

# 3	Conclusion

- In this contribution, we further discussed the signaling and procedures over CN-RAN interface for Topology 1, and got the following proposals:

- Observation 1: There is a need for the A-IoT RAN to to differentiate command and inventory.

- Proposal 1: The A-IoT RAN needs to store the A-IoT Device Identification in order to allow the paging repetition over the A-IoT radio, but there is no need for the A-IoT RAN to interpret/process it.

- Proposal 2: To support RAN2 agreement, the Inventory Request from A-IoT CN should be able to include “targeted for one or more than one device”, and “approximate number of target devices”.

- Proposal 3: Includes the Inventory Session ID in the Inventory Request/Response/Failure/Report messages.

- Proposal 4:  Introduce an Inventory Complete procedure to support the completion of the inventory service.

- Proposal 5: Support Command Transmission after Inventory procedure for command operation.

- Proposal 6: Introduce XXAP DL Command Transport procedure and UL Command Transport procedure to transmit the DL Command and UL Command response over XX interface.

- Proposal 7. To support command operation, it is needed to establish the RAN/CN Device XXAP ID to establish XXAP device association.

The corresponding TP for TR 38.769 is provided in section 5.

# 4. Reference

[1] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

# 5. TP for TR 38.769 v1.0.0

----------------Start of the Change----------------

## 6.5	Impacts on CN-RAN interface

### 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

NOTE 1:	The definition of this identification is out of RAN3 scope.

NOTE 2: The A-IoT RAN needs to store this identification to enable paging repetition over the A-IoT radio, but the A-IoT RAN does not need to interpret/process this identification.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

(3) Inventory Session ID (to distinguish different requests from A-IoT CN(s).)

(4) Assistance Information, which may include:

-	targeted for one or more than one device

-	approximate number of target devices

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

NOTE 3: It is up to SA2 whether device ID is sent transparent or not.

#### 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device. The A-IoT RAN is able to differentiate the type of request (command and inventory) from the A-IoT CN.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

### 6.5.2	Signaling and Procedures for Topology 1

#### 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

#### 6.5.2.2	Candidate procedures for A-IoT Command for Topology 1

Figure 6.5.2.2-1 illustrates overall call flow for command operation, i.e., Command Transmission is performed after Inventory procedure.

Figure 6.5.2.2-1. Command transmission after Inventory procedure for Topology 1

1.	A-IoT CN node triggers Inventory Request with A-IoT Device ID towards the relavent A-IoT RAN, and indicates that there will be command transmission for the device.

2.	The A-IoT RAN sends Inventory Response to the CN.

3.	The A-IoT RAN triggers inventory procedure over the A-IoT radio interface.

4.	The A-IoT device performs random access and provides UL Data (Device ID) to the A-IoT RAN.

5.	The A-IoT RAN sends the Inventory Report towards the CN with the UL Data (Device ID) received in step4. Based on the command indication received in step1, the A-IoT RAN allocates RAN Device XXAP ID for the device, and include it in the Inventory Report.

6.	The A-IoT CN node triggers DL Command Transport towards the A-IoT RAN. The A-IoT CN node allocates CN Device XXAP ID for the device, and provide the RAN/CN Device XXAP ID pair and the DL Data towards the A-IoT RAN, a command type may also be provided.

7.	The A-IoT RAN transmits the DL Data towards the A-IoT Device.

8.	The A-IoT Device transmits the UL Data towards the A-IoT RAN.

9.	The A-IoT RAN triggers UL Command Transport towards the A-IoT CN node, including the RAN/CN Device XXAP ID pair and the UL Data.

----------------End of the Change----------------