3GPP TSG-RAN WG3 Meeting #125bis	R3-245081

Hefei, China, 14th – 18th October 2024

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP to BLCR for TR 38.769) AIoT RAN architecture

Document for:	Other

1	Introduction

In this contribution, we’d like discuss the general aspects for topology 1 and topology 2 based on previous agreements and progress of SA2.

2	Discussion

Topology 1

According to the latest conclusion in TR 23.700-13[1] by SA2, it has been agreed to introduce a new AIoT NF to support Ambient IoT for Topology 1.

At least the following principles are agreed for the architecture to support topology 1: - A new core network function is introduced to support Ambient IoT. Editor's note: Whether the new core network function also applies to topology 2 is FFS.

In this topology, it is more efficient to establish a direct connection between AIoT RAN and AIoT AF without involving AMF for transferring AIoT related information, as non-UE associated operations are not required by AMF.

There are two possible deployment scenarios:

- if the AIoT NF can be deployed within AMF, then the XX interface can be NGAP.

- if the AIoT NF is deployed independently of AMF, the NGAP protocol can still be utilized. The interface management procedures can be reused between AIoT RAN and AIoT NF, the ambient IoT specific information can be introduced.

If new interfaces are introduced between AIoT RAN and AIoT NF, there may be two possible options:

- new logical P2P interface, which may require defining interface management similar to NGAP

- new service-based interface, where the registration of AIOT RAN services with NEF is necessary, and followed by discovery and selection performed by the AIOT NF, resulting in significant impacts on specifications.

Observation 1, SA2 agrees to introduce a new function support AIoT in topology1.

Observation 2, it’s more efficient to have direct communication between AIoT RAN and AIoT NF for T1.

Observation 3, introducing new interface (either P2P or SBI) may have more impacts than re-using NGAP.

Proposal 1, NGAP can be used for AIoT RAN and AIoT NF communication for T1.

Topology 2

For topology 2, although SA2 has not reached a conclusion yet, it is believed that communication between AIoT-enabled gNB and AIoT NF should go through AMF due to its association with specific UEs and knowledge of UE contexts. Connecting AIoT NF directly with AIoT-enabled gNB could lead to issues such as maintaining UE states and capabilities in two CN functions.

Observation 4, as the AIoT operation is related to a specific UE in T2, it’s better for AIoT NF communicate with the serving gNB of the UE via AMF.

Proposal 2, the AIoT NF communicate with the AIoT-enabled gNB communicate via AMF for T2.

For T2, three signalling solutions has been provided and discussed.

- RRC-based signalling solution

- NAS-based signalling solution

- UP-based signalling solution

From RAN3 perspective, we could only capture the connection related to gNB, but an overall view can help us to make decision. No matter which solution is used, there’re two main types of communication to support AIoT:

- AIoT operation information

- AIoT resource allocation related information

For RRC-based solution, exchange of AIoT operation information occurs among AIoT NF and AMF as well as between AMF and gNB before reaching UEs. And the resource allocation related information can be implied by gNB based on the AIoT operation information from AMF.

Figure 1 impacted protocols for NAS-based solution

For NAS-based solution, the AIoT operation information are exchanged over NAS, the gNB has no idea of it, however, the gNB is responsible for AIoT radio resource allocation, we think either the AMF or the UE can request the gNB for AIoT resource allocation via NGAP message or RRC message.

Figure 2 impacted protocols for NAS-based solution

For UP-based solution, the AIoT operation related information are exchanged over UP, same issue as NAS-based solution, gNB does not aware of the AIoT operation but it should be responsible for AIoT resource allocation, one possible way is that the UE request the AIoT resource via RRC message.

Figure 3 impacted protocols for UP-based solution

Observation 5, For topology 2, there’re two types of communication need to be specified.

-AIoT operation information

- AIoT resource allocation related information

Observation 6, for RRC-based solution, the NGAP specification needs to support for AIoT operation information transfer and utilize this information for AIoT resource allocation.

Observation 7, for NAS-based solution, gNB is not aware of the AIoT operation for the UE reader, the NGAP and/or RRC can be enhanced to support AIoT resource allocation.

Observation 8, for UP-based solution, gNB and AMF are not aware of AIoT operation for the UE reader, the RRC can be enhanced to support AIoT resource allocation.

Proposal 3, RAN3 should assess the potential impacts on AIoT operation information transmission and resource allocation for each solution.

3	Conclusion

In this contribution, we discussed the general aspects of the AIoT and have the following proposals.

Topology 1

Observation 1, SA2 agrees to introduce a new function support AIoT in topology1.

Observation 2, it’s more efficient to have direct communication between AIoT RAN and AIoT NF for T1.

Observation 3, introducing new interface (either P2P or SBI) may have more impacts than re-using NGAP.

Proposal 1, NGAP can be used for AIoT RAN and AIoT NF communication for T1.

Observation 4, as the AIoT operation is related to a specific UE in T2, it’s better for AIoT NF communicate with the serving gNB of the UE via AMF.

Proposal 2, the AIoT NF communicate with the AIoT-enabled gNB communicate via AMF for T2.

Topology 2

Observation 5, For topology 2, there’re two types of communication need to be specified.

-AIoT operation information

- AIoT resource allocation related information

Observation 6, for RRC-based solution, NGAP spec needs to support AIoT operation information transfer, and it may use the AIoT operation information for AIoT resource allocation.

Observation 7, for NAS-based solution, gNB is not aware of the AIoT resource allocation, the NGAP and/or RRC can be enhanced to support AIoT resource allocation.

Observation 8, for UP-based solution, gNB and AMF are not aware of the AIoT resource allocation, the RRC can be enhanced to support AIoT resource allocation.

Proposal 3, RAN3 should assess the potential impacts on AIoT operation information transmission and resource allocation for each solution.

4	Reference

[1]	 TR 23.700-13 V1.0.0

5	Annex (TP to R3-244837 for TR 38.769)

<<<<<<<<<<<<<<<<<<<< Change Start >>>>>>>>>>>>>>>>>>>>

### 6.3.2 Support of Topology 2

Figure 6.3.2-1 depicts a logical system architecture for topology 2, wherethe Common reader function is located at anAIoT-enabled UE, and the AIoT RAN node function is located at an AIoT-enabled gNB.

The following definitions apply:

AIoT-enabled gNB: a gNB supporting AIoT RAN node function, which is able to communicate with the AIoT enabled UE via NR Uu interface.

AIoT-enabled UE: a UE supporting Common reader function, which is able to communicate with the AIoT device via the AIoT radio interface.

Figure 6.3.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.3.2-1 doesn’t illustrate the protocol between AIoT enabled UE and AIoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The AIoT CN could include AMF and AIoT related functions. This is up to SA2 decision.

Editor’s Note 4:	The AIoT enabled gNB performs radio resource management for AIoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

#### 6.3.2.1 Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying AIoT upper layer information:

- RRC based solution. With this solution, AIoT CN applies AIoT upper layer information explicitly over XXAP signaling. AIoT upper layer information is then relayed explicitly to/from the AIoT-enabled UE via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted over AIoT-enabled UE's NAS.

- UP based solution. With this solution, there is no explicit termination of AIoT upper layer information at AIoT-enabled gNB. AIoT upper layer information is transmitted as AIoT-enabled UE's user plane data.

Editor’s note 1: how to enable radio resource control, i.e. trigger AIoT RAN node functions for above solutions is FFS.

Editor’s note 2: depiction and further details of the options above are FFS.

For RRC-based solution, the NGAP specification needs to support for AIoT operation information transfer and utilize this information for AIoT resource allocation.

For NAS-based solution, gNB is not aware of the AIoT operation for the UE reader, the NGAP and/or RRC can be enhanced to support AIoT resource allocation.

For UP-based solution, gNB and AMF are not aware of AIoT operation for the UE reader, the RRC can be enhanced to support AIoT resource allocation.

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>