3GPP TSG-RAN WG3 Meeting #125bis	R3-245083

Hefei, China, 14th – 18th October 2024

Agenda item:	16.4

Source:	Xiaomi

Title:	(TP to BL CR for TR 38.769) Locating AIoT device

Document for:	Other

1	Introduction

In previous meetings, we discussed how to locate AIoT device in different granularities, however, the granularities other than “reader ID level” are hard to progress if there’s no RAN1 inputs.

In this contribution, we’d like to focus on “reader ID” level locating issues based on the following progress.

RAN3#125: For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device. The encoding of AIoT RAN node ID needs to be further checked. Other granularities are FFS, e.g., TRP ID. For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting.

2	Discussion

## Definition of Reader ID in T1

The reader ID in T1 is believed to be able to refer to the TRP ID, and the definition of TRP in TS 38.305 is provided below.

Reception Point (RP): A set of geographically co-located receive antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one UL-SRS-only RP. Reception Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a UL-SRS-only RP, etc. One cell can include one or multiple reception points. For a homogeneous deployment, each reception point may correspond to one cell. Transmission Point (TP): A set of geographically co-located transmit antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one DL-PRS-only TP. Transmission Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a DL-PRS-only TP, etc. One cell can include one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell. Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

The AIoT RAN in T1 may consist of one or more sets of antennas deployed in different locations, each covering a distinct area. A set of geographically co-located receive antennas covering a specific area can be considered as the Transmission/Reception Point (TRP) of a reader. Therefore, the TRP ID can serve as the reader ID. In case there is a need to introduce a new reader ID, it can have a similar definition as that of the TRP ID. If reusing the TRP ID, the TRP of a reader can be defined as follows: The TRP represents a set of geographically co-located antennas supporting both AIoT Transmission Point and/or AIoT Reception Point functionalities.

Observation 1, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 2, RAN node ID and TRP ID can be used to identify a specific reader that supports the functionality of AIoT Radio Transmission Point and/or AIoT Reception Point.

## Improving AIoT operation

The following use cases was agreed in previous RAN3 meeting:

improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

The target device(s) may be surrounded by multiple readers, which can either be AIoT RAN readers or UE readers. The AIoT NF may need to identify appropriate reader(s) that can contact with the target AIoT device for further operations.

The following figure shows an example for how the AIoT NF knowns which readers can connect with the target AIoT device.

a) The AIoT NF can send inventory paging to the readers in a specific area. Subsequently, the readers independently initiate Inventory procedures to establish communication with the target devices, as depicted in Figure 1(a).

b) Upon receiving a response from the target device. The inventory report message can include the reader ID and optionally the RSSI of the backscatter signal received from the target device as shown in Figure 3(b). This enables AIoT NF find to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

Figure 1 solution 2 to find an appropriate “reader” close to the A-IoT device

Observation 2, In scenarios where a target device is surrounded by multiple readers, the inclusion of reader IDs along with the respective measurements of the received backscatter signals can improve the further AIoT operations.

Proposal 3, the reader ID and the corresponding receiving signal strength from the target device can be included in the AIoT operation response message, e.g. inventory report, command response to improve the AIoT operation.

## Providing location information

The following use cases was agreed in previous RAN3 meeting:

providing location information to the consumer of the AIoT service.

For topology 1, providing location information can be interpreted as providing the reader’s location information. There are two options:

- Option 1: Similar to TRP information exchange, the gNB provides TRP information to AIoT NF in advance, including geo-coordinates for each TRP, using non-AIoT operation-associated signalling.

- Option 2: The TRP’s geo-coordinates can be included in the inventory/command response message.

For option 1, the possible signaling flow can be as follows:

In steps 1-2, the AIoT RAN can provide a list of TRP IDs that support AIoT operations, along with the corresponding geo-coordinates for each TRP. This information can help the AIoT NF decide which TRP to use for a specific AIoT operation. For example, the AIoT NF can determine that the target device is near to a specific TRP based on the service request and the TRP’s geo-coordinates.

In steps 3-5, the AIoT RAN performs AIoT operations, such as inventory, as requested by the AIoT NF. It can include the TRP ID, representing the reader ID, in the response or report message. The CN can then determine the reader’s location based on the TRP ID and its corresponding geo-coordinates.

For option 2, the possible signaling flow can be as follows:

In steps 1-3, the AIoT RAN performs AIoT operations, such as inventory, as requested by the AIoT NF. It can include the TRP ID, representing the reader ID, and the corresponding geo-coordinates of the TRP in the response or report message. This allows the CN to directly determine the reader’s location.

Both options are viable. Option 1 may reduce signaling overhead, as the AIoT RAN does not need to report the TRP’s geo-coordinates every time. Additionally, obtaining the TRP information in advance can be helpful for reader selection.

For topology 2, the information may vary depending on the signaling solution used.

RRC-based signaling solution: The serving gNB of the UE reader is aware of the AIoT operation but typically does not have right to obtain the UE’s GNSS location information. From the serving gNB’s perspective, it can only provide the user location information of the UE reader, including cell and TA information, to the CN.

In steps 1-5, the UE reader performs the AIoT operation as requested. In step 5, the serving gNB includes the XXAP UE ID and user location information, such as CGI, TAI, and Age of Location (existing user location information can be reused).

In step 6, if the CN requires more precise UE location information, it may initiate location services to obtain the geo-coordinates of the UE reader.

For NAS/UP-based solution, either the UE reader or the CN may initiate location service to obtain more precise UE reader location information.

In steps 1, 2, and 4, the UE reader performs the AIoT operation as requested. In step 4, the UE reader may include the UE ID and user location information, such as serving cell, TAC information, and geo-coordinates if step 3 is performed.

In step 6, the CN can initiate location services to obtain the geo-coordinates of the UE reader. The location service can be performed using existing mechanisms.

Proposal 4, for topology 1, the TRP ID can be included in inventory/command response/report message.

Proposal 5, RAN3 discuss the following options for AIoT NF to obtain the TRP’s geo-coordinates.

- Option 1: the gNB provide TRP’s geo-coordinates in advance using non-AIoT operation-associated signalling

- Option 2: the gNB provide TRP’s geo-coordinates in the inventory/command response/report message.

Proposal 6, for topology 2, if RRC-based solution is used, the gNB provides the user location information of the UE reader in the inventory/command response message to CN.

Proposal 7, for topology 2, if NAS/UP solution is used, UE reader can provide its location information including cell, TAC, geo-coordinates in the NAS/UP inventory/command response message to CN.

Proposal 8, RAN3 agrees the TP for TR 38.386 in the Annex.

3	Conclusion

In this contribution, we discussed how to locating an AIoT device, and have the following observations and proposals.

## Definition of Reader ID in T1

Observation 1, The TRP of a reader can be regarded a set of geographically co-located antennas supporting AIoT Transmission Point and/or AIoT Reception Point functionalities.

Proposal 2, RAN node ID and TRP ID can be used to identify a specific reader that supports the functionality of AIoT Radio Transmission Point and/or AIoT Reception Point.

## Improving AIoT operation

Observation 2, In scenarios where a target device is surrounded by multiple readers, the inclusion of reader IDs along with the respective measurements of the received backscatter signals can improve the further AIoT operations.

Proposal 3, the reader ID and the corresponding receiving signal strength from the target device can be included in the AIoT operation response message, e.g. inventory report, command response to improve the AIoT operation.

## Providing location information

Proposal 4, for topology 1, the TRP ID can be included in inventory/command response/report message.

Proposal 5, RAN3 discuss the following options for AIoT NF to obtain the TRP’s geo-coordinates.

- Option 1: the gNB provide TRP’s geo-coordinates in advance using non-AIoT operation-associated signalling

- Option 2: the gNB provide TRP’s geo-coordinates in the inventory/command response/report message.

Proposal 6, for topology 2, if RRC-based solution is used, the gNB provides the user location information of the UE reader in the inventory/command response message to CN.

Proposal 7, for topology 2, if NAS/UP solution is used, UE reader can provide its location information including cell, TAC, geo-coordinates in the NAS/UP inventory/command response message to CN.

Proposal 8, RAN3 agrees the TP for TR 38.386 in the Annex.

4	Reference

[1] R3-244840 (TP for TR 38.769) Locating Ambient-IoT device

5	Annex (TP to BL CR for TR 38.769)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.8	Locating Ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT NF sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT NF needs further study.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

### 6.8.2	Providing Location Information

For topology 1, providing location information can be interpreted as providing the reader’s location information.

Option 1, the AIoT RAN node provide the TRP information to AIoT NF including the geo-coordinates of each TRP, using non-AIoT operation-associated signalling in step 2. If there’s AIoT operation, the AIoT RAN node include the TRP ID in the AIoT operation response or report in step 5.

Figure 6.8.2-1: Providing reader TRP location information for T1 (option 1).

Option 2, If there’s AIoT operation, the AIoT RAN node include the TRP ID and the corresponding geo-coordinates of the TRP in the AIoT operation response or report in step 3.

Figure 6.8.2-2: Providing reader TRP location information for T1 (option 2).

For topology 2, the information may vary depending on the signaling solution used.

RRC-based signaling solution: The serving gNB of the UE reader provide the user location information of the UE reader, including cell and TA information, to the CN.

Figure 6.8.2-3: Providing reader location information for T2 (RRC-based solution).

For NAS/UP-based solution, UE reader may include the UE ID and user location information, such as serving cell, TAC information, and geo-coordinates if step 3 is performed, either the UE reader or the CN may initiate location service to obtain more precise UE reader location information.

Figure 6.8.2-4: Providing reader location information for T2 (NAS/UP-based solution).

<<<<<<<<<<<<<<<<<<<< Change Ends>>>>>>>>>>>>>>>>>>>>