3GPP TSG-RAN WG3 Meeting #125bis	R3-245085

Hefei, China, 14th – 18th October 2024

Agenda item:	14.3

Source:	Xiaomi

Title:	Support of Regenerative Payload

Document for:	Other

1	Introduction

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Inactive UE support

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Feeder link switch issue

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

In this contribution, we’d like to further discuss the Support of Regenerative payload based on RAN3’s below agreements.

The following topics are discussed in this paper.

- NG/Xn connection handling

- Inactive UE support

- Mapped cell ID

2	Discussion

### NG connection handling

In case of non-GEO network, the connection between gNB onboard and AMF on ground may be disconnected due to the movement of the satellite. How to handle the NG connection has been discussed in RAN3, and agreed NG Removal/Setup can be used for Regenerative Payload NG interface management.

Another option for suspending NG connections has also been discussed but not agreed. One argument is that it is not worthwhile to maintain certain connections or configuration contexts since a satellite in LEO typically takes 90-110 minutes to complete one orbit of the earth. However, we do not believe that the time it takes for a satellite to complete one orbit of the earth is equivalent to the disconnection time of NG connection between gNB onboard and an AMF. The NG connection is a logical connection between two nodes, which relies on the transport connection between the satellite and on-ground gateway. We believe that the following two scenarios shows that the NG connection can remain active even when there is a feeder link switch from one gateway to another:

- In case 1, as shown in figure 1, even if gNB1 loses its feeder link connection with NTN GW1, if there are intersatellite links between gNBs onboard, the NG connection between gNB1 and AMF can still active.

- In case 2, as shown in figure 2, gNB1 may still connect with AMF via NTN GW2.

Figure 1 NG connection via ISL                                                      Figure 2 NG connection with another GW

Another argument is that the gNB onboard needs to store the interface context of AMFs worldwide. However, we disagree with this argument as introducing connection suspend and resume does not imply that the gNB must keep the connections with all AMFs globally and store all their contexts. According to satellite regulations, a satellite from country A cannot be utilized in the coverage area of country B, indicating that the satellite will not connect to all the AMFs in all coverage areas. Therefore, we propose using NG suspend/resume as an optional feature for operators when dealing with frequently removed or set up NG connections. In order to avoid redundant transmissions, node-level configurations can be stored in both nodes. To minimize specification impacts, it can be implemented as an optional indicator or flag in NG removal requests. For example, if this indicator is included in the NG removal request, the AMF can store node configuration information.

Observation 1, The NG connection can remain active through intersatellite links or other gateway connections, therefore, completion time for one orbit of Earth by a satellite cannot accurately represent the disconnection time of a NG connection.

Observation 2, Due to configuration or regulatory constraints, there are certain areas where communication between gNB onboard and AMFs is not possible, hence storing all contexts within its coverage is unnecessary.

Observation 3, Supporting both release of all interface contexts and retention of some allows informed decisions by gNB on whether to release or suspend a NG connection.

Proposal 1: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

### Xn connection handling

We believe that it is feasible for the gNB onboard a satellite to establish an Xn connection with the gNB on the ground through the feeder link. This aspect of the Xn connection via the feeder link appears to be omitted in the figure presented in the BL CR. To provide a more comprehensive representation of the scenario, we propose to add the Xn connection via the feeder link to the figure in the BL CR.

Proposal 2, add Xn connection via feeder link between onboard gNB and on-ground gNB in the figure in BL CR for TS 38.300.

### Inactive UE support

The topic of inactive UE support was discussed in the last RAN3 meeting, and most companies believe that the existing mechanism is sufficient. However, we believe that the current mechanism is primarily designed for TN and may not be well-suited for NTN. This mismatch could potentially lead to more frequent failures, negatively impacting a large number of users and resulting in poor user experience as well as reduced network efficiency.

When a UE transitions to the RRC_INACTIVE state and the anchor gNB is located on a non-GEO satellite, if UE resumes, the existing retrieve UE context procedure can be re-used, it is crucial to note that the existing retrieval procedure is contingent upon the availability of an inter-satellite link between the anchor gNB and the receiving gNB. As shown the below figure, at T1, orange satellite sends UE to RRC Inactive state and keeps UE context, when UE resumes, the orange satellite that have UE context may move to other side of the earth, the blue satellite may receive UE’s resume request but it may not able to communicate with the orange satellite.

If reusing existing mechanism, the following scenarios may occur.

Scenario 1, when UE initiates resume, as stated in TS 38.300[1], if the new serving gNB cannot retrieve UE context, it can either reject the UE or setup new RRC connection, the call flows can be as follows:

For the reject case, the user experience is negatively impacted by the service latency.

For the RRC setup case, the serving gNB (LEO1) may store the UE context for a long time even through the UE has already connected to the network via other satellites, this wastes storage space and network efficiency on the satellite.

Scenario 2, in case of the network-initiated resume, e.g., there may DL signalling or DL data, as specified in TS 38.300[2], the call flow can be as follows:

If the last serving gNB receives DL data/signaling, it cannot send the RAN paging to other satellites if there’s no ISL or ISL is not available, resulting in a failure of the RAN paging. In such cases, as specified in TS 23.501[2], the last serving gNB may request the CN to release the UE context.

In addition, if NG removal is introduced, if the “last serving gNB” releases all the UE contexts, if there’s DL data/signalling, the CN will initiate the CN paging to the UE. The drawback for the case is that the UE does not know network release its context, it will act as inactive UE until it receives CN paging. The UE's power is wasted as it remains in listening RAN paging, even though RAN paging never occurs.

All above cases can be solved by existing system but they have drawbacks for UEs and networks. By re-using existing mechanism that designed for TN network does not fully and well support inactive UE mobility in context of NTN.

Observation 4, The following issues may arise when applying existing mechanisms for RRC_INACTIVE UE mobility:

- UE may have a poor experience if the receiving gNB rejects the RRC resume request due to retrieval UE context failure, which is likely to occur frequently in NTN systems.

- The last serving gNB may not be aware that the UE has connected to a new gNB and continues to maintain an irrelevant UE context.

- UE have different RRC states compared to the network, resulting in misalignment of their behaviour

Observation 5, reusing existing mechanisms designed for TN networks does not adequately support inactive UE mobility within the context of NTN.

Proposal 3, RAN3 to discuss NTN-specific solutions to address inactive UE mobility challenges in NTN regenerative payload.

One possible solution can be the UE context can be anchored on-ground before being transitioned to the RRC_Inactive state by the satellite-gNB. The following example illustrates this possibility.

The serving gNB onboard can proactively relocate the UE context to an on-ground gNB (referred to as an anchor gNB) if it decides to send the UE to RRC_INACTIVE state, as depicted in step 1 and step 2 in the figure above. This anchor gNB does not require a radio connection with the UE and may be located within the same area as the UE, enabling anchoring of the UE context on ground. The selection of the anchor gNB can vary based on implementation. In step 1, the serving gNB transfers the UE context to the anchor gNB. In step 2, the anchor gNB responds with information for resuming communication with the UE, such as providing I-RNTI generated by itself. The existing RRC release message can be reused without any impacts, only its contents is generated by the anchor gNB.

After the UE transitions to the RRC_INACTIVE state, the information required for RRC resume on the UE side is generated by the anchor gNB. If the UE initiates RRC resume, the receiving gNB can retrieve UE context from the anchor gNB on ground. In case of DL data/signalling, they are sent by CN to the anchor gNB so that it can initiate RAN paging. All existing signalling can be reused.

Observation 6, retaining the UE context on-ground could be a potential solution to support inactive UEs in the context of NTN regenerative payload

Observation 7, the serving gNB can proactively transfer UE context to an on-ground gNB before transitioning the UE to RRC_INACTIVE state, which may have impacts on RAN3 but does not impact UEs.

### Issue 3, mapped cell ID

In Non-GEO Satellites, due to their orbital movement, their satellite beams cover different portions of that area. In R17, for transparent payload, mapped cell ID is introduced to solve the issue. And RAN3 agrees this mapped cell ID can be reused in regenerative payload.

We have concerns on whether the mapped cell ID have relation to the gNB ID of the moving satellite-gNB in NTN regenerative payload.

If the mapped cell ID has no relation to the gNB ID of the moving satellite-gNB, we think there’s no issue, the mapped cell ID can be planed and setup by OAM.

If the mapped cell ID includes the gNB ID of the moving satellite-gNB, meanwhile it should represent the fixed area on-ground, the cell ID will be changed as the satellite moves, we have concerns on whether the cell ID can be enough to represent all the areas that the satellite covers. According to the NCGI content as below, the minimum cell number can be assigned to a gNB is range 0-15 in case 4 bits is allocated to cell ID.

No matter how the mapped cell ID designs, we want to make it clear, so we propose RAN3 to discuss this issue.

Observation 8: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Observation 9, if the mapped cell ID include the gNB ID, the cell ID may not be enough to represent all the fixed areas on ground.

Proposal 4, RAN3 discuss whether the mapped cell ID has relation to the gNB ID.

3	Conclusion

Based on the discussion, we have the following observations and proposals.

### NG/Xn connection handling

Observation 1, The NG connection can remain active through intersatellite links or other gateway connections, therefore, completion time for one orbit of Earth by a satellite cannot accurately represent the disconnection time of a NG connection.

Observation 2, Due to configuration or regulatory constraints, there are certain areas where communication between gNB onboard and AMFs is not possible, hence storing all contexts within its coverage is unnecessary.

Observation 3, Supporting both release of all interface contexts and retention of some allows informed decisions by gNB on whether to release or suspend a NG connection.

Proposal 1: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

Proposal 2, add Xn connection via feeder link between onboard gNB and on-ground gNB in the figure in BL CR for TS 38.300.

### Inactive UE support

Observation 4, The following issues may arise when applying existing mechanisms for RRC_INACTIVE UE mobility:

- UE may have a poor experience if the receiving gNB rejects the RRC resume request due to retrieval UE context failure, which is likely to occur frequently in NTN systems.

- The last serving gNB may not be aware that the UE has connected to a new gNB and continues to maintain an irrelevant UE context.

- UE have different RRC states compared to the network, resulting in misalignment of their behaviour

Observation 5, reusing existing mechanisms designed for TN networks does not adequately support inactive UE mobility within the context of NTN.

Proposal 3, RAN3 to discuss NTN-specific solutions to address inactive UE mobility challenges in NTN regenerative payload.

Observation 6, retaining the UE context on-ground could be a potential solution to support inactive UEs in the context of NTN regenerative payload

Observation 7, the serving gNB can proactively transfer UE context to an on-ground gNB before transitioning the UE to RRC_INACTIVE state, which may have impacts on RAN3 but does not impact UEs.

### mapped cell ID

Observation 8: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Observation 9, if the mapped cell ID include the gNB ID, the cell ID may not be enough to represent all the fixed areas on ground.

Proposal 4, RAN3 discuss whether the mapped cell ID has relation to the gNB ID.

4	Reference

[1] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

[2] TS 23.501, 5G; System Architecture for the 5G System