3GPP TSG-RAN WG3 Meeting #125bis	R3-245089

Hefei, China, from 14th to 18th Oct 2024

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in AI/ML-assisted Network Slicing, as captured in TR 38.743 [2], we present our analysis of the normative work for the network slicing use case. Additionally, the corresponding TP is included.

- Discussion

  - Predicted information exchange

As discussed during the Rel-19 SI phase, the deployment to support AI/ML-assisted Network Slicing is similar to the Rel-18 use cases.

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Therefore, only the Xn interface will be impacted to support the exchange of predicted information for AI/ML-based network slicing. The existing DATA COLLECTION INITIATION and DATA COLLECTION REPORTING procedures can be utilized for the exchange of predicted information in AI/ML-assisted network slicing.

The predicted information exchanged over Xn interface to support AI/ML assisted Network Slicing, as concluded in TR 38.743 [2], involves predicted radio resource status per slice, and predicted slice available capacity.

- Following information is introduced exchanged over existing DATA COLLECTION INITIATION and DATA COLLECTION REPORTING procedures:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

In the DATA COLLECTION REQUEST message, the requested node should configure the slice information when requesting slice-level predicted information.

Potential Implementation:

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |

- Introduce Slice To Report List IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

The DATA COLLECTION RESPONSE message is used to indicate which requested information failed to be initiated. Now, with the introduction of slice-level information, the response message also needs to include an indication of the requested slice-level information that cannot be reported.

- Introduce Slice Measurement Failure Cause List IE to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

Regarding the DATA COLLECTION UPDATE message, the current specification already includes the Predicted Radio Resource Status IE, which refers to the legacy Radio Resource Status IE. This IE already includes the Slice Radio Resource Status. Additionally, semantic description captured is that the Predicted Radio Resource Status IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE.

Therefore, to include the predicted radio resource status per slice in the DATA COLLECTION UPDATE message for Rel-19, the semantic description of the Predicted Resource Status IE needs to be modified as follows:

“The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

- Modify the description of the Predicted Resource Status IE to “The IE includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

Next, predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE in the DATA COLLECTION UPDATE message and refers to the Slice Available Capacity IE

- Predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE and refers to the Slice Available Capacity IE

As specified during Rel-18 phase, measured radio resource status per slice and measured slice available capacity can be achieved via legacy resource status procedures instead of introducing new IEs in the DATA COLLECTION INITIATION and DATA COLLECTION REPORTING procedures.

- Measured radio resource status per slice and measured slice available capacity can be achieved via legacy resource status procedures

  - Per Slice UE performance

In Rel-18, RAN3 agreed to introduce UE-level performance feedback to evaluate the performance of decisions made after handover. Currently, UE performance feedback includes metrics such as Average UE Throughput (UL/DL), Average Packet Delay(UL/DL), and Average Packet Loss (DL). During the Rel-19 SI phase, it was acknowledged that finer granularity UE performance feedback should be introduced for specific slice.

As described in TS 28.552 [3], average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

5.1.1.1	Packet Delay

5.1.1.1.1	Average delay DL air-interface

a)	This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.1.3	Average delay UL on over-the-air interface

a)	This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

5.1.1.3	UE throughput

5.1.1.3.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.1.3.3	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

5.1.3.1.3	DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI.

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

When a UE initiates a PDU Session, the UE selects an appropriate S-NSSAI based on the application requirements to specify the needed network slice. The network uses the S-NSSAI to determine which slice is best suited to carry the PDU Session. For example, if a PDU Session is intended for eMBB services, it might select an S-NSSAI that identifies a network slice specifically configured for high-bandwidth, low-latency services. A PDU Session is mapped to a specific network slice.

Therefore, following option is proposed to reflect the slice-level UE performance:

-	Option 1: UE performance per S-NSSAI per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >S-NSSAI |  |  |  |  |
| >Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >Average Packet Delay | O |  | 9.2.3.187 |  |
| >Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 2: UE performance per QoS Flow per PDU Session

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDU Session List |  | 0..1 |  |  |
| >PDU Session Item |  | 1..<maxnoofPDUSessions> |  |  |
| >PDU Session ID | M |  | 9.2.3.18 |  |
| >QoS Flow List |  | 1 |  |  |
| >>QoS Flow Item |  | 1..<maxnoofQoSFlows> | 9.2.3.10 |  |
| >>QoS Flow ID |  |  |  |  |
| >>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

-	Option 3: UE performance per S-NSSAI

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| >>>Average Packet Delay | O |  | 9.2.3.187 |  |
| >>>Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

- RAN3 is kindly asked to discuss which option can be accepted to reflect the slice UE performance:

      - UE performance per S-NSSAI per PDU Session

      - UE performance per QoS Flow per PDU Session

      - UE performance per S-NSSAI

3	Conclusion

Following is our proposals and observations:

Predicted information exchange:

- Following information is introduced exchanged over existing DATA COLLECTION INITIATION and DATA COLLECTION REPORTING procedures:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

- Introduce Slice To Report List IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

- Introduce Slice Measurement Failure Cause List IE to indicate the predicted radio resource status per slice or predicted slice available capacity can not be initiated.

- Modify the description of the Predicted Resource Status IE to “The IE includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and the Slice Radio Resource Status List IE.”

- Predicted slice available capacity IE is introduced in Cell Info Result for Data Collection Item IE and refers to the Slice Available Capacity IE

- Measured radio resource status per slice and measured slice available capacity can be achieved via legacy resource status procedures

Per Slice UE performance:

- Average packet delay UL/DL, average UE throughput (UL/DL),and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

Proposal 7	RAN3 is kindly asked to discuss which option can be accepted to reflect the slice UE performance:

1.	UE performance per S-NSSAI per PDU Session

2.	UE performance per QoS Flow per PDU Session

3.	UE performance per S-NSSAI

Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 28.552