3GPP TSG-RAN WG3 Meeting #125bis	R3-245095

Hefei, China, from 14th to 18th Oct 2024

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on AI/ML terminology

Document for:	Discussions & Approval

1	Introduction

In previous RAN3 meetings, there were extensive discussions on the terminology related to AI/ML in NG-RAN, and the corresponding CRs [1][2] were agreed upon. Additionally, LS [3] was approved to inform SA5 that AI/ML model training and inference follow the definitions referenced in TS28.105. SA5 has now responded to the LS [4], providing further clarification on the terminologies in their specification.

However, we find some misalignment remains across working groups (WGs). To address this, we present our analysis and understanding of the AI/ML terminology, along with proposed solutions for alignment.

2	Discussion

In the updated TS28.105, following is the definitions for model training and model inference:

ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

NOTE 5:	the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

From this definition, it can be seen that SA5 makes a detailed distinction between the terms "AI" and "ML." For model training, only ML (machine learning) models are trained, while for inference, both AI algorithms and trained ML models can be used for inference.

- SA5 makes a detailed distinction between the terms "AI" and "ML."

From perspective of RAN3, following is the definition for model training and model inference captured in TR37.817 [6].

-	AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

-	AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

-	AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

Also, from perspective of RAN1, the following definitions are captured in TR38.843:

- AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

- AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

- RAN1 and RAN3 use the prefix 'AI/ML' for all terminology related to AI/ML definitions.

Therefore, with the respect to the training features, misalignment exits between “AI/ML” and “ML”. To address the misalignment above, following solutions are proposed:

- Solution 1: Update the RAN3 spec TS38.300 and TS38.401 to use “ML training” instead of “AI/ML training”. And send the LS to SA5 about RAN3’s decision.

- Solution 2: Send the LS to SA5 to let them clarify the difference between “ML training” and “AI/ML training”.

- RAN3 is kindly asked to discuss which solution is accepted:

      - Update the RAN3 specs TS38.300 and TS38.401 to use “ML training” instead of “AI/ML training”.

      - Send the LS to SA5 to let them clarify the difference between “ML training” and “AI/ML training”.

Both "ML training" and "AI/ML training" can be used depending on the specific context, but their meanings are slightly different.

- ML training: Refers specifically to the training process of machine learning (ML).

- AI/ML training: Emphasizes that both artificial intelligence (AI) and machine learning (ML) are included. This is usually used in a broader context, especially when discussing different components of AI technology, with machine learning being a part of AI.

While the majority of AI systems today are based on ML models, the broader concept of AI training can encompass other aspects beyond ML, such as knowledge-based AI and reinforcement learning.

And, it was agreed that AI/ML algorithms is out of 3GPP scope. To avoid discussing the specific implementation details of AI/ML technologies, a more neutral approach is to use the AI/ML prefix.

- Suggest using the 'AI/ML' prefix for all terminology and proceed with Solution 2.

The reply LS are prepared in [8].

Reference

[1] R3-243928, Correction on definition of AI/ML terminology, Samsung

[2] R3-243927, Stage 2 corrections for AI/ML for NG-RAN, Huawei

[3] R3-243948, LS on terminology definitions for AI-ML in NG-RAN, RAN3

[4] R3-245023, LS on terminology definitions for AI-ML in NG-RAN, SA5

[5] TS28.105

[6] TR37.817

[7] TR38.843

[8] R3-245096, Reply LS on terminology definitions for AI-ML in NG-RAN, ZTE Corporation, Qualcomm, CMCC