3GPP TSG-RAN WG3 Meeting #125bis	R3-245097

Hefei, China, from 14th to 18th Oct 2024

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Positioning (Case 3a)

Document for:	Discussions & Approval

1	Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3. In this contribution, we provide our understanding on the case 3a about assisted AI/ML positioning.

2	Discussion

As described in TR 38.843 [2], the AI/ML assisted positioning (case 3a) is illustrated below:

Figure 1. Assisted positioning with multi-TRP construction

For AI/ML-assisted positioning, the output of the AI/ML model is an intermediate positioning measurement, such as new measurements and/or enhancements of existing measurements (e.g., LOS/NLOS indication, timing, and/or angle of measurement). In Case 3a, the model is located on the gNB side, and the gNB can generate the intermediate measurements based on legacy measurements.

Therefore, the case 3a during measurement report is further illustrated in Figure 2 below:

Figure 2. General procedures of NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- In Case 3a, the NG-RAN node generates the intermediate positioning measurement and sends it to the LMF via the AI/ML function.

2.1 Intermediate positioning measurement

Following is the agreement in RAN1 regarding the intermediate positioning measurement from NG-RAN node to LMF:

Agreement

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

LOS/NLOS indicator and timing information, generated by AI/ML model was agreed to report to LMF from NG-RAN node. Checking the current TS 38.455 [3], the existing LOS/NLOS indicator and timing information (e.g., UL RTOA or gNB Rx-Tx time difference) is already supported.

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

- The intermediate positioning measurements reported from the NG-RAN node to the LMF, such as LoS/NLoS Information, UL RTOA, and gNB Rx-Tx Time Difference, are already supported in the current specification. There is no need to introduce additional new IEs for them.

Then, one question may be raised whether to support AI/ML report includes an indicator that indicates the measurement information is derived using AI/ML model or not. This issue are under discussion in RAN1. From our perspective, the indicator is not needed, the reasons are as follows.

- The mentioned indicator will not affect LMF’s calculation. For example, in a general TDOA/RTT positioning procedure, LMF will collect the RSTD/RTT measurement information of different TRP pairs, and calculate UE’s location information based on TRPs’ location information and the reported RSTD/RTT. LMF doesn’t care the RSTD/RTT measurement which is derived using AI/ML model or legacy measurement.

- If the LOS/NLOS indicator is associated with the AI/ML model output, i.e., virtual LOS path, the reported LOS/NLOS indicator should always be equal to 1. It can be inferred that the timing information reported with full LOS indications is likely to be inferenced by AI/ML model.

- For reporting model output of AI/ML assisted positioning case 3a, an indicator identifying whether the measurement is based on AI/ML model is not needed.

2.2 Model performance evaluation

Another aspect is about the model performance evaluation for case 3a. The entity where the model inference is performed needs evaluate the model performance based on the inference results and ground-truth.

And following is the RAN1 agreement on this issue:

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

There are four different ways for model performance evaluation：

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

For model performance evaluation, RAN3 discussed this topic during the R17 and R18 stages and concluded that model performance is out of RAN3's scope. This wording to reflect this into stage2 specification is still under discussion regarding whether it needs further revision, but the principle is to avoid transmitting model performance metrics over NG-RAN interfaces (e.g., loss, accuracy, etc.).

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a.

RAN1 requests confirmation from RAN3 regarding the performance monitoring metric calculation.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

2.3 CU/DU split architecture

In RAN3 AI/ML for NG-RAN topic, following is captured in TS 38.401:

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Both AI/ML Model Training and AI/ML Model Inference are located in the gNB-CU. However, in legacy positioning, it is the DU that collects TRP result measurements and reports them to the CU upon request. The question raised is whether the AI/ML model could be deployed on the DU side. This question has not been raised during the RAN1 working group discussions.

We believe that both the DU and CU can perform inference. The CU can obtain legacy measurements from the DU and use AI/ML model to infer the AI/ML measurement. Alternatively, the DU can directly collect the data and perform AI inference itself.

- RAN3 is kindly asked to discuss which entity (CU or DU) is responsible for model inference to generate the intermediate positioning measurement.

3	Conclusion

Following is our observations and proposals:

- In Case 3a, the NG-RAN node generates the intermediate positioning measurement and sends it to the LMF via the AI/ML function.

- The intermediate positioning measurements reported from the NG-RAN node to the LMF, such as LoS/NLoS Information, UL RTOA, and gNB Rx-Tx Time Difference, are already supported in the current specification. There is no need to introduce additional new IEs for them.

- For reporting model output of AI/ML assisted positioning case 3a, an indicator identifying whether the measurement is based on AI/ML model is not needed.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- Option 1 (gNB performs model monitoring metric calculation and makes monitoring decision), namely option A is the appropriate way for model performance evaluation for case 3a.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

- RAN3 is kindly asked to discuss which entity (CU or DU) is responsible for model inference to generate the intermediate positioning measurement.

3	Reference

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] TR 38.843

[3] TS 38.455

4	Draft LS to RAN1

3GPP TSG-RAN WG3 Meeting #125bis	R3-24xxxx

Hefei, China, from 14th to 18th Oct 2025

Title:	[Draft] LS on AI/ML assisted Positioning about model performance monitoring

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

- Overall description

RAN3 discussed the following options for model performance monitoring in the case of case 3a,

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision

RAN3 prefers the option 1 that gNB performs model monitoring metric calculation and makes monitoring decision to avoid transmitting model performance metrics over NG-RAN interfaces.

- Actions

To SA5:.

ACTION: 	RAN3 kindly asks SA5 to take the above information into consideration.

3	Dates of next TSG SA WG 5 meetings

RAN3#126	18 November - 22 November 2024		Orlando, US

RAN3#127	17 February 	- 21 February   2025		Athens, GR