3GPP TSG-RAN WG3 Meeting #125bis	R3-245101

# Hefei, China, 14 – 18 Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1489 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Positioning SI for connected UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CATT, Ericsson, Huawei, Nokia, Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous RAN3#118 meeting, it was agreed in R3-226395 to update the procedural text for the SYSTEM INFORMATION DELIVERY COMMAND as follows: “Upon reception of the SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the requested Other SI, and if the UE corresponding to the confirmed UE ID IE is not in RRC connected state, delete the UE context corresponding to the confirmed UE ID IE, if any.” The intention is to clarify that the UE context corresponding to the Confirmed UE ID IE, can be deleted by the DU, only for idle or inactive UEs (i.e. the deletion of the UE context is not applicable for connected UE). However, the same issue still exists in the POSITIONING SYSTEM INFORMATION DELIVERY message. It is proposed to update the procedural text for the POSITIONING SYSTEM INFORMATION DELIVERY message to indicate that when the UE is not in RRC connected state, the UE context can be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in the POSITIONING SYSTEM INFORMATION DELIVERY message that the UE context corresponding to the Confirmed UE ID IE, can be deleted by the DU, only for idle or inactive UEs (i.e. the deletion of the UE context is not applicable for connected UE). Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated functional impact with the previous version of the specification (same release) because the change only affects the positioning SI related function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE contexts for RRC connected UE are erroneously deleted POSITIONING SYSTEM INFORMATION DELIVERY procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.13.20	Positioning System Information Delivery

8.13.20.1	General

The purpose of the Positioning System Information Delivery procedure is to command the gNB-DU to broadcast the requested one or several Positioning SI messages indicated by the gNB-CU. The procedure uses non-UE associated signalling.

8.13.20.2	Successful Operation

Figure 8.13.20.2-1: Positioning System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Upon reception of the POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the requested one or several Positioning SI messages, indicated by the PosSITypeList IE, and if the UE corresponding to the Confirmed UE ID IE is not in the RRC connected state, delete the UE context, if any.

Interactions with gNB-DU Configuration Update procedure:

Upon reception of POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU Configuration Update procedure may be performed, and as part of such procedure the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in GNB-DU CONFIGURATION UPDATE message for UEs that are unable to receive system information from broadcast.

8.13.20.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>