3GPP TSG-RAN WG3 Meeting #125 Bis	             R3-245107

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	17.3

Source:	CMCC

Title:	Support On-Demand SIB1 for UEs

Document for:	Discussion

Introduction

In the RAN3 last # 125 meeting, it is agreed [1]:

[Agreement] Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Discussions

UL WUS Configuration IE

Figure 1 depicts the Case 2: the NES Cell sends UL WUS Configuration to the Cell A before switching to the on-demand SIB1 operation mode (step 0). The Cell A periodically broadcasts the UL WUS Configuration of the NES Cell to UEs in step 1. The UE triggers the on-demand SIB1 on the NES Cell by sending the UL WUS signal directly to the NES Cell in step 2. The NES Cell sends the on-demand SIB1 information to the UE in step 3. This approach minimizes the delay for the UE to connect to the NES Cell but may result in less energy savings for the NES Cell.

Figure 1 On-Demand SIB1 of NES Cell, Case 2

It is required that the UL WUS Configuration is sent from a NES Cell to a Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS Configuration is changed.

If the CU/DU split architecture is adopted in gNB, a NES Cell and Cell A could be in different DU(s). In order to transit the UL WUS Configuration, both F1 interface and Xn interface could be impacted.

- The NES Cell and Cell A are on the same gNB, but on different DU(s). gNB-CU is in the middle to forward the UL WUS configuration between two gNB-DUs on the same gNB.

- The NES Cell and Cell A are on different gNB(s). Two gNB-CU(s) are in the middle to forward the UL WUS Configuration between two gNB-DUs on different gNB(s).

For both Xn and F1 interfaces, a new IE needs to be defined to carry UL WUS configuration between NES cell and Cell A.

UL WUS Configuration Broadcast Activation and Deactivation

If a NES cell returns to its active mode and broadcasts its SIB1 periodically, Cell A no longer needs to broadcast the UL WUS Configuration for the NES Cell. Thus, the UL WUS Configuration broadcast by the Cell A could be activated and deactivated based on NES Cell’s mode change and the Cell A needs to be aware of the mode change of the NES Cell.

The NES cell could initiate the change between active mode and OD SIB1 operation mode, which tiggers the UL WUS configuration broadcast activation and deactivation on the requested Cell A.

In the CU/DU split architecture, the NES gNB-DU could initiate the change between active mode and on-demand SIB1 operation mode.

The UL WUS configuration broadcast can be activated and deactivated on the Cell A based on the NES Cell on-demand SIB1 status change.

The Cell A starts to broadcast the UL WUS Configuration for the NES Cell after receiving the UL WUS Configuration and UL WUS Configuration Broadcast Activation indicator.

The Cell A stops to broadcast the UL WUS Configuration for the NES Cell after receiving the UL WUS Configuration Broadcast Deactivation indicator.

Cell A may request the NES cell to deactivate the on-demand SIB1 operation mode if it can no longer or chooses not to broadcast the UL WUS configuration for the NES cell. This may occur under the conditions such as exceeding expected power consumption or being scheduled to enter energy-saving mode.

The Cell A could request the NES Cell to deactivate the on-demand SIB1 operation mode.

Figure 2 shows the procedure in the CU/DU split architecture in details, in which the NES Cell and Cell A are on different gNB(s). The NES Cell gNB is the gNB hosting the NES Cell and the Cell A gNB is the gNB hosting the Cell A. The NES Cell gNB DU requests the Cell A gNB CU to broadcast the UL WUS configuration for the NES Cell. Figure 3 shows the NES Cell exits the On-Demand SIB1 operation mode, the corresponding NES gNB-DU indicates this to the corresponding Cell A gNB-DU.

Figure 2 On-Demand SIB1 Operation Activation Procedure, Case 2

Figure 3 On-Demand SIB1 Operation Deactivation Procedure, Case 2

The gNB-DU Configuration Update procedure could be leveraged to send/update the UL WUS configuration from the NES Cell DU to the NES Cell CU.

The NG-RAN Node Configuration Update procedure could be leveraged to send/update UL WUS configurations from the NES Cell  gNB to Cell A gNB over Xn, or from the NES Cell gNB-CU to the Cell A gNB-CU over Xn.

The gNB-CU Configuration Update procedure could be leveraged to send/update the UL WUS configuration from the Cell A CU to the Cell A DU.

Cell A Capability Notification

To help the NES Cell gNB to make a decision on which neighbouring cells are suitable to be Cell A and broadcast UL WUS configuration for the NES Cell, the Cell A capability of the neighbouring cells could be notified to the NES Cell gNB.

Cell A capability of neighbouring cells could be indicated from one gNB to another gNB, or from one gNB-CU to another gNB-CU, or/and from one gNB-DU to its CU, which could have Xn and F1 interface impact.

Decision of NES Cell

This subsection mainly focuses on the split architecture.  The CU manages signalling between the network and the UE, including connection setup, release, mobility management, and security functions. And it has more global picture of the neighbouring cell deployment, inference level, signal strength and quality, energy consumption, power status, etc.  The CU can be helpful by providing information before the DU initiates the on-demand SIB1 operation for some NES cells. For instance, the CU could supply a list of cells where the on-demand SIB1 operation should be enabled.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled and a list of corresponding cells could be requested as Cell A) before a DU initiates the OD SIB1 operation.

Conclusions

In this submission, we discussed the procedures of the OD-SIB1 Case 2 that is agreed to be focused on in RAN 3. The split CU-DU architecture is also being considered in the paper. The following proposals are presented in the paper:

- For both Xn and F1 interfaces, a new IE needs to be defined to carry UL WUS configuration between NES cell and Cell A.

- The NES cell could initiate the change between active mode and OD SIB1 operation mode, which tiggers the UL WUS configuration broadcast activation and deactivation on the requested Cell A.

- In the CU/DU split architecture, the NES gNB-DU could initiate the change between active mode and on-demand SIB1 operation mode.

- The UL WUS configuration broadcast can be activated and deactivated on the Cell A based on the NES Cell on-demand SIB1 status change.

- The Cell A starts to broadcast the UL WUS Configuration for the NES Cell after receiving the UL WUS Configuration and UL WUS Configuration Broadcast Activation indicator.

- The Cell A stops to broadcast the UL WUS Configuration for the NES Cell after receiving the UL WUS Configuration Broadcast Deactivation indicator.

- The Cell A could request the NES Cell to deactivate the on-demand SIB1 operation mode.

- The gNB-DU Configuration Update procedure could be leveraged to send/update the UL WUS configuration from the NES Cell DU to the NES Cell CU.

- The NG-RAN Node Configuration Update procedure could be leveraged to send/update UL WUS configurations from the NES Cell  gNB to Cell A gNB over Xn, or from the NES Cell gNB-CU to the Cell A gNB-CU over Xn.

- The gNB-CU Configuration Update procedure could be leveraged to send/update the UL WUS configuration from the Cell A CU to the Cell A DU.

- Cell A capability of neighbouring cells could be indicated from one gNB to another gNB, or from one gNB-CU to another gNB-CU, or/and from one gNB-DU to its CU, which could have Xn and F1 interface impact.

- The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled and a list of corresponding cells could be requested as Cell A) before a DU initiates the OD SIB1 operation.

Reference

- draft_RAN3#125_MeetingReport_ver20240903