3GPP TSG-RAN WG3 Meeting #125 bis	             R3-245109

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	20.2

Source:	CMCC

Title:	Support Direct AI/ML Positioning

Document for:	Discussion

Introduction

In the RAN #105 meeting, the following WID relevant to RAN3 work item of AI/ML for positioning has been reached [1]:

[WID] - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Discussions on Direct AI/ML Positioning

Traditional 5G NR positioning approaches are largely based on geometric techniques, involving two main steps: 1) taking measurements of radio signals, and 2) estimating the device’s location by solving a set of non-linear equations that connect the position of the user equipment (UE) to the measurements. The precision of these geometry-based techniques heavily relies on the availability of line-of-sight (LOS) signal paths. In cases where LOS is weak or in complex environments with significant multipath interference, such as inside factories, the accuracy of these methods tends to decline. AI/ML-driven algorithms can help enhance positioning accuracy, particularly in scenarios where non-line-of-sight (NLOS) conditions are prevalent.

Direct AI/ML positioning leverages an AI/ML model to estimate the UE’s location directly. This approach can include methods like fingerprinting, where channel characteristics—such as the channel impulse response (CIR) or power delay profile (PDP)—are used as inputs to the AI/ML model to determine the UE's position.

The direct AI/ML positioning has the following cases that have been discussed and prioritized in other RAN WGs.

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

- (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Case 1 (1st Priority)

The Case 1 is illustrated in Figure 1.

Figure 1: Case 1: UE based direct AI/ML positioning

Model Training

Both the PRU (Positioning Reference Unit) and UE perform channel measurements based on PRS (Positioning Reference Signals). The existing PRS configuration can be utilized to gather training measurements for Case 1. If legacy methods can provide very accurate positionings to UEs, then it is doubtful that why AI/ML model is needed. On the other hand, inaccurate positioning of UEs from legacy methods should not be used for label generation. Therefore, it is our view that the PRU is prioritized for label generation. In the following, we still use UE as the term for the training purpose, however, we prioritize the PRUs for the label generation.

The samples from the PRUs should be prioritized for label generation for model training and updating, which could be applied to all cases.

UE-side direct positioning AI/ML model training can be carried out at UE itself or on an OTT server, which is up to UE-side implementation. Figure 2 shows the procedure of model training and updating in Case 1.

Figure 2: Case 1 model training, transfer of measurements and labels from PRU/UE to UE-side training entity

Model Inference

For Case 1, as captured in TR 38.843, the UE performs measurements based on PRS and uses them as inference input to the model that is trained locally or on the UE’s OTT server. The UE’s location is the output of the interference.

Model Monitoring

As discussed in the previous sub-sessions, both model training and inference are run on the UE itself or on the UE’s OTT server, the entity that generates the monitoring metric and makes the monitoring decision can be the same.

For Case 1, the model monitoring decision entity should not be on the network side.

For Case 1, no RAN 3 impact is foreseen for model training, model inference and model monitoring. UE’s location report uses the legacy procedure.

Case 3b (1st Priority)

The Case 3b is illustrated in Figure 3.

Figure 3: Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Model Training

For Case 3b, the following entities have been identified to generate the label and measurements:

- For label generation: LMF, which has the knowledge of PRU location.

- For measurement data generation: TRP (Transmission Reception Point), which is the NG-RAN node, and measures SRS. The legacy configuration of SRS (Sounding Reference Signal) can be employed for obtaining the measurements for training in Case 3b.

The AI/ML model is located at the LMF. The SRS measurements need to be transferred from the NG-RAN to the LMF as shown in Figure 4.

Figure 4: Case 3b model training, transfer of measurements and labels from NG-RAN Node to LMF

For Case 3b, the measurements need to be sent from the NG-RAN node to the LMF, which are used by the LMF for model training.

A quality indicator associated with the measurements has been identified for data collection [2], which makes sure that the measurements used for training maintain the same quality as those used for inference. The NG-RAN Node could report the quality indicator along with the measurements to the LMF. Alternatively, the quality indicator, as a requirement, could be signalled to the NG-RAN Node from the LMF and applied when the measurements are generated. This would allow for the exclusion of low-quality measurements, ensuring that only those meeting the quality standard are reported to the LMF, which can avoid transferring those useless measurements from the NG-RAN node to the LMF.

For Case 3b, a quality indicator associated with the measurements could be reported from the NG-RAN Node to the LMF along with the measurements.

For Case 3b, a quality requirement could be signalled from the LMF to the NG-RAN Node, such that only those measurements that satisfy the quality requirement will be transferred from the NG-RAN Node to the LMF.

Model Inference

In Case 3b, the LMF estimates the position based on SRS measurements generated and reported by the NG-RAN Node. The legacy SRS configuration can be used to obtain the measurements needed for model inference input. In this case, reporting the measurements from the NG-RAN Node to the LMF is necessary.

For Case 3b, the measurements need to be sent from the NG-RAN node to the LMF, which are used by the LMF to estimate the position of the UE as model inference.

Model Monitoring

In Case 3b, the LMF should be the entity responsible for deriving the monitoring metric, as outlined in [2]. The LMF gather measurements and estimated positions as labels, it is logical for the LMF to make the monitoring decisions. However, it is likely that NG-RAN node could provide further measurements to the LMF for the model monitoring purpose.

For Case 3b, NG-RAN node might provide further measurement to the LMF for the model monitoring purpose. RAN 3 should wait for the RAN 1 progress.

Case 2b (2nd Priority)

If Case 2b is supported, the training data collection should follow a similar approach to Case 3b, with the same type of measurements for inference being sent from the PRUs/UEs to the LMF as shown in Figure 5. However, given the increased air-interface load, we believe that Case 2b should de-prioritized in RAN 3.

Figure 5: Case 2b model training, transfer of measurements and labels UE to LMF

RAN 3 should deprioritize the study on the Case 2b. If the Case 2b is considered, no RAN 3 impact is foreseen. RAN 3 can closely monitor RAN1’s progress after RAN 1 resumes the discussion on Case 2b.

Conclusions

In this submission, we discussed the direct AI/ML positioning cases and potential RAN 3 impact. We presented the following proposals:

- The samples from the PRUs should be prioritized for label generation for model training and updating, which could be applied to all cases.

- For Case 1, the model monitoring decision entity should not be on the network side.

- For Case 1, no RAN 3 impact is foreseen for model training, model inference and model monitoring. UE’s location report uses the legacy procedure.

- For Case 3b, the measurements need to be sent from the NG-RAN node to the LMF, which are used by the LMF for model training.

- For Case 3b, a quality indicator associated with the measurements could be reported from the NG-RAN Node to the LMF along with the measurements.

- For Case 3b, a quality requirement could be signalled from the LMF to the NG-RAN Node, such that only those measurements that satisfy the quality requirement will be transferred from the NG-RAN Node to the LMF.

- For Case 3b, the measurements need to be sent from the NG-RAN node to the LMF, which are used by the LMF to estimate the position of the UE as model inference.

- For Case 3b, NG-RAN node might provide further measurement to the LMF for the model monitoring purpose. RAN 3 should wait for the RAN 1 progress.

- RAN 3 should deprioritize the study on the Case 2b. If the Case 2b is considered, no RAN 3 impact is foreseen. RAN 3 can closely monitor RAN1’s progress after RAN 1 resumes the discussion on Case 2b.

References

RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.