3GPP TSG-RAN WG3 Meeting #125 bis	             R3-245111

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on multi-modality awareness at RAN

Document for:	Discussion

Introduction

SA2 replies the RAN2 LS on multi-modality awareness at RAN as following [1]:

SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively.

Discussions

RAN awareness of Multi-modal Service ID (MMSID) is beneficial, enabling RAN to coordinate scheduling and synchronize multiple QoS flows associated with the same MMSID. Via control plane, MMSID could be forwarded from PCF to RAN through AMF.

Via control plane, MMSID could be made aware to RAN by being forwarded from PCF to RAN through AMF.

In split-bearer dual connectivity scenarios, where different QoS flows belonging to the same MMSID could be transmitted across both bearers, careful coordination is essential to ensure proper flow synchronization. The behaviour or transmission conditions of a QoS flow in one bearer can impact how related QoS flows should be managed in the other bearer. This coordination is particularly crucial for ensuring that the synchronization between flows is maintained, as any mismatch could degrade the user experience for time-sensitive services like XR or real-time communications. Additionally, this affects congestion management, as handling congestion in one bearer could influence how traffic is prioritized or adjusted in the other bearer to avoid delays or disruptions in the overall service. In order to achieve optimal performance for multi-modal services, many problems in split-bearer dual connectivity scenarios need to be revisited in RAN3.

Multi-modal services pose new challenges in split-bearer dual connectivity scenarios, particularly in maintaining synchronization and coordination of multiple QoS flows under the same MMSID. RAN3 needs to revisit the problems in split-bearer dual connectivity scenarios.

Conclusions

In this submission, we discussed the multi-modality awareness at RAN. The following proposals are made:

- Via control plane, MMSID could be made aware to RAN by being forwarded from PCF to RAN through AMF.

- Multi-modal services pose new challenges in split-bearer dual connectivity scenarios, particularly in maintaining synchronization and coordination of multiple QoS flows under the same MMSID. RAN3 needs to revisit the problems in split-bearer dual connectivity scenarios.

References

R3-245017, LS reply on multi-modality awareness at RAN