3GPP TSG-RAN WG3 Meeting #125bis	R3-245123

# Hefei, China, October 14th – 18th, 2024

Agenda item:	9.2

Source: 	Samsung, ZTE, CATT, China Unicom, Lenovo

Title: 	Discussion on QMC support in NR-DC for RRC segmentation

Document for:	Discussion and decision

Introduction

Last RAN3 meeting further discussed the necessity to introduce the RRC segmentation over XnAP. After the discussion, all participating companies agreed to have a solution on RRC segmentation coordination. Regarding which option to follow, however, no consensus has been achieved and consequently the following open issue is captured,

Continue the discussion based on Opt1 and Opt2 for RRC segmentation.

In this contribution, we further discuss this open issue.

Discussion

Last meeting continued to discuss the necessity to introduce the RRC segmentation coordination over XnAP. After further discussion, companies have achieved the consensus that the RRC segmentation coordination over XnAP is needed, but how to coordinate is still FFS, i.e. which option to follow needs further discussion.

Before we dig into more details on candidate options, we need to bear in mind that as indicated by RRC spec, whether the RRC segmentation is enabled over SRB4/5 is configured on the UE basis, i.e. if the rrc-SegAllowedSRBx is enabled, all QoE measurement reported over SRBx will enable RRC segmentation. While whether to enable the RRC segmentation needs to be coordinated on a per QoE configuration basis (i.e. such coordination will be triggered by the QoE configuring node of this QoE configuration, and remember that the QoE configuring node is a concept subject to a specific QoE configuration.)

Observation 1: If the rrc-SegAllowedSRB4/5 is enabled, all QoE measurement (subject to all QoE configurations configured to the UE) reported over SRB4/5 will enable RRC segmentation.

Observation 2: The concept of QoE configuring node and non QoE configuring node is subject to a specific QoE configuration. Namely, for different QoE configurations configured to a UE, the QoE configuring node may be different.

Observation 3: Whether to enable the RRC segmentation needs to be coordinated on a per QoE configuration basis.

After the above clarification, we go back to the candidate options on the table, there are two coordination options on the table,

Option 1: The QoE configuring node sends the request explicitly on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [1]).

Option 2: The QoE configuring node sends the QMC coordination request which implicitly requests on whether the non QoE configuring node enables the RRC segmentation, and the non QoE configuring node sends the response on whether the non QoE configuring node enables the RRC segmentation (as provided in [2]).

It can be observed that the main difference between Option 1&2 is that whether an explicit request on whether the non QoE configuring node enables the RRC segmentation is needed.

For Option 2, it is always up to the non QoE configuring node to decide on whether to enable the RRC segmentation for its own SRB.

While for Option 1, it additionally requires the QoE configuring node to firstly decide whether to initiate the RRC segmentation coordination, and if the QoE configuring node decides not to initiate the RRC segmentation coordination (i.e. the RRC segmentation request is not contained in the QMC coordination request), then the non QoE configuring node will not enable the RRC segmentation for its own SRB by default.

According to the discussion during last several meetings, the proponent of Option 1 considers the QoE configuring node can be act as the ‘dealbreaker’, i.e.,

- Case 1: If the QoE configuring node found that the QoE reports were not segmented based on previous knowledge (e.g. initially only a single connectivity is established between the gNB and the UE, and there was an S-based QoE configuration for which the QoE reports were not segmented), then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node NOT to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node may not enable the RRC segmentation accordingly.

- Case 2: If the QoE configuring node found that the QoE reports were segmented based on previous knowledge, then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node should try its best to enable the RRC segmentation.

- As a comparison, Option 2 implies that the non QoE configuring node is better to enable the RRC segmentation if it is able to do so.

Regarding the above two cases, in our understanding,

- For both Case 1&2, the pre-condition is that the QoE configuring node is able to obtain the previous knowledge on whether the QoE reports were segmented subject to a QoE configuration. However, it is also possible that there’s actually no previous knowledge: for example, if a new m-based QoE configuration is received at the SN, and as the potential QoE configuring node, SN performs QMC coordination request to the MN. At this point of time, the SN still has no previous knowledge since no QoE reports have been reported subject to this QoE configuration. As a result, if there’s no previous knowledge, what RRC segmentation information is requested (enable or not enable) from the QoE configuring node still remains unclear.

- Assuming the QoE configuring node is able to obtain the previous knowledge, for Case 1, it is true that the non QoE configuring node may make better RRC segmentation decision by getting suggestions from the QoE configuring node. However, the benefit looks limited. From one aspect, if the non QoE configuring node for QoE configuration 1 also needs to receive the QoE reports for QoE configuration 2 which needs RRC segmentation, then the non QoE configuring node still has to enable the RRC segmentation regardless of the suggestion from the QoE configuring node. From the other aspect, there’s no harm for the non QoE configuring node to still enable the RRC segmentation for such case, since no matter whether the QoE reports will be segmented, enabling the RRC segmentation is always preferred to avoid over-sized QoE reports to be discarded at the UE side.

- Assuming the QoE configuring node is able to obtain the previous knowledge, for Case 2, if we can acknowledge that enabling the RRC segmentation is always preferred to avoid over-sized QoE reports to be discarded at the UE side, then both Option 1 and Option 2 could work, with no one prevails the other.

As a summary, based on the above analysis, Option 2 can accommodate all possible cases no matter whether the previous knowledge can be obtained or not. And the logic behind Option 2 is simpler by acknowledging that the non QoE configuring node should try its best to enable the RRC segmentation, no matter whether the QoE reports are in fact segmented or not.

Observation 4: The pros identified to use Option 1 depends on the previous knowledge the QoE configuring node had, but there are also cases that there’s actually no previous knowledge.

Observation 5: Even for cases when the QoE configuring node can obtain the previous knowledge, Option 1 only brings limited benefit for some specific cases.

Observation 6: Option 2 can accommodate all possible cases no matter whether the previous knowledge exists or not. And the logic behind Option 2 is simpler by acknowledging that the non QoE configuring node should try its best to enable the RRC segmentation when it is able to do so.

Based on the above analyses and observations, we propose,

Proposal 1: Adopt Option 2 as the solution for RRC segmentation.

Proposal 2: Agree the CR in R3-245124.

Conclusion

In this paper, we provides further considerations for QoE on support of NR-DC. The following observations are provided,

Observation 1: If the rrc-SegAllowedSRB4/5 is enabled, all QoE measurement (subject to all QoE configurations configured to the UE) reported over SRB4/5 will enable RRC segmentation.

Observation 2: The concept of QoE configuring node and non QoE configuring node is subject to a specific QoE configuration. Namely, for different QoE configurations configured to a UE, the QoE configuring node may be different.

Observation 3: Whether to enable the RRC segmentation needs to be coordinated on a per QoE configuration basis.

Observation 4: The pros identified to use Option 1 depends on the previous knowledge the QoE configuring node had, but there are also cases that there’s actually no previous knowledge.

Observation 5: Even for cases when the QoE configuring node can obtain the previous knowledge, Option 1 only brings limited benefit for some specific cases.

Observation 6: Option 2 can accommodate all possible cases no matter whether the previous knowledge exists or not. And the logic behind Option 2 is simpler by acknowledging that the non QoE configuring node should try its best to enable the RRC segmentation when it is able to do so.

Based on the above analyses and observations, we propose,

Proposal 1: Adopt Option 2 as the solution for RRC segmentation.

Proposal 2: Agree the CR in R3-245124.

References

[1] R3-244550

[2] R3-244551