3GPP TSG-RAN WG3 Meeting #125bis	R3-245125

# Hefei, China, October 14th – 18th, 2024

Agenda item:	14.2

Source: 	Samsung

Title: 	Further discussion on support MBS broadcast service for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of MBS broadcast service for NR NTN, and the agreement and open issues are captured as follows,

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

Check RAN2 progress on the geographical area information.

In this contribution, we further discuss these open issues.

Discussion

According to the R19 NR NTN WID, one of the objectives is to support MBS broadcast service. And the following details are provided in the WID,

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

Regarding the first bullet, the motivation to work on enhancement is that for some cases the intended service area is expected to be smaller than the coverage of a Uu cell. So the SIB signalling needs to provide the intended service area more granular than Uu cell provided by NTN. According to the WID, it is up to RAN2 to determine the detailed content related to the intended service area in the SIB to be used. If a new SIB is determined to be introduced, RAN3 can focus on potential enhancement on new SIB coordination in case of split architecture.

More specifically, the open issues could be, e.g.

- Which node provides the intended service area related information contained in the new SIB?

- Which node encodes the new SIB, CU or DU?

- Whether the existing mechanism on SIB coordination over F1AP is enough for the new SIB?

According to RAN2 progress, the new SIB has not been introduced, and it is also possible to reuse the existing SIBs or RRC messages. Therefore, such open issues could be further checked once RAN2 has progress on the detailed contents of the SIB.

Proposal 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Regarding the second bullet, last RAN3 meeting we have achieved the following agreement,

The mapped cell ID can be used to define MBS broadcast service in NTN.

In addition, a TP to BLCR for TS 38.300 has been agreed in R3-244813, wherein the following text is newly captured,

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

The Cell Identity used for MBS broadcast service may be a Mapped Cell ID.

Editor’s Note: the wording for above texts of using Mapped Cell ID for MBS may be further clarified.

It should be noted that the newly added sentence is not listed as a new bullet of the previous text. In our understanding, the intention here is to imply that the cell identity used for MBS broadcast service may also be a Uu Cell ID, especially when the cell is an earth fixed cell or a quasi earth fixed cell. In addition, if both RAN and CN know in advance that the cell identity used for MBS broadcast service is Uu cell ID/Mapped cell ID, there will be no inter-operability issue. So the question is, why do we need to provide flexibility on the cell ID setting for MBS broadcast service.

Normally, it is a consensus that the coverage area for a mapped cell is smaller than a uu cell, which means that a mapped cell provides a more granular coverage area information than a uu cell. We are still uncertain on whether a MBS broadcast service could be provided as large as the coverage area of a earth fixed cell or a quasi earth fixed cell. So the necessity to provide such flexibility on cell ID setting needs further check with operators with plausible use cases.

Proposal 2: Whether the cell identity used for MBS broadcast service may also be a Uu Cell ID needs further check with convincing use cases.

For the open issue on the table,

Check RAN2 progress on the geographical area information.

After checking RAN2 chair notes achieved last meeting, the following agreements are captured,

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

It can be observed from RAN2 has agreed to define a geographical area in terms of a set of referenceLocation and radius or a set of polygons. However, RAN2 has not made the final decision on which option to go or agree both. So RAN3 still cannot decide whether to also introduce the similar information over NG interface.

Proposal 3: Wait for RAN2 progress on the intended service area before RAN3 makes the decision on what extra information is needed over NG interface.

Conclusion

In this paper, we provide discussion on support MBS broadcast service for NR NTN. The following proposals are provided,

Proposal 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Proposal 2: Whether the cell identity used for MBS broadcast service may also be a Uu Cell ID needs further check with convincing use cases.

Proposal 3: Wait for RAN2 progress on the intended service area before RAN3 makes the decision on what extra information is needed over NG interface.

References