3GPP TSG-RAN WG3 Meeting #125bis	R3-245126

# Hefei, China, October 14th – 18th, 2024

Agenda item:	14.3

Source: 	Samsung

Title: 	Further discussion on support of regenerative payload for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of regenerative payload for NR NTN, and the following consensus is achieved,

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

Inactive UE support:

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

Feeder link switch issue:

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

Continue the discussion on above open issues…

In this contribution, we further discuss the support of regenerative payload.

Discussion

Last RAN3 meeting continued to discuss the NG interface management. And the following agreement is achieved,

NG Interface management Issue:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

And whether to introduce NG suspend/resume remains to be an open issue.

According to the discussion during last several RAN3 meetings, the proponents of NG suspend/resume mentioned that the introduction of NG suspend/resume procedures will save transmission energy when recovering the connection between gNB and AMF since less information is expected to be transmitted compared to NG setup procedure. However, the introduction of NG suspend/resume will require much more storage on NG interface management related information for gNB on board, especially after several orbits, the gNB on board may need to store the NG interface related information with every AMF along the orbits. In addition, there’s also a concern that whether the original suspended information is still valid when the gNB on board connects back to a specific AMF.

In our understanding, unless the operator claims that saving transmission energy is more important than the storage issue, we still see little need to introduce NG suspend/resume as new NG interface management.

Proposal 1: No need to introduce NG suspend/resume procedures.

From the other aspect, for the newly introduced NG Removal procedure, the details have not been discussed. In our understanding, the first question needed to be asked is which node initiates the NG Removal procedure, RAN, CN or both. In our opinion, if RAN initiates the NG Setup, by following the similar logic, RAN also initiated the NG Removal.

Proposal 2: RAN initiates Class 1 NG Removal procedure.

According to the R19 NR NTN WID, one of the main objectives is to support regenerative payload. And R19 WID indicates to support the gNB on board, rather than gNB-DU on board case.

In general, as provided by TR 38.821, there are basically two architecture options that are related to gNB on board, i.e.,

3)	Regenerative satellite and on-board gNB(s) (Sec. 5.2.1);

4)	Regenerative satellite with Inter-Satellite Links (ISLs), gNB processed payload (Sec. 5.2.1);

In addition, TR 38.821 has provided a brief summary on mobility support for different NTN architectures, and the summary table has been given as follows,

Table 8.3-2: Mobility support for the various NTN architectures

|  | Arch. 3 | Arch. 4 |
| --- | --- | --- |
| Intra-gNB mobility ("monolithic" gNB) | Supported, no standards impact | Supported, no standards impact |
| Xn mobility | Depends on Xn over SRI, no standards impact | Supported if Xn exists |
| Mobility through the 5GC | Supported, no standards impact | Supported, no standards impact |

As provided by the summary table, for intra-gNB mobility and mobility through 5GC, there’s no standard impact identified. While for Xn mobility, the main gap is whether an Xn interface can be setup over SRI or ISL; in other words, if it can be agreed that an Xn interface can be setup over SRI or ISL, the main impact will be on the transport network layer which is out of RAN3 scope, and the current XnAP spec looks enough to support the Xn mobility case for NTN regenerative payload.

Observation 1: As summarized by TR 38.821, no standard impact has been identified for intra-gNB mobility and mobility through 5GC.

Observation 2: If it is agreed to support Xn interface over SRI or ISL, no XnAP spec impact is identified to support Xn mobility for gNB on board architecture.

In order to investigate whether Xn mobility can be supported for gNB on board, the first thing to identify is whether Xn interface can be setup over ISL or SRI.

Firstly, since ISL is a transport link between satellites, it is possible that the satellite payload (with gNB on board) provides ISL between satellites. If it is possible for the satellite payload to provide ISL between satellites, it is feasible for the gNB on board to exchange the XnAP messages through ISL.

In addition, for the user plane, it can be investigated that whether the ISL could support user plane related operations such as data forwarding. As indicated by TR 38.821, both ISL and SRI will feature an availability probability of 99.999%, so generally we assume that ISL will provide comparable transport performance to SRI. Since the data forwarding is also inevitable over SRI for mobility through 5GC, we assume the ISL could also accomplish similar user plane operations including data forwarding.

While if there’s no ISL can be established, the next question to investigate is whether to support Xn interface over SRI.

In general, there are two mobility scenarios that may take Xn interface over SRI into consideration,

- Case 1: NTN-NTN mobility between two gNB on board when there’s no ISL between satellites.

- Case 2: NTN-NTN mobility between one gNB on board and one gNB on ground with transparent payload.

- Case 3: NTN-TN mobility between one gNB on board and one gNB for TN.

For Architecture 3 as follows,

It can be observed that the NG interface has already been established over SRI. If we would like to support Xn mobility for such architecture, firstly it means that we need to additionally establish Xn interface over SRI.

For Case 1 and Case 2, if we support Xn based handover, the only benefit compared to NG based handover is to save the transport overhead between the NTN GW and 5G CN. On the other hand, if the target gNB (either on board the satellite or on ground with transparent payload) is connected to another NTN GW, the extra transport overhead is needed between NTN GWs.

For Case 3, our understanding is that before we discuss whether to support Xn based handover for Case 3, we firstly need to identify whether to support Case 3 itself.

As a result, for Case 1&2, it is required to identify that whether the transport network between the NTN GW and 5G CN is the bottleneck for Xn signalling and data transport, and whether the support of Xn mobility could solve this issue. While for Case 3, it is required to identify whether to support such case. So far, the benefit of introducing Xn over SRI is still unclear to us.

Proposal 3: Support Xn over ISL in case of regenerative payload with ISL.

Proposal 4: RAN3 is kindly asked to identify whether to support/enhance NTN-TN mobility case between one gNB on board and one gNB for TN.

Mobility due to the movement of a UE

If the UE moves out of the coverage of the current cell provided by gNB on board, then intra-gNB mobility, inter-gNB intra-AMF mobility and inter-AMF mobility can occur.

Our understanding is that for intra-gNB mobility and inter-gNB intra-AMF mobility, little standard impact is identified.

While for the inter-AMF mobility, it seems still possible that the satellite that provides the service to the UE is not changed.

Observation 3: It is possible that the satellite providing the service link to the UE is unchanged in case of inter-AMF mobility.

If the satellite providing the service link is unchanged in case of inter-AMF mobility, there are basically two options depending on whether a logical gNB is able to connect to multiple AMFs.

Option 1: The logical gNB on board the satellite providing the service link to UE is changed, i.e. the same physical gNB implements two logical gNBs (i.e. gNB1 and gNB2), and gNB1 connects to AMF1 while gNB2 connects to AMF2.

In this option, the gNB1 and gNB2 are collocated, and the UE needs to be handed over from gNB1 to gNB2. In case of NG based handover, the basic call flow is as follows,

However, if gNB1 and gNB2 are collocated, whether we still should use NG based handover is questionable; instead, we can use Xn based handover as follows,

Note that the Xn Setup procedure and Xn Handover Preparation procedures may be implemented internally. Such solution also minimizes the NG signalling over SRI, and reuses the existing mechanism as much as possible.

And another potential solution is,

Option 2: AMF re-allocation.

The solution is also mentioned by TR 38.821; however, TR 38.821 does not describe too much on this solution. In our understanding, the main difference between Option 2 and Option 1 is that Option 1 needs to additionally establish a new collocated gNB while Option 2 keeps the logical gNB unchanged, i.e. gNB1 disconnects from AMF1 and connects to AMF2 for a specific UE. And such case cannot be supported by the current NG based handover procedure because current NG based handover gets two gNBs involved.

Recall that in existing SA2 spec, AMF re-allocation is only used in the case when the Initial UE message needs to be rerouted from one AMF to another AMF. And this option will extend the applied scenario for the AMF re-allocation, i.e. the enhancement may be needed to transfer UE context from AMF1 to AMF2. Moreover, in order to avoid NG signalling over SRI as much as possible, the direct communication between AMF1 and AMF2 should be used for AMF re-allocation in such scenario, instead of transferring UE context from AMF1 to AMF2 via gNB1.

For this option, if we intend to reuse the existing NGAP procedure as much as possible, the tentative overall call flow is provided as follows,

It can be observed from the call flow that a total of four messages will be transferred over NG over SRI, in which two Class-1 procedures are used; however, considering the fact that the gNB1 is kept unchanged, the current NG signalling can be reduced to achieve the similar operation. For example, the gNB1 can internally release the UE context with AMF1 instead of using explicit release procedure, since anyway gNB1 will lose connection to AMF1 after moving out of the coverage of AMF1.

Proposal 5: RAN3 is kindly asked to discuss the mobility due to UE movement in case of regenerative payload, and identify the solution(s) for such open issue.

Proposal 6: For mobility due to UE movement, if we adopt the gNB collocation solution, a satellite should establish an additional logical gNB as the target gNB which is collocated with the source gNB. Consider the Xn based-like handover for this solution.

Proposal 7: For mobility due to UE movement, if we adopt the AMF re-allocation solution, RAN3 is kindly asked to consider the enhancement to reduce the NG signalling over SRI.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for NR NTN. The following proposals are provided,

Proposal 1: No need to introduce NG suspend/resume procedures.

Proposal 2: RAN initiates Class 1 NG Removal procedure.

Observation 1: As summarized by TR 38.821, no standard impact has been identified for intra-gNB mobility and mobility through 5GC.

Observation 2: If it is agreed to support Xn interface over SRI or ISL, no XnAP spec impact is identified to support Xn mobility for gNB on board architecture.

Proposal 3: Support Xn over ISL in case of regenerative payload with ISL.

Proposal 4: RAN3 is kindly asked to identify whether to support/enhance NTN-TN mobility case between one gNB on board and one gNB for TN.

Observation 3: It is possible that the satellite providing the service link to the UE is unchanged in case of inter-AMF mobility.

Proposal 5: RAN3 is kindly asked to discuss the mobility due to UE movement in case of regenerative payload, and identify the solution(s) for such open issue.

Proposal 6: For mobility due to UE movement, if we adopt the gNB collocation solution, a satellite should establish an additional logical gNB as the target gNB which is collocated with the source gNB. Consider the Xn based-like handover for this solution.

Proposal 7: For mobility due to UE movement, if we adopt the AMF re-allocation solution, RAN3 is kindly asked to consider the enhancement to reduce the NG signalling over SRI.

References