3GPP TSG-RAN WG3 Meeting #125bis	R3-245132

Hefei, China, October 14th – 18th, 2024

Agenda item:	 10.4

Source:		Samsung

Title:	(TP for SON BLCR for 38.423, 36.423) Rel-18 SON and MDT leftovers

Document for:		Discussion and Decision

1	Introduction

The following leftovers in Rel-18 SON/MDT have been agreed in the scope of Rel-19 SON/MDT WI.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

The contribution discussed how to support the three leftover issues based on the agreement so far.

2	Discussion

2.1 RACH optimization for SDT

RA-SDT has been designed to send the small data over RACH resources, which enables UE to transmit/send data in RRC inactive state. It is an efficient solution to avoid UE coming back to RRC connected state for the small data. gNB sets the following configuration parameters for RA-SDT and sends to UE via SIB1:

- RSRP threshold for UE to determine whether to perform SDT procedure

- Data volume threshold used to determine whether SDT can be initiated

- The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT

- A timer for SDT (T319a)

The SDT performance may be not good due to poor configuration. For example, with the low RSRP threshold, the data transmission may be interrupted due to poor channel condition. In addition, if data volume threshold is high but the timer is small, the data can not be sent within the timer limited period, so that the UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission. Thus, to do self-optimization, the information of RA-SDT is needed to transfer to the node who sets the configuration. The configuration information is broadcasted in system information block. For split architecture, the configuration parameters are set by DU, so F1AP impact should be considered to transfer the related information from CU to DU for DU self-optimization.

With above analysis, the following information should be provided by the UE to give the reference for node to do self-optimization.

- RSRP when SDT fails: this information gives reference information to the node to decide whether the RSRP threshold is set too low. For example, the RSRP threshold is set as a low value, while the channel condition as such threshold is not good to support the SDT transmission. If SDT fails, the nodes need to set the RSRP threshold as a higher value.

- Remaining data volume after SDT transmission: this information tells the node whether the data volume threshold is set properly or not. When the remaining data volume is small, it means that UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission for such a small data volume. The more desired way is to set a higher data volume threshold for the SDT triggering or set a larger timer for transmission.

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

2.2 MR-DC SCG failure scenario

In last RAN3#125 meeting, RAN3 made the following agreement and the TP in R3-244825 has been agreed as well.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

There was some offline discussion in the last day about the semantic description of IE “Time SCG Failure” in the SCG FAILURE INFORMATION REPORT message. But it cannot be discussed in detail due to lack of time. Furthermore, there is no procedure text about this optional IE. A TP is proposed for these two aspects in Annex Y.

Proposal 2: Agree the TP for TS36.423 in Annex B.

For NG-ENDC, the IE “Time SCG Failure” should be added in the SCG FAILURE INFORMATION REPORT message as well.

A TP for TS38.423 is given in Annex Z.

Proposal 3: Agree the TP for TS38.423in Annex C.

2.3 MHI Enhancement for SCG Deactivation/Activation

RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. For how to collect the time spent information, RAN2 identified two options as listed in R3-245009:

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

RAN2 kindly asks RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

MN knows whether SCG is deactivated or not, MN can collect the time percentage information. Based on the time information, the MN could understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity. E.g. if the time that SCG activation is low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the MN could deactivate DC for the UE.

Observation: Network based solution is enough to collect the information about time spent in the PSCell in activated or deactivated state.

Reply LS to RAN2 is provided in Annex D.

Proposal 4: Agree the LS to RAN2 in Annex D.

3	Conclusion

This contribution discussed MRO for MR-DC SCG failure and RACH optimization for SDT. We have the following proposals.

RACH optimization for SDT:

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

MRO for MR-DC SCG failure:

Proposal 2: Agree the TP for TS36.423 in Annex B.

Proposal 3: Agree the TP for TS38.423in Annex C.

MHI Enhancement for SCG Deactivation/Activation:

Observation: Network based solution is enough to collect the information about time spent in the PSCell in activated or deactivated state.

Proposal 4: Agree the LS to RAN2 in Annex D.

Reference

[1] TS38.331

[2] TS36.331

[2] TS38.423

Annex A: LS for RACH optimization for SDT

3GPP TSG RAN WG3 125-bis	R3-24XXXX

Hefei, China, 14 - 18 October, 2024

Title:	[Draft] LS on RACH Optimization for SDT

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Samsung (to be RAN3)

To:	RAN2

Cc:

Contact Person:

#### Name:	Yanru Wang

E-mail Address:	yanru <dot> wang <at> samsung <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 would like to inform RAN2 about the decision made on RACH optimization for SDT:

RAN3 agreed that there is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations, with the intention of avoiding signaling burden in Uu interface.

And RAN3 agreed to enhance RA report to include the information below:

- the time from the start of RA-SDT to the reporting of RA report

With such information, node can derive the corresponding configuration under which the SDT is done.

2. Actions:

To RAN2

RAN3 kindly asks RAN2 to take above into account and do corresponding specification work.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #126	18-22 November 2024			Orlando, USA

TSG-RAN3 Meeting #127	17-21 February 2025 			Athens, GR

Annex B: TP for TS36.423

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>

#### 9.1.4.x	SCG FAILURE INFORMATION REPORT

This message is sent by the MeNB to the en-gNB to report a PSCell change failure event.

Direction: MeNB  the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Source PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9] | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | timeSCGFailure included in SCGFailureInformationNR message as defined in TS36.331 [9] | YES | ignore |

Annex C: TP for BLCR TS38.423

#### 8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | timeSCGFailure included in SCGFailureInformationNR message as defined in TS36.331 [9] | YES | ignore |

Annex D: LS on MHI enhancement for SCG deactivation/activation

3GPP TSG RAN WG3 125-bis	R3-24XXXX

Hefei, China, 14 - 18 October, 2024

Title:	[Draft] Reply LS on MHI enhancement solution for SCG deactivation/activation

Response to:	R3-245009/ R2-2407664

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Samsung (to be RAN3)

To:	RAN2

Cc:

Contact Person:

#### Name:	Lixiang Xu

E-mail Address:	lx <dot> xu <at> samsung <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks RAN2’s LS on MHI enhancement solution for SCG deactivation/activation.

RAN3 has discussed how to collect the time spent information and concluded that network based solution is efficient enough.

2. Actions:

To RAN2

RAN3 kindly asks RAN2 to take above into account.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #126	18-22 November 2024			Orlando, USA

TSG-RAN3 Meeting #127	17-21 February 2025 			Athens, GR