3GPP TSG-RAN WG3 Meeting #125bis	   				         R3-245140

Hefei, China, 14th – 18th Oct 2024

Agenda Item:		14.3

Source:			China Telecom

Title:	Support of regenerative payload in NR NTN

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

The following agreement has been made at the end of previous RAN3 meeting:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads on NGAP.

- Discussion

2.1 NG interface management

In the previous RAN3 meeting, we agreed that NG Removal/Setup can be used for Regenerative Payload NG interface management. In addition, further discussions on benefits to introduce Suspend/Resume procedure for NG interface management are not precluded

We note that some companies argue that the NG connection with the old AMF may be suspended when the satellite leaves and be resumed when the satellite re-visit this area, so they tend to introduce a suspend/resume procedures to achieve fast recovery. However, there is no benefit for the NG suspend/resume after further consideration. First, it does not significantly save NG signaling overhead. Second, AMF is inefficient in managing a large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration. Due to the earth rotation, it may take several hours for the same NGSO satellite/gNB to return to the previous AMF service area. Maintaining these interfaces will result in large storage consumption of AMF and gNB.

Based on above analysis, we think that introducing NG removal procedure to realize the mobility management of on-board gNB is sufficient at the current stage, additional support for suspend/resume procedure does not seem to be beneficial.

Proposal 1: RAN3 does not pursue NG suspend/resume procedure for Regenerative Payload interface management.

2.2 Feeder Link Switch over

In NGSO, the high-speed movement of satellites along a predetermined orbit can lead to frequent feeder link switch over. RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Considering the gNB will also connect to new AMF, if during the feeder link switch these UEs can not be switched to subsequent gNB, e.g. it is moving cell, to support the assumption of AMF not change for old UEs the gNB should still keep connection with old AMF for a while for all the old UEs after feeder link switch over. The bearer connection between gNB and old AMF after feeder link switch over can be achieved by deployment. For the new UEs, the gNB should connect these UEs to new AMF. For the old UEs, the gNB should keep these UEs connected with old AMF until these UE are switched to subsequent gNB connecting to old AMF. These procedures are depicted as below:

Figure 1: Feeder link switch with AMF not changed for old UEs

Proposal 2: After feeder link switch over, gNB should keep connection with old AMF for a while for all the old UEs until these UE are switched to subsequent gNB, to support AMF no change for old UEs.

In the case of soft feeder link switch over, the gNB IP address changes can be handled according to existing NG-RAN procedures, i.e. gNB can establish a new SCTP association with AMF by using the new TNL address anchored in new NTN-GW. The gNB send a RAN CONFIGURATION UPDATE message over the new TNL, and AMF can know the new TNL association is available. After feeder link switch over, gNB can initiate a RAN CONFIGURATION UPDATE message to remove the old TNL address.

Proposal 3: The gNB IP address changes due to soft feeder link switch over can be handled by reusing existing multiple SCTP association.

As for NG-U, When the NTN-GW changes, the NG-U channel may also changes. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by the gNB initiating the PDU Session Resource Modify Indication procedure or Path Switch procedure for each UE. The Path switch procedure may need to be implemented via the Handover, which requires the satellite to support additional functions (e.g. the same physical gNB implementations two logical gNBs), with additional impact on CN,e.g. connection with two logical gNBs during Handover and one connection with one logical gNB after Handover.

Proposal 4: It is proposed to discuss how to support the NG-U tunnels update in case of Soft feeder link switch without relocation of the AMF. Our preference is using the PDU Session Resource Modify Indication procedure if we use existing UE-associated signalling.

In case of hard feeder link switch over, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch over, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Although the existing retransmission mechanism at application layer can work, it is better not to cause interruption for some services requiring service continuity (such as voice services). It is beneficial to inform the 5GC to buffer DL data based on actual situation of the UEs and feeder links, so the 5GC can suspend the DL NG-C/U.

Figure 2: Feeder link switchover without AMF change

- The gNB onboard the satellite connects to 5GC via NTN-GW1. In case of hard feeder link switchover, the gNB may informs AMF about the upcoming hard feeder link switchover. It can be per UE procedures or non-UE associated procedures.

2~4. The AMF informs the corresponding SMF(s) about the upcoming feeder link switchover receive from gNB. Based on feeder link switchover information, the SMF(s) controls the UPF(s) to buffer the DL data of the corresponding PDU sessions. It can be existing per UE procedures or new non-UE associated procedures, depending on further SA2 study.

- The gNB onboard the satellite performs Feeder link switchover to NTN-GW2. After the Feeder link is switched, the gNB is assumed to get a new TNLA.

- The gNB connects to AMF via NTN-GW2 by using the new TNLA as described in proposal 3.

7~8. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by using existing procedures as described in proposal 4 or define new non-UE associated procedures.

Proposal 5: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

2.3 TAI update

The Supported TAI List of onboard gNB is change over time for the NGSO. This requires pre-configuring mapping relationship among Global RAN Node ID, supporting TAI list and valid period. In the context of R17/R18, when RAN updates the TAI list, RAN will inform AMF of the updates. Even though there are frequent interactions between RAN and AMF, the cost of interactions is limited because the interactions are via ground link (e.g. fibre, cable, etc.). However, such interactions will be over feeder link in R19 and may be more frequent, whose bandwidth is scarce. OAM based solution is preferred to exchange the supported TAI list of the gNB.

Proposal 6: The Supported TAI List of onboard gNB can be exchanged via OAM.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: RAN3 does not pursue NG suspend/resume procedure for Regenerative Payload interface management.

Proposal 2: After feeder link switch over, gNB should keep connection with old AMF for a while for all the old UEs until these UE are switched to subsequent gNB, to support AMF no change for old UEs.

Proposal 3: The gNB IP address changes due to soft feeder link switch over can be handled by reusing existing multiple SCTP association.

Proposal 4: It is proposed to discuss how to support the NG-U tunnels update in case of soft feeder link switch without relocation of the AMF. Our preference is using the PDU Session Resource Modify Indication procedure if we use existing UE-associated signalling.

Proposal 5: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

Proposal 6: The Supported TAI List of onboard gNB can be exchanged via OAM.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.