3GPP TSG RAN WG3#125bis	R3-245144

14th October – 18th October - 2024

Hefei, China

Title: 	(TP for BLCR of TS38.300) On support of inter-CU LTM

Source: 	China Telecom

Agenda item:	13.3

Document for:	Approval

Introduction

Based on our discussion paper R3-245142, we provide this TP for BLCR of TS38.300 to support inter-CU LTM, including following undates:

- For Step3, add the description that the HANDOVER REQUEST message is sent for each LTM candidate cell, and update the corresponding editor’s notes.

- For Step5, update the HO REQUEST ACKNOWLEDGE to HANDOVER REQUEST ACKNOWLEDGE

- For Step6/7, add the description that the new introduced message can be used to transfer the collected LTM information.

- For Step10/11, update the description.

- Remove the editor notes under step 18 and 20.

TP for TS38.300

Based on the endorsed BLCR for TS38.300 [R3-244855].

<< First Change >>

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). A HANDOVER REQUEST message is sent for each LTM candidate cell.

Editor’s Note: Detials are FFS on step3.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HANDOVER REQUEST ACKNOWLEDGE) to the source gNB.

6.	The source gNB may send a  LTM CONFIGURATION UPDATE message to the candidate gNB(s) containing the information for subsequent LTM or for updating the configurations of candidate cells.

Editor’s Note: Details are FFS on step 6.

7.	The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB which may including the updated lower layer configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

Note: Step 6 may also be triggered after step 15, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. The UE may perform early synchronization with LTM candidate cell(s) before receiving the cell switch command. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB, which includes the TA values.

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

<< End of Changes >>