3GPP TSG RAN WG3#125bis	R3-245152

Heifei, China, 14 – 18 Oct. 2024

Agenda item:	12.3

Source: 	China Telecom

Title: 	On access control for NR Femto

Document for:	Discussion and decision

- Introduction

In RAN#105, the work on “Additional topological enhancements for NR” goes on the work item phase from the study item [1]. The Femto related objectives in the new work item were listed below:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this paper, we discuss on access control related issues and give our observations and proposals.

- Discussion on access control for NR Femtos

- At study item phase, we have agreed to reuse the existing CAG mechanism for supporting access control, including the open, hybrid and closed access mode, the details in TR 38.799 are as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

- Basically, reusing the CAG (Closed Access Group) mechanism can support all the desired functions for the NR Femto node. However there are still some differences between the LTE CSG (Closed Subscriber Group) access control mechanism and the NR Femto mechanism.

2.1 Access control handing

For LTE HeNB (Home eNodeB), the eNB could handle UE (User Equipment) access. When a UE attempts to connect to a HeNB, the HeNB sends the CSG ID, which represents the Closed Subscriber Group identifier for the UE, within the INITIAL UE MESSAGE to the MME. The MME then verifies the UE's authorization and responds with the CSG Membership Status, included in the INITIAL CONTEXT SETUP REQUEST message. Based on the CSG Membership Status, the HeNB decides whether to allow the UE to access the CSG cell.

However, for NR Femto, the existing CAG mechanism does not support NR Femto in handling UE access to a CAG cell. In the current CAG mechanism, when a UE attempts to access a CAG cell, the base station sends a CAG ID list, representing the supported CAG cells of it, within the INITIAL UE MESSAGE to the AMF. The AMF verifies the UE’s authority but does not send the CAG status of the UE back to the base station via the INITIAL CONTEXT SETUP REQUEST message. As a result, the base station cannot reject unauthorized UEs from accessing the CAG cell. Instead, the AMF determines whether the UE can access the CAG cell using NAS signaling. If we reuse the existing mechanism without updates, the NR Femto will be unable to manage UE access control effectively.

Observation 1: If the current CAG mechanism is reused without any modifications, the NR Femto will not be able to manage UE access control effectively.

However, if a large number of NR Femto nodes are to be deployed in a scalable manner, handling all UE access control to the CAG cells solely through the 5GC is inefficient, as Femto deployments are more scattered and diverse.

Observation 2: Access control managed by the NR Femto is more efficient than having it handled by the AMF.

Proposal 1: For Femto cells, the access control should be managed by the NR Femto, similar to how it is done by the HeNB.

2.2 Mobility handing

For LTE Femto mechanism, HeNB GW is responsible for relaying UE-associated S1 application part messages between the MME serving the UE and the HeNB. It is logical for the NR Femto GW to take on the similar responsibility.

For the HeNB GW, both the CSG ID Information Element (IE) and Cell Access Mode IE are included in the S1 PATH SWITCH REQUEST and S1 HANDOVER REQUEST ACKNOWLEDGE messages. This allows the gateway to verify that, for a closed HeNB, the indicated cell access mode and CSG ID are valid for that HeNB.

In the current CAG mechanism, no CAG-related information is included in NG PATH SWITCH REQUEST message. As a result, the NR Femto GW cannot verify whether the indicated cell access mode and CAG ID are valid for a closed HeNB through the path switch message. All the decisions are made by AMF.

Observation 3: The NR Femto GW is unable to verify whether the indicated cell access mode and CAG ID are valid for a closed HeNB.

Proposal 2: A NR Femto GW should have the ability to verify the validity of the indicated cell access mode and CAG ID for a closed HeNB during the PATH SWITCH procedure.

- Conclusion

Based on the above discussion of access control related issues for NR Femto, we have the following observations and proposals:

Observation 1: If the current CAG mechanism is reused without any modifications, the NR Femto will not be able to manage UE access control effectively.

Observation 2: Access control managed by the NR Femto is more efficient than having it handled by the AMF.

Observation 3: The NR Femto GW is unable to verify whether the indicated cell access mode and CAG ID are valid for a closed HeNB.

Proposal 1: For Femto cells, the access control should be managed by the NR Femto, similar to how it is done by the HeNB.

Proposal 2: A NR Femto GW should have the ability to verify the validity of the indicated cell access mode and CAG ID for a closed HeNB during the PATH SWITCH procedure.

- Reference

[1] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[2] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.