3GPP TSG RAN WG3#125bis	R3-245153

Heifei, China, 14 – 18 Oct. 2024

Agenda item:	17.2

Source: 	China Telecom

Title: 	On support on-demand SSB SCell operation

Document for:	Discussion and decision

- Introduction

In RAN#105, the work item on “Enhancements of network energy savings for NR” was revised [1]. The three objectives in the work item were listed below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA;

- Specify support for on-demand SIB1 for UEs in idle/inactive mode;

- Specify adaptation of common signal/channel transmissions.

In this paper, we discuss on the OD-SSB SCell related issues based on the latest progress of RAN1 and RAN2.

- Discussion on OD-SSB SCell operation

2.1 Scenarios for on-demand SSB SCell option

RAN1 has agreed four use cases for on-demand SSB SCell operation (Scenario #2 and Scenario #2A combined with case #1 and case #2) in RAN1#116bis.

In last RAN2 meeting, the following agreements have been made on scenarios on OD-SSB.

Agreements on OD-SSB Scenarios on OD-SSB: RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

Based on the progress from RAN1 and RAN2, Scenarios 2/2A including both case 1 and case 2 are clearly supported in the WID. Following the steps from RAN1 and RAN2, we can initiate the discussion on necessary procedures and signaling methods to support on-demand SSB SCell operation while awaiting the conclusion on scenarios 3A/3B.

Observation 1: RAN3 can begin the discussion on the required procedures and signaling methods to support on-demand SSB SCell operation.

2.2 Analysis on potential RAN3 impacts

The agreements related to RRC signaling from the most recent RAN2 and RAN3 meeting were listed below.

RAN1 agreements in RAN1#118

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. R1-2407332 Summary #3 of on-demand SSB for NES Moderator (LG Electronics) Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

RAN2 agreements in RAN2#127

OD-SSB transmission indication: RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Based on the above agreements, RRC signaling will be used to indicate on-demand SSB transmission and the activation/deactivation state of OD-SSB. In the case of a CU/DU split, changes in RRC signaling affect the F1 interface, so we need to discuss the specification impact on F1.

Observation 2: For CU/DU split case, since the RRC signaling is related to F1 interface, the specification impact on F1 need to be discussed.

The inter-gNB scenario between Pcell and NES Scell has not been discussed yet, so there is currently no need to address the protocol impact of Xn interface. We can follow the progress of RAN1 and RAN2 to see there is any impact on Xn interface.

Observation 3: At this time, there is no need to discuss the protocol impact on the Xn interface.

2.3 Analysis on potential F1 impacts

- Which node should configure SCell to use the on-demand SSB transmission mode

Currently, the CU configures the UE to use carrier aggregation (CA) to improve the traffic throughput as it has better knowledge of the transmission requirements and mobility state of the UE than the DU. Thus, CU should be responsible for deciding whether the SCell should stake the responsibility of deciding whether SCell should switch to the on-demand SSB operation. If the CU determines the transmission state of UEs does not require continuous SSB transmission, it can configure the UE to be ready for the on-demand SSB transmission via RRC signaling, and inform the DU that the cell can switch to the on-demand SSB operation mode.

Proposal 1: CU should decide whether the SCell can operate in the on-demand SSB transmission mode and inform the DU via F1 signaling.

- Which node make the decision to trigger the on-demand SSB operation

The purpose of on-demand SSB operation is to reduce power consumption during SSB transmission. Unlike always-on SSB transmission, the on-demand SSB is only transmitted when required by UEs. If users can synchronize with the SCell or access it normally without the need for SSB transmission, the SSB will not be transmitted to reduce the gNB power consumption.

Since cell load changes frequently, the on-demand SSB operation needs to be performed quickly. Therefore, the DU should trigger the on-demand SSB operation by using MAC CE, which is agreed by RAN1 and RAN2 as MAC CE is used for faster configuration of UE operation.

Proposal 2: DU should trigger the activation/deactivation of the on-demand SSB operation using MAC CE and inform the CU of the SSB operation state via F1 signaling.

- Conclusion

Based on the above discussion regarding support for on-demand SSB operation, we have the following observations and proposals:

Observation 1: RAN3 can begin the discussion on the required procedures and signaling methods to support on-demand SSB SCell operation.

Observation 2: For CU/DU split case, since the RRC signaling is related to F1 interface, the specification impact on F1 need to be discussed.

Observation 3: At this time, there is no need to discuss the protocol impact on the Xn interface.

Proposal 1: CU should decide whether the SCell can operate in the on-demand SSB transmission mode and inform the DU via F1 signaling.

Proposal 2: DU should trigger the activation/deactivation of the on-demand SSB operation using MAC CE and inform the CU of the SSB operation state via F1 signaling.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.