3GPP TSG-RAN WG3 #125-bis                                                                R3-245163

Hefei, China, 14 - 18 October, 2024

Agenda item:	13.2

Source:	Samsung

Title:	Additional discussion on Inter-gNB-CU LTM

Document for:	Discussion & Decision

Introduction

In this contribution, we will further discuss some questions based on the conclusions from last meeting or other issues on the procedure of inter-gNB-CU LTM.

Discussion

## Issue 1: Early TA CFRA resource allocation

In RAN2#125bis meeting, the following agreement was reached on early UL Sync:

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

From the RAN2’s agreements, we note Rel-18 option is baseline for early TA acquisition. In R18, early TA acquisition by PDCCH order is supported, and the dedicated different CFRA resource is allocated by candidate cell for each candidate gNB-DU. CFRA resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU.

In our view, this holds true also for inter-gNB-CU case and we should allow the possibility of CFRA resource shared only for among UEs belonging to same DU. Hence, if LTM gNB is a split gNB, dedicated different CFRA resource still can be allocated by candidate cell for each candidate gNB-DU.

However, only gNB-DU ID information is not enough since two gNB-DUs of different gNB-CUs may have the same gNB-DU ID and candidate gNB does not know which gNB to the gNB-DU belongs. Therefore, both the gNB-DU ID IE and the belonging gNB ID IE should be included in the HANDOVER REQUEST message to candidate gNB for requesting early TA CFRA resource.

Moreover, to support subsequent LTM, early TA CFRA resource can be allocated previously for each LTM gNB-DUs. Therefore, HANDOVER REQUEST is initiated by source gNB-CU which contains a list of pair <gNB-DU ID and associated belonging gNB ID>, and next, candidate gNB-CU carries this list of the pair <gNB-DU ID and associated belonging gNB ID> in the UE CONTEXT SETUP REQUEST message to request early TA CFRA resource to candidate gNB-DU. The candidate gNB-DU allocates different dedicated early TA CFRA resources based on each pair of <gNB-DU ID and associated belonging gNB ID>.

Proposal 1: Introduce a list of the pair <gNB-DU ID and associated belonging gNB ID> in the HANDOVER REQUEST and UE CONTEXT SETUP REQUEST message for early TA CFRA resource request.

Proposal 2: The candidate gNB-DU allocates different dedicated early TA CFRA resources based on each pair of <gNB-DU ID and associated belonging gNB ID>.

## Issue 2: TA information transfer

PDCCH ordered early RA procedure is supported for inter-CU LTM, while candidate gNB-DU calculates out the TA value according to Msg1 from UE, early TA information transfer procedure is triggered from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU. F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message could be reused to transfer TA value and the associated CFRA resource information from the candidate gNB-DU to candidate gNB-CU. However, only indicating source gNB-DU ID in DU-CU TA INFORMATION TRANSFER message as in current specification is not enough, since candidate gNB-CU does not know which gNB/gNB-CU via a non-UE-associated XnAP signalling to forward this information to. Therefore, source gNB ID IE is also indicated to candidate gNB-CU in the F1AP DU-CU TA INFORMATION TRANSFER message.

Proposal 3: Both source gNB-DU ID and source gNB ID are included in F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message, for candidate gNB-CU to forward TA information to source gNB-CU indicated by source gNB ID using XnAP non-UE-associated signaling.

## Issue 3: Data forwarding mechanism for subsequent LTM

This issue is raised in last meeting, for subsequent LTM, how to transmit the collected data forwarding information to the candidate gNB(s) need to be further discussed in RAN3.

Data forwarding is a mechanism that can reduce data transmission interruption during inter-gNB (conditional) handover procedure. In current data forwarding principle, source gNB generates the data forwarding proposal via the Data Forwarding and Offloading Info from source NG-RAN node IE in HO REQ message, target gNB provides the data forwarding addresses per-PDU session or per-DRB via the Data Forwarding Info from target NG-RAN node IE in HO REQ ACK message.

From the view of RAN3, data forwarding is also supported for inter-CU LTM. However, in subsequent inter-CU LTM, the serving gNB will be changed after the execution of LTM, any one candidate gNB may be changed as source gNB for the following execution of LTM. If new serving gNB generates the data forwarding proposal to other candidate gNBs and gets the data forwarding addresses after each execution of LTM, it could delay the subsequent data forwarding operation from the new serving gNB and involve with additional signalling interactions. From this sense, we think the better approach is that candidate gNB provides the data forwarding proposals in advance during LTM preparation phase. Therefore, we propose that source gNB collects the data forwarding addresses and data forwarding proposal information in Handover Request Ack message from all the candidate gNBs, and then it should send all the data forwarding addresses to each candidate gNB if this candidate gNB proposed data forwarding. When UE access this candidate gNB, data forwarding, especially for early data forwarding, can be initiated timely from this candidate gNB to other candidate gNBs.

In short, based on above analysis, in order to avoid to delay the subsequent data forwarding operation from the new serving gNB and involve with additional signalling interactions after each execution of LTM, we have below proposals for the data forwarding mechanism for subsequent LTM.

Proposal 4: Candidate gNB provides the data forwarding proposal and TNL addresses information for admitted PDU sessions or DRBs in HO REQ ACK in advance during LTM preparation phase.

Proposal 5: Source gNB provides the collected all the data forwarding addresses information (including source and candidate gNB) to other each candidate gNB if this candidate gNB proposes data forwarding proposal during LTM preparation phase.

## Issue 4: Inter-node message for LTM-Candidate

During inter-CU LTM candidate cell preparation procedure, the gNB-CU of LTM candidate cell needs to provide the information in LTM-candidate to the source gNB-CU. In R18, the LTM-Candidate IE contains several IEs in addition to RRCReconfiguration container, as shown below:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

However, during legacy handover preparation procedure, the target gNB-CU only provides the RRCReconfiguration message as the inter-node RRC message to the source gNB-CU. Thus, the issue is how to provide other information in LTM-Candidate by the LTM candidate gNB-CU. Two options can be considered:

- Option 1: Define a new inter-node RRC message for LTM-Candidate

This option has RAN2 specification impact. Meanwhile, the source gNB-CU should be able to decode this message since some information inside needs to be sent to the source gNB-DU, e.g., Early UL sync configuration, UE measured TA ID.  Such option can simplify RAN3 signaling design since the Handover Request Acknowledge message only needs to include the container of LTM-Candidate.

- Option 2: Define new XnAP IEs to include other information in the LTM-Candidate

This option does not have RAN2 specification impact. Instead, it will cause large specification impact in RAN3. The reason is that RAN3 should add IEs in LTM-candidate over Handover Request Acknowledge message.

In our understanding, Option 1 is a simple solution, and the further extension to the ltm-candidate IE can be kept in RAN2 scope in the future.

Proposal 6: RAN3 is kindly asked to discuss the LTM candidate configuration (LTM-Candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

- Option 1: Define a new inter-node RRC message for LTM candidate configuration

- Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration

Conclusion

Early TA CFRA resource allocation

Proposal 1: Introduce a list of the pair <gNB-DU ID and associated belonging gNB ID> in the HANDOVER REQUEST and UE CONTEXT SETUP REQUEST message for early TA CFRA resource request.

Proposal 2: The candidate gNB-DU allocates different dedicated early TA CFRA resources based on each pair of <gNB-DU ID and associated belonging gNB ID>.

TA information transfer

Proposal 3: Both source gNB-DU ID and source gNB ID are included in F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message, for candidate gNB-CU to forward TA information to source gNB-CU indicated by source gNB ID using XnAP non-UE-associated signaling.

Data forwarding mechanism for subsequent LTM

Proposal 4: Candidate gNB provides the data forwarding proposal information for admitted PDU sessions or DRBs in HO REQ ACK in advance during LTM preparation phase.

Proposal 5: Source gNB provides the collected all the data forwarding addresses information to other each candidate gNB if this candidate gNB proposes data forwarding proposal.

Inter-node message for LTM-Candidate

Proposal 6: RAN3 is kindly asked to discuss the LTM candidate configuration (LTM-Candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

- Option 1: Define a new inter-node RRC message for LTM candidate configuration

- Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration