3GPP TSG-RAN WG3 Meeting #125bis		R3-245165

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:	Discussion on LS from SA2 on multi-modality

Agenda Item:	21.1

Document for:	Discussion and Decision

- Introduction

In RAN3#125bis meeting, a replied LS from SA2 is received on multi-modality awareness at RAN [1]:

SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information.

In this contribution, we will discuss the feasibility and specification impacts of forwarding the Multi-Modal Service ID (MMSID) from CN to RAN from RAN3 point of view.

- Discussion on MMSID to RAN via CP solution

In TS 23.501, MMSID is defined as “The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.” So that the MMSID is an indication per QoS flow. Such information could be included in IE “QoS Flow Level QoS Parameters” in PDU SESSION RESOURCE SETUP REQUEST，HANDOVER REQUEST，PDU SESSION RESOURCE MODIFY REQUEST of NGAP messages. Once serving NG RAN Node obtains such information, it should be stored as UE context together with other QoS related parameters.

Observation 1: MMSID could be sent from CN to NG RAN in “QoS Flow Level QoS Parameters” IE in PDU SESSION RESOURCE SETUP REQUEST，HANDOVER REQUEST，PDU SESSION RESOURCE MODIFY REQUEST messages.

Furthermore, in order to support mobility and dual connectivity operation, such QoS information should also be forwarded between two gNBs via XnAP messages in the following cases.

- Handover Request from source NG-RAN Node to target NG-RAN Node

- S-Node addition request or S-Node modification request from M-NG-RAN Node to S-NG-RAN Node for SN terminated and MN terminated DC operation

- S-Node modification require from S-NG-RAN Node to M-NG-RAN Node for S-NG-RAN node initiated UE context modification

Observation 2: MMSID could be sent between two gNBs via XnAP messages in “QoS Flow Level QoS Parameters” IE in HANDOVER REQUEST, S-NODE ADDITION REQUEST ，S-NODE MODIFICATION REQUEST ，NODE MODIFICATION REQUIRED messages.

Thirdly, when considering Multi-Modal service information could help RAN to do scheduling enhancement, such information should also be transferred to gNB-DU via F1 signaling in the following cases.

- UE Context Setup Request from gNB-CU to gNB-DU

- UE Context Modification Request from gNB-CU to gNB-DU

Observation 3: MMSID could be sent from gNB-CU to gNB-DU via F1AP messages in “QoS Flow Level QoS Parameters” IE in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

With the above analysis, we think there is no technical feasibility issue when forwarding MM association information (e.g. MMSID) to RAN via control plane, as well as further exchange between gNB or from gNB CU to DU.

Proposal 1: From RAN3 perspective, MM association information i.e. MMSID, could be sent from CN to RAN, and further exchange between two gNBs and from gNB CU to gNB DU via NGAP, XnAP and F1AP signalling respectively. Send a LS to SA2/RAN2/SA4 to reply the feasibility of forwarding Multi-Modal service association information.

- Conclusion

In this contribution, we discuss the issues in the LS from SA2 [1]. Based on the discussion, we have the following observations and proposals:

Observation 1: MMSID could be sent from CN to NG RAN in “QoS Flow Level QoS Parameters” IE in PDU SESSION RESOURCE SETUP REQUEST，HANDOVER REQUEST，PDU SESSION RESOURCE MODIFY REQUEST messages.

Observation 2: MMSID could be sent between two gNBs via XnAP messages in “QoS Flow Level QoS Parameters” IE in HANDOVER REQUEST, S-NODE ADDITION REQUEST ，S-NODE MODIFICATION REQUEST ，NODE MODIFICATION REQUIRED messages.

Observation 3: MMSID could be sent from gNB-CU to gNB-DU via F1AP messages in “QoS Flow Level QoS Parameters” IE in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

Proposal 1: From RAN3 perspective, MM association information i.e. MMSID, could be sent from CN to RAN, and further exchange between two gNBs and from gNB CU to gNB DU via NGAP, XnAP and F1AP signalling respectively. Send a LS to SA2/RAN2/SA4 to reply the feasibility of forwarding Multi-Modal service association information.

- References

- R3-245017 /S2-2409444, LS reply on multi-modality awareness at RAN, SA2

- Annex - draft reply LS

Title:	[Draft] reply LS on multi-modality awareness at RAN

Response to:	R3-245017/S2-2409444

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	TBD [to be RAN3]

To:	SA2

Cc: 	RAN2, SA4

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

RAN3 would like to thank SA2 for the LS reply on multi-modality awareness at RAN in [R3-245017/S2-2409444].

RAN3 has discussed the questions in the LS, and concluded the following responses [To be updated based on the discussion conclusion]:

Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts.

From RAN3 perspective, MM association information i.e. MMSID, could be sent from CN to RAN, and further exchange between two gNBs and from gNB CU to gNB DU via NGAP, XnAP and F1AP signalling respectively.

# 2. Actions:

# To SA WG 2:

# RAN3 kindly request SA2 to take the above information into account during the future work, and provide feedback, if any.

# 3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #126		18th Nov. – 22nd Nov. 2024		Orlando, USA

TSG RAN WG3 Meeting #127		17th Feb. – 21st Feb. 2025		Athens, Greece