3GPP TSG-RAN WG3 Meeting #125bis													R3-245169

Hefei, China, 14th - 18th October 2024

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the LS related to XR

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from SA2 on multi-modality awareness at RAN. RAN3 also received a reply LS ([2]) from RAN2  on UL PSI based PDU discarding in NR-DC.  This contribution analyses these LS.

# 2	Discussion on multi-modality awareness at RAN

In the SA2 LS ([1]) on multi-modality awareness at RAN, the RAN3 related aspects are highlighted.

1. Overall Description: SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. 2. Actions: To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively.

A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). The Multi-modal Service ID (MMSID) is an explicit indication that data flows are related to a multi-modal service. During the QoS Flow establish/Modification procedure, the PCF receives the Multi-modal Service ID, the service requirements and the QoS monitoring requirements.

It is worth to mention that extensive study on coordinated/synchronized delivery has been done in SA2 during Rel-18 SA2 study item. In the end, SA2 agreed to only introduce MMSID which can be used by PCF to derive the correct PCC rules and appropriate QoS policies for example packet delay budget, partially due to the relatively relaxed synchronization requirements according to SA1 [Table 6.43.1-1, TS 22.261]. In SA2#163, it has been discussed if any multi-modality information can be provided to RAN based on the LS from RAN2. However, no consensus has been made in SA2, which is the same situation as Rel-18 SA2 study item.

Observation 1: In SA2, there is no consensus reached in providing MMSID to RAN, which is the same situation as Rel-18 SA2 study item.

This issue was also discussed in RAN plenary #105, and agreed following objective in the WID update ([3]):

Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information based approach as an alternative. RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#106 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q1/25 onwards.

In case SA2 agree the NG-RAN node need to know the MMSID, the CN can provide MMSID to the NG-RAN node via NGAP procedure, e.g. during the QoS Flow establishment/modification procedure. It is technically feasible to enhance NGAP, but whether the enhancement is needed should be decided by SA2.

Regarding the applicability and usage of this information, it should be the gNB’s implementation, e.g. use the information for scheduling or admission control, etc.

Proposal 1: it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node during the QoS Flow establishment/modification procedure. The usage of the MMSID is up to the implementation of the NG-RAN node.

It is proposed to reply SA2 based on above proposal. The draft Reply LS can be found in [4]

# 3	Discussion on UL PSI based PDU discarding in NR-DC

In RAN3#124, RAN3 send a LS to RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE. RAN2 provided a reply LS ([2]).

# 1. Overall Description: RAN2 thank RAN3 for the questions on PSI-based SDU discard on UL split bearer in NR-DC. The following is the current UE behavior: - UE activates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE activating the discard, regardless of whether the MAC CE is received from MN or SN; - UE deactivates PSI-based SDU discard on a UL split-bearer upon reception of a PSI-Based SDU Discard De-/Activation MAC CE deactivating the discard, regardless of whether the MAC CE is received from MN or SN. It is up to RAN3 to discuss whether any network side solution to improve PSI-based SDU discard is needed.

RAN3 had similar discussion in previous releases on PDCP duplication.  RAN3 discussed various options with/without the coordination between MN and SN, and adopted the solution without the coordination between MN and SN. In addition, this DC objective does not have impact to RAN2. So it is preferred that RAN3 adopt a solution without requiring the information to be transferred from MN to SN, or from SN to MN.

Proposal 2: The RAN2 reply LS can be noted. No XnAP enhancement is needed to support the UL PSI based discard in NR-DC.

# 4	Conclusion

In this contribution, we have analyzed the LS from SA2 and RAN2. Our proposals are:

For multi-modality awareness at RAN:

Proposal 1: it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node during the QoS Flow establishment/modification procedure. The usage of the MMSID is up to the implementation of the NG-RAN node.

It is proposed to reply SA2 based on above proposal. The draft Reply LS can be found in [4]

For UL PSI based PDU discarding in NR-DC:

Proposal 2: The RAN2 reply LS can be noted. No XnAP enhancement is needed to support the UL PSI based discard in NR-DC.

# References

- R3-245017/S2-2409444	LS on multi-modality awareness at RAN

- R3-245011/R2-2407780	Reply to RAN3 LS on UL PSI based PDU discarding in NR-DC

- RP-241771					Revised WID on XR (eXtended Reality) for NR Phase 3

- R3-235170					[Draft] Reply LS on multi-modality awareness at RAN