3GPP TSG-RAN WG3 Meeting #125bis													R3-245176

Hefei, China, 14th - 18th October 2024

Agenda item:	12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) Discussion on WAB mobility

Document for:	Discussion and Decision

# 1	Introduction

RAN#105 agreed WI on topological enhancements for NR [1]. The objectives related to WAB mobility are following, [1]:

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - … - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - … NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

This contribution analyses the issues related to WAB-node mobility, and the support for UEs sent to RRC_INACTIVE via a WAB-gNB.

# 2	WAB-gNB mobility without change of UE’s AMF(s)

Last RAN3 meeting the following was agreed:

##### --- Start of TR38.799 text ---

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [7].

##### --- End of TR38.799 text ---

There is no Stage-3 impact to RAN3. Stage-2 text may be needed.

Proposal 1: Update Stage-2 to capture the WAB-node mobility without change of UE’s AMF.

The TP can be found in ([3])

# 3	WAB-gNB mobility with change of UE’s AMF(s)

The SI concluded with the approval that two logical WAB-gNB based approach is feasible, and proposing to study three options with single WAB-gNB approach further.

###### 4.3.4.2.2.2	Solution with single logical WAB-gNB

It may be possible to support the change of UE’s AMF(s) with a single logical WAB-gNB on the WAB-node. The following options may be considered, their feasibility and the impact on CN needs to be confirmed with SA2, and potentially RAN2 if needed.

Option 1: Single WAB-gNB with a single cell using mobility registration update due to TAC change

In this option, the WAB-gNB establishes a new NG connection towards the new AMF and concurrently maintains NG connections to both AMFs. The WAB-gNB reports a new TAC only to the new AMF. The WAB-gNB initiates the change of the UE’s AMF by updating the SI with the new TAC. When the UE detects the new TAC in the SI broadcast, it initiates the Mobility Registration Update procedure as defined in TS 23.502 [7] clause 4.2.2.2.3. After all UEs have been migrated to the new AMF, the NG connection between the WAB-gNB and the initial AMF(s) can be removed.

To enable this option, modifications to gNB behavior may be needed.

Option 2: Single WAB-gNB with two cells with different TACs, using NG-based HO

In this option, the procedures defined in clause 4.3.4.2.2.1 of the present document can be reused with the difference that the new cell(s) and the old cell(s) are served by the same WAB-gNB, i.e., no new logical WAB-gNB needs to be instantiated. The gNB-ID part of the cell ID of the new cell is the same as that of the old cell. The WAB-gNB further has to report the new TAC only to the new AMF as described in Option 1. This ensures that AMF reallocation can be achieved via the NG-based handover for RRC_CONNECTED UEs and via Mobility Registration Update for RRC_IDLE/RRC_INACTIVE UEs.

To enable this option, modifications to gNB behavior may be needed.

Option 3: Single WAB-gNB single cell without TAC change

In this option, during the AMF change, the WAB-gNB retains its TAC. When the WAB-gNB establishes an NG connection to the new AMF, the WAB-gNB indicates the TAC to the new AMF, and removes the TAC from the supported TAC list at the initial AMF. After this, the UE context transfer between the old and the new AMF is triggered, which requires enhancements in the core network (e.g., either the initial AMF or the new AMF can trigger UE context transfer for both RRC_CONNECTED and RRC_IDLE UEs, for example based on the GUAMI of the new or initial AMF, respectively).

SA2 had made the following conclusion:

The MWAB instantiates a new logical MWAB gNB when it is configured with a new TAC (and possibly Cell ID) and with a different gNB ID. This may be used to support migrating / handing over of UEs served by the MWAB to the new logical gNB. Once all UEs have been migrated, the MWAB removes the old logical MWAB-gNB instance. This can happen e.g. when AMF, serving the UEs of the MWAB, change is required based on the operator policy.

Observation 2-1: SA2 conclusion indicates that WAB-gNB initiates a new gNB when configured with a new TAC and a different gNB ID, e.g., in case of AMF change.

Note that both Option 1 and 3 were raised in SA2 during study phase, which were then ruled out. Option 1 corresponds to Solution#8 in SA2 TR. Also, neither Option 1, nor Option 3 can provide service continuity, and UEs would face interruption, which can be avoided/minimized for Option 2.

Furthermore, SA2 agreed normative text in S2-2409245 that captures the following: When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change. Therefore, both option 1 and option 3 are ruled out by SA2 already.

PCI (re-)configuration for WAB

When the mobile WAB moves, there may be a potential PCI collision between the cells from different WAB, or between WAB’s cell and a stationary gNB-DU’s cell. The WAB may need to change PCI for its cell. The connected UE will see the different cell before and after the PCI change. HO procedure can be used to move the UE from the “old” WAB cell using an old PCI and the “new” WAB cell using the new PCI. It can be considered as inter-cell intra-gNB HO. To support this HO, the WAB may need to have both “old” cell and “new” cell activate simultaneously.  This is similar to Rel-18 mobile IAB-DU migration that the UE is HO from the source logical IAB-DU’s cell to the co-located target logical IAB-DU’s cell.  After all connected UEs are HO to the new cell, the WAB-gNB may deactivate the old cell.

The support for PCI change only works by active two logical cells simultaneously, i.e. one cell using the old PCI and the new cell using the new PCI. So this also proves the need for Option 2 above. Option 1 and Option 3 cannot support the PCI change. In case RAN3 send a LS to SA2 for the three options, it is necessary to add the support for change of PCI.

Observation 2-2: To support for PCI change of a WAB’s cell, the WAB need to have two logical cells active simultaneously. The connected UE can be HO from the old cell to the new cell.

Proposal 2-1: To support the mobility of WAB-node with the change of UE’s AMF, and the support for PCI change, the WAB-gNB need to have two logical cells active simultaneously.

The other question left for RAN3 is whether it can be same logical gNB to setup NG with old AMF and new AMF.  This is possible. The main requirement is separate cells are required, i.e. separate “old cell” and “new cell”. Whether they belong to same logical gNB is just a matter of whether same gNB ID or different gNB ID is used in both old cell ID and new cell ID. A gNB may setup NG connection with multiple AMFs. In NG-Flex configuration, a gNB is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node. In NG-Flex case, the gNB uses same Supported TA List. In WAB case, the WAB-gNB connects with AMFs from different AMF Region, so the WAB-gNB shall use different Supported TA List for connecting with “Old AMF” and “New AMF”. It is not possible to use same Supported TA List for connecting with “Old AMF” and “New AMF”, since TAI is used for routing to different AMF. It may be possible to use same gNB ID, but should use different cell ID in order to be different cells to the UE.

Observation 2-3: it works by using a single logical gNB.

Proposal 2-2: WAB-gNB shall use different Supported TA List for setting up NG connection with “New” AMF.

# 4	Change of backhaul resources during mobility

WAB-gNB resources/interfaces, e.g. NG, OAM, Xn, and on-board UE sessions may be mapped to different backhaul PDU sessions and network slices. During WAB-node mobility, the availability of different network slices and/or QoS resources from different BH-gNBs could change, which could impact the backhaul quality of the WAB-node and hence the resources of the WAB-gNB and the UEs connected to it (note that this problem could even happen when WAB is not moving, e.g. if the serving BH-gNB is experiencing congestion and needs to scale down BH resources).

Depending on which slices/resources are impacted the problem can be more severe. For example, if a slice or PDU session associated with the WAB-gNB’s NG interface can no longer be supported, there would be loss of connection to the UE’s 5GC, in which case both the WAB-gNB and the on-board UEs would be disconnected suddenly.

Observation 3-1: During WAB-node mobility, WAB-gNB resources can be degraded due different QoS resources and/or network slices that can be supported at BH-gNBs.

Therefore, it would be beneficial for the BH-gNB to be able to indicate to the WAB-gNB when backhaul service requirements can no longer be maintained so that the WAB-gNB can react accordingly, e.g. free up resources by releasing Xn with neighbouring nodes, handing over UEs to surrounding gNBs, remapping not supported slices/resources to supported slices/resources, etc.

Proposal 3-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

In order to make this determination, it would be useful for the BH-gNB to know which slices/PDU sessions are associated with specific resources at the WAB-gNB (or to at least have some awareness of which slices/sessions should be prioritized). During handover preparation, such information could be used by the source BH-gNB to determine whether a candidate target BH-gNB can maintain BH service and inform the WAB-gNB if QoS is expected to be degraded.

Proposal 3-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

# 5	PCI collision avoidance

For PCI collision detection and avoidance with WAB deployment, there are similar options as in legacy systems as well as with mobile IAB. These can be the served cell and neighbour cell information exchange over Xn which can indicate PCIs in the surrounding cells. Additionally, UE ANR measurements using the legacy procedures can be used to detect unknown cells. The ANR procedure can be initiated by the WAB-gNB but also the WAB-MT itself can scan the cells to detect new cells when moving. All, these options are supported by existing specifications.

Furthermore, like considered for mIAB, there could be a split of PCI value ranges for fixed and mobile cells which prevents the collisions between fixed and mobile cells. The remaining problem would be the potential PCI collision between the WAB cells. However, also for such case the legacy features can be used to prevent the collisions.

Observation 4-1: Legacy procedures are applicable for PCI collision detection for WAB deployments.

If a collision is detected, or there is a risk of PCI collision, WAB-gNB can be configured with a new PCI by OAM. The PCI change would be a cell change for the UEs despite connecting to the same node. The WAB-gNB can instantiate another logical cell with the new PCI and UEs connected to the WAB-gNB can be handed over from the old cell to the new cell. The procedure in this case can be identical to the mobility scenarios e.g. when the AMF changes and  a new cell (or logical gNB) is configured to hand over UEs to the new cell/gNB.

Observation 4-2: The PCI change can apply the WAB mobility procedure where new (logical) cell is activated and UEs are handed over between old and new cell.

Proposal 4: No WAB specific enhancements are needed for PCI collision avoidance.

# 6	Discussion on the SA2 LS

RAN3 received a SA2 LS ([3]) with following questions:

Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

This section analyses aspects related to questions in the SA2 LS.

Additional ULI

SA2 conclusion indicates the MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

- TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

- If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN,

- option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

- option 2: geo-location of the MWAB.

- option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

The purpose of the additional ULI is to enable the UE’s CN to have a better knowledge of UE’s location and may perform location-based policy control.

For Option 1, it is unclear on what the Mapped TAI is, since TAI always corresponds to a geographic area. The Mapped Cell ID is used in NTN, but the WAB use case may be different to NTN. In NTN, a Uu cell ID corresponds to a very large geographic area and this geographical area can be dynamic when the satellite moves, thus requiring a Mapped Cell ID corresponds to a small fixed geographic area. In WAB, the TN cell (e.g. BH-gNB’s cell) corresponds to a small fixed geographic area, there is no need to introduce Mapped Cell.  In addition, Option 1 requires the WAB-gNB to know its geo-location, which is same as Option 2. Instead of providing the geo-location of WAB to CN (Option 2), Option 1 requires the WAB to map the geo-location to a Mapped Cell ID. So, Option 2 is simpler than Option 1.

For Option 3, it is similar to legacy IAB. This would necessitate that the LMF should be able to map the TAC/Cell ID of the cell serving MWAB-UE to a geolocation, which could be based on a configuration in the LMF.

Note that some companies raised a concern regarding option-2, indicating that a new format, i.e., geolocation, in the additional ULI is not only to be interpreted by AMF and LMF, but also other NFs such as SMF. Based on our understanding, even the additional ULI of MBSR is not provided to any other NFs, which is expected to be the same for WAB. Note that CT4 messages do not contain additional ULI, e.g., transmitted from AMF to SMF. Therefore, such concern is not viable.

So, it is preferred to adopt Option 2 or Option 3.

Proposal 5-1: For issue-2, RAN3 to reply to SA2 that Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

Multi-hop

For scenario a), to prevent multi-hop, a WAB-MT cannot connect with any cell of a WAB-gNB. In legacy IAB/mobile IAB, a gNB-CU may have some cells belonging to IAB-DU, and other cells belonging to stationary gNB-DU. In WAB, it does not happen that some cell(s) of a gNB-CU belonging to WAB, and other cell(s) belonging to non-WAB.

There are multiple options

- Option 1: a WAB-gNB broadcast a new indication in SIB. By this indication, WAB-MT does not connect with the WAB-gNB

This option is similar to legacy IAB. However, this Option requires AS changes, and should be discussed in RAN2. It further requires mechanisms to disallow handover of WAB-MT to a WAB-gNB.

- Option 2: When the WAB-gNB initiates the NG Setup procedure with AMF, it can indicate it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB (as in documented SA2 solution in the TR 23.700-06).

Note that SA2 conclusion indicates:

The MWAB-gNB indicates to the AMF(s) it is a MWAB when it establishes the N2 connection to the AMF(s).

NOTE 2: This requires RAN3 to define this indication.

Proposal 5-2: When the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB.

In order to avoid handover of the WAB-MT to a WAB-gNB, as SA2 had agreed, there will be specific slices for serving WAB-MT. For that reason, target gNB, i.e., WAB1-gNB can already see that those slices are there in the handover request for WAB2-MT, and reject the handover. This is similar to normal HO that target gNB rejects the PDU sessions associated to S-NSSAIs not supported by target NG-RAN. No enhancement is needed.

Another option would be to have such rejection from the AMF after path switch request is sent from WAB-gNB to AMF, but this means that the handover is already completed, so less preferred than the above slice-based option.

Proposal 5-3: Target WAB-gNB of a handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover to avoid multi-hop in Xn/NG handovers.

For scenario b), WAB-UE and WAB-gNB are co-located, the WAB-UE can know whether a cell belongs to the co-located WAB-gNB by implementation.  This can be similar to legacy IAB/mobile IAB that the IAB-MT does not access the cell of the co-located IAB-DU.

Proposal 5-4: WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation.

Proposal 5-5: For issue-1, RAN3 reply to SA2 with following:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

The draft reply LS can be found in Annex B – Text proposal for the reply-LS to SA2

# 7	Radio-resource coordination between access and backhaul links

As noted earlier, resource coordination between access and BH links are relevant only for in-band operation of WAB-MT and WAB-gNB. Out-of-band deployment inherently provides isolation between access and BH and both links can be operated independently without need for coordination.

Observation 6-1: WAB resource coordination is not needed for out-of-band deployments.

WAB in-band deployment corresponds to that of IAB and it is acknowledged that IAB capabilities can be the basis for WAB as well. Legacy IAB releases provide signalling options for variety of resource usage scenarios.

WAB, however, may have following specific assumptions that could differ from IAB. E.g. it could be desirable that the BH network would require no or limited enhancements implemented specifically for WAB support. This is different from IAB where the donor nodes must be IAB capable e,g, for routing (BAP) support. IAB donor is therefore always a specific implementation even based on an existing gNB. BH network for WAB, on the other hand, can consider IAB type features optional as the BH connection itself is a legacy PDU session without additional requirements for the radio interface.

For radio resource coordination, (IAB) Xn signalling supports the exchange of cell resource configurations (in gNB-DU Cell Resource Configuration) including the DL/UL/F configuration (TDD pattern) as well as H/S/NA configuration for the DU (access) resources. This signalling can be re-used also for WAB deployments when Xn is established between the BH-gNB and the WAB-gNB. The requirement is that the BH-gNB knows the co-location of WAB-MT and WAB-gNB, see the discussion below.

Observation 6-2: Xn signaling to exchange cell resource configurations, with DUF and HSNA configurations, is applicable for WAB deployments.

As a simple example for resource configurations, the BH-gNB could have a split of HARD and NA configuration for DL and UL resources while the WAB-gNB has the opposite HARD/NA configuration. Such configurations can come from OAM or they could be fixed for certain WAB deployment. The lower layer control (e.g. DCI for explicit indication of resource availability) as specified for IAB can be left up to the degree how much WAB specific features are implemented in the BH-gNB which also the WAB implementation supports. As there are no RAN1 involvement, no WAB related features can be specified. Lower layer control is also beyond RAN3 scope. Also F1 is out of the WI scope.

Proposal 6-1: Xn signalling to exchange cell resource configurations specified for IAB can be made applicable to WAB. F1 enhancements and lower layer control is out of the RAN3/WI scope.

Detection of co-location of WAB-MT and WAB-gNB

For BH-gNB serving WAB-MT, it is beneficial in some scenarios to detect the co-location of the WAB-MT and WAB-gNB. For example, the BH-gNB may use this information, e.g. for resource multiplexing handling. In IAB case, the Donor detects the co-location based on the IAB-MT’s BAP address received in the F1 SETUP REQUEST message. In WAB, the co-location detection may be performed via the Xn interface. For example, when the WAB-gNB initiates the Xn Setup with the gNB serving WAB-MT, the WAB-gNB may include an indication in the XN SETUP REQUEST message. The indication may be a UE ID (e.g. C-RNTI) received from the BH-gNB during the RRC establishment procedure..

### Note that RAN3 TR also includes the following:

### For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

Proposal 6-2: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the Xn Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XN SETUP REQUEST message or Xn SETUP RESPONSE message.

TP for TS 38.423 can be found in Annex A – Text proposal for TS 38.423

# 8	Conclusion

In this contribution we have analyzed WAB mobility from RAN3 perspective.

Proposal 1: Update Stage-2 to capture the WAB-node mobility without change of UE’s AMF.

The TP can be found in ([3])

Proposal 2-1: To support the mobility of WAB-node with the change of UE’s AMF, and the support for PCI change, the WAB-gNB need to have two logical cells active simultaneously.

Proposal 2-2: WAB-gNB shall use different Supported TA List for setting up NG connection with “New” AMF.

Proposal 3-1: BH-gNB can indicate to the WAB-gNB that specific BH resources cannot be maintained.

Proposal 3-2: BH-gNB is aware which slices/PDU sessions are associated with specific resources at the WAB-gNB and may use this information to determine whether a target BH-gNB is able to maintain BH resources for the WAB-gNB.

Proposal 4: No WAB specific enhancements are needed for PCI collision avoidance.

Proposal 5-1: For issue-2, RAN3 to reply to SA2 that Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

Proposal 5-2: When the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB.

Proposal 5-3: Target WAB-gNB of a handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover to avoid multi-hop in Xn/NG handovers.

Proposal 5-4: WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation.

Proposal 5-5: For issue-1, RAN3 reply to SA2 with following:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

The draft reply LS can be found in Annex B – Text proposal for the reply-LS to SA2

Proposal 6-1: Xn signalling to exchange cell resource configurations specified for IAB can be made applicable to WAB. F1 enhancements and lower layer control is out of the RAN3/WI scope.

Proposal 6-2: The BH-gNB detects the co-location of the WAB-gNB and WAB-MT during the Xn Setup procedure initiated by the WAB-gNB, via the WAB-MT’s ID (e.g. C-RNTI) received from the WAB-gNB in XN SETUP REQUEST message.

TP for TS 38.423 can be found in Annex A – Text proposal for TS 38.423

# References

- RP-242395, New WID on additional topological enhancements for NR

- TR 38.799 v2.0.0

- R3-245175 (TP for TS 38.401) Discussion on high level aspects for WAB

- R3-244019/S2-2407345, LS on questions regarding FS_VMR_Ph2

# Annex A – Text proposal for TS 38.423

### 8.4.1	Xn Setup

#### 8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE 1:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE 2:	Exchange of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

----- Unmodified part skipped -----

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the XN SETUP REQUEST message or the XN SETUP RESPONSE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

If the Identity of WAB-MT IE is included in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider the information therein for discovering the co-location of a WAB-gNB and a WAB-MT.

Interactions with other procedures:

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

----- Next Change -----

#### 9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| AMF Region Information | M |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the ng-eNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Identity of WAB-MT | O |  | x | Contains the identity of the WAB-MT assigned by the BH-gNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

#### 9.1.3.2	XN SETUP RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 | This IE is not used in this version of the specification. | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| AMF Region Information | O |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Identity of WAB-MT | O |  | x | Contains the identity of the WAB-MT assigned by the BH-gNB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

# Annex B – Text proposal for the reply-LS to SA2

3GPP TSG-RAN3 Meeting #125bis	R3-24xxxx

Title:	Reply LS on LS on questions regarding FS_VMR_Ph2

Response to:	R3-244019/S2-2407345: LS on questions regarding FS_VMR_Ph2

Release:	Release 19

Work Item:	FS_VMR_Ph2

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact person:	Steven Xu

Steven.1.xu@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 thanks SA2 for the LS in S2-2407345. RAN3 has analysed the issues and concluded following as the responses to SA2 questions.

SA2 Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell)  for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3 Answer to Issue-1:

a) SA2 agreed solution (as in documented SA2 solution in the TR 23.700-06) is feasible, i.e., when the WAB-gNB initiates the NG Setup procedure with AMF, it indicates it is a WAB-gNB. This enables the AMF to reject a further WAB-MT connect with the WAB-gNB.

To avoid multi-hops during handover of WAB-MT, target WAB-gNB of the handover sees slices deployed for WAB-MTs in the WAB-MT’s handover request, and rejects the handover which is similar to legacy HO that target gNB rejects the PDU session(s) associated to S-NSSAIs not supported by target gNB.

b) WAB-UE knows whether a cell belongs to the co-located WAB-gNB by implementation and does not connect to WAB-gNB belonging to same MWAB.

SA2 Issue-2:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3 Answer to Issue-2: Option 2 or Option 3 defined in clause 8.5 of TR 23.700-06 is preferred.

# 2	Actions

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	18th – 22nd November 		Orlando, US

RAN2#127	17th – 21st February    		Athens, Greece