3GPP TSG-RAN WG3 Meeting #125bis													R3-245177

Hefei, China, 14th - 18th October 2024

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300) Discussion on the support of MBS Broadcast Service

Document for:	Discussion and Decision

# 1	Introduction

Last RAN3 agreed Turn WA into agreement:

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

RAN3 also agreed the BL CR for TS 38.300. But there are different views on the text proposal. This contribution analyses the issues and proposes a way forward.

# 2	Discussion

Stage-2 TP

The agreed Stage-2 TP is

The Cell Identity used for MBS broadcast service may be a Mapped Cell ID.

Editor’s Note: the wording for above texts of using Mapped Cell ID for MBS may be further clarified.

The main argument is whether the cell ID can be a Uu cell ID, e.g. “should not exclude using Uu Cell Id for MBS broadcast service in NR NTN, e.g. for earth fixed cells in GEO system”

It is worthy to first check the definition for Mapped Cell ID in TS 38.300:

Mapped Cell ID: in NTN, it corresponds to a fixed geographical area.

Based on current definition, as long as a cell ID can be corresponded to a fixed geographical area, it is a Mapped Cell ID. It does not matter whether the same cell ID is also used over the Uu interface. So in GSO case, a specific cell ID value used by Uu interface (i.e. as Uu cell ID) corresponds to a fixed geographical area, so it is can be considered as a Mapped Cell ID.

Observation 1: In GSO case, a specific cell ID used by Uu interface can be considered as a Mapped Cell ID based on the definition of Mapped Cell ID.

So there is no issue to change the TP to align with other bullets for Mapped Cell ID. For example,

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

- 	The Cell Identity used for MBS Service Area.

There is Rel-17 contribution ([4]) proposed to reorganize the bullets for Mapped Cell ID, i.e. only list the cases using Uu cell ID. In case the Rel-17 CR ([4]) is agreed, then it can already cover the MBS in Rel-19, and the Stage-2 TP agreed in last meeting will be not needed.

Proposal 1-1: In case Rel-17 CR is not agreed. update the Stage-2 TP to align with other bullets for Mapped Cell ID.

TP for TS 38.300 can be found in Annex A – TP for TS 38.300

Proposal 1-2: In case Rel-17 CR is agreed. The stage-2 TP agreed in last meeting can be covered and not needed.

Whether need Option 2 and Option 3

The main objective is to support the intended service area that is smaller than the coverage of a Uu cell. RAN3 need to specify the necessary signaling between CN and gNB to support this objective. Last meeting discussed the following options, and agreed the WA to use Option 1:

- Option 1: Mapped cell ID/list in MBS service area

- Option 2: New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Option 3: Based on GNSS description

Option 2 and Option 3 fulfil the same requirements as Option 1, but Option 2 and Option 3 requires enhancements to various CN nodes and interfaces. Since they fulfil the same requirement as Option 1, there is no need to develop multiple solution to fulfil the same requirements.

Proposal 2: No need to adopt Option 2 or Option 3.

How to join/leave a MBS service

In current MBS system, the AMF transfers the BROADCAST SESSION SETUP REQUEST message, which contains the N2 SM information in the received Namf_MBSBroadcast_ContextCreate Request to all NG-RANs which support MBS in the MBS service area (Step 2). The AMF determines the target NG-RAN node(s) based on the MBS service area information received from MB-SMF and the information (gNB ID, supported TAI) received from the NG-RAN nodes during the NG Setup procedure or RAN Configuration Update procedure.

In TN, operator can deploy a new cell/gNB for an existing Tracking Area. Once the AMF know the gNB starts to serve a Tracking Area of the MBS Service Area, the AMF can initiate the Broadcast Session Setup procedure in the new gNB. Similar for removing a cell/gNB from serving a Tracking Area. The AMF can initiate the Broadcast Session Release procedure to a gNB when the gNB stops to serve a Tracking Area.

Similar procedure can be reused for NTN. When a gNB/satellites moves in (or move out) a geo-area, the gNB initiate informs AMF for its updated TAI, so the AMF can initiate Broadcast Session Setup procedure (or Broadcast Session Release procedure) to start (or stop) the broadcast session in the satellite’s coverage.

Observation 2: current MBS Broadcast Session Setup/Release procedure can be reused for NTN when a gNB/satellite move in (or move out).

Proposal 3: No enhancement to current MBS Broadcast Session Setup/Release procedure when a gNB/satellite move in (or move out).

# 5	Conclusion

In this contribution, we discussed how to support MBS broadcast service in NR NTN. Our proposals are:

Observation 1: In GSO case, a specific cell ID used by Uu interface can be considered as a Mapped Cell ID based on the definition of Mapped Cell ID.

Proposal 1-1: In case Rel-17 CR is not agreed. update the Stage-2 TP to align with other bullets for Mapped Cell ID.

TP for TS 38.300 can be found in Annex A – TP for TS 38.300

Proposal 1-2: In case Rel-17 CR is agreed. The stage-2 TP agreed in last meeting can be covered and not needed.

Proposal 2: No need to adopt Option 2 or Option 3.

Observation 2: current MBS Broadcast Session Setup/Release procedure can be reused for NTN when a gNB/satellite move in (or move out).

Proposal 3: No enhancement to current MBS Broadcast Session Setup/Release procedure when a gNB/satellite move in (or move out).

# References

- TS 38.300

- TS 38.413

- R3-243354, (TP for TS 38.300) Discussion on the support of MBS Broadcast Service (Nokia, Nokia Shanghai Bell)

- R3-245357, Correction of mapped cell support list

# Annex A – TP for TS 38.300 BL CR

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

- 	The Cell Identity used for MBS Service Area.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.