3GPP TSG-RAN WG3 #125bis                                                 R3-245183

Hefei, CN, 14th – 18th Oct, 2024

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 38.300 and 37.340) MRO for R18 mobility mechanisms

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

LTM:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

- 2.1 MRO for LTM

Stage2 description

In last RAN3 meeting, Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell failure type has been defined. Here we continue discussing the detection mechanism.

Following is the agreement achieved in last meeting:

Take the existing detection mechanism descriptions as baseline.

LTM failure case defined by RAN2 is similar to CHO failure type captured in R17. The main difference is for too early failure type. CHO can only perform RRC reestablishment to source cell while LTM may perform Recovery procedure to source cell because source cell may be included in candidate cell list in LTM. So, for too early failure type only the procedure of UE going back to source cell is different. Others are all the same.

Therefore, the updated detection mechanisms are as below:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/LTM recovery cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO/LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

Proposal 1: It is proposed for RAN3 to discuss the above LTM detection mechanism.

RACH-less and RACH-based access

First, fall back mechanism from RACH-less to RACH-based access is not supported currently.

UE is triggered to perform LTM via a MAC CE (LTM Cell Switch Command), not legacy handover command. And LTM is specified in subsection 9.2.3.5 in TS38.300, not the subsection 9.2.3.6.

More specifically, the following text describes UE behavior when receiving LTM Cell Switch Command captured in TS38.321.

- 5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

We can see that UE would perform RACH less LTM cell switch when Timing Advance is configured. Then UE performs RACH-less access until RACH failure occurs, i.e. t304 expires. There is not fallback mechanism in above procedure.

In last RAN2 meeting, the following agreement was agreed:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

Network can make RACH-based or RACH less optimization according to the access type in RLF report and there is no more work on specification to assist RACH optimization.

Proposal 2: Fall back mechanism from RACH-less to RACH-based access is not supported and RAN2 has agreed to include access type in RLF Report, so, there is no more work on specification to assist RACH optimization.

Split architecture

For split architecture, the following agreements have been achieved.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

As legacy handover, CU receives RLF report and performs MRO analysis. For LTM switch failure, it is source DU to perform optimization. CU sends RLF Report and failure type to source DU.

For RLF occurs shortly after successful LTM switch, CU may detect Too Early LTM cell switch or LTM cell switch to Wrong Cell failure type, and then sends RLF Report and the failure type to source DU.

For RLF occurs after successful LTM switch with the UE reported timer longer than the configured threshold, CU may detect Too Late LTM cell switch, CU sends RLF Report and Too late failure type to the last serving DU.

Proposal 3: It is proposed to send RLF Report and LTM cell switch failure type from CU to DU.

Other issues

The goal of LTM is to enable a serving cell change via L1/L2 signaling, in order to reduce the latency, overhead and interruption time. UE may perform frequent and repeated handover within candidate cell list. Is there Ping-Pong issue during LTM triggered frequent and repeated handover needs consideration.

According to the LTM scenario, frequent and repeated handover may be normal. Moreover, it is hard for RRC to record every PCell information in legacy UHI because L1L2 make trigger handover very fast and RRC cannot catch up with L1L2.

Proposal 4: It is proposed for RAN3 to discuss how to tell frequent LTM handover from ping-pong issue and whether/how to update UHI during LTM.

During the following LTM triggered handover, more than one times of failure or near failure maybe occur. Since the LTM triggered handover may be frequent and quick, network may be not able to fetch SHR or/and RLF Report before the next failure or near failure occurs which may cause overwriting issue. RAN3 may discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. LS may be sent to ask RAN2 to record multiple failures or near failures information in UE, if needed.

Proposal 5: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

- 2.2 MRO for CHO with candidate SCGs

In last RAN3 meeting, on failure case the following agreements were captured:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

For case 4/5/6, CHO only or CHO with SCG instead of CHO with candidate SCGs is configured to UE. If failure occurs, MRO for CHO and MRO for CHO with SCG would be performed. MRO for CHO has been supported in R17, but MRO for CHO with SCG is not supported now. So, for MRO for CHO with SCG, there is specification impact.

In our understanding, to support MRO for CHO with SCG, RLF Report may be enhanced to include CHO candidate cell and execution conditions which is similar like the solution on MRO for CHO. But considering MCG failure and/or SCG failure may occur during CHO with SCG execution, it is a little like CHO with candidate SCGs. How to handle two failures in one handover is not an easy task. So, we believe MRO for CHO with SCG may be discussed after MRO for CHO with candidate SCGs if time permitted. According to our understanding there may be no extra work to support MRO for CHO with SCG based on the solution on MRO for CHO with candidate SCGs.

Observation 1: MRO for CHO with SCG is not supported now, especially for both failure cases. But the specification impact may be same as MRO for CHO with candidate SCG

Proposal 6: It is proposed for RAN3 to check whether any additional specification impact in MRO for CHO with SCG after supporting MRO for CHO with candidate SCGs.

For case 1c, 2c, 3c, 7c, 8c, MCG failure + SCG failure occurs. In our understanding, it does not mean MCG failure and SCG failure occurs at the same time. Most of time it may be MCG failure occurs first and then SCG failure occurs, or SCG failure occurs first and then SCG failure occurs. For example, too late failure type, UE moves out of courage of PSCell and PCell gradually without triggering handover. SCG failure occurs first due to the courage of PSCell is smaller than that of PCell most of the time, then MCG failure occurs when UE moves out of courage of PCell. Therefore, MCG failure after SCG failure or SCG failure after MCG failure is a common case should be supported.

Proposal 7: It is proposed to support case 1c, 2c, 3c, 7c, 8c because MCG failure occurs after SCG failure or SCG failure occurs after MCG failure is a common case.

In case of both MCG and SCG failure during CHO with candidate SCGs, only one failure (MCG failure or SCG failure) could be used to detect failure type, i.e. MCG and SCG failure cannot be used at the same time because different failure type may be detected by MCG failure and SCG failure. If too early failure type for MCG and to wrong cell handover failure type for SCG is detected, we do not know how to optimize the configuration of CHO with candidate SCGs.

Proposal 8: It is proposed to use only one failure (MCG failure or SCG failure) to define MRO failure type for case 1c, 2c, 3c, 7c, 8c because detection on two failure type may lead to conflicting result.

On how to define failure type for MRO, the solution may be:

Solutin1: the first occurred failure may be selected to perform MRO.

Solution2: MCG failure would be selected because MCG connection is more important than SCG.

Solution 1 may have less specification impact than solution2 because it is hard for solution 2 to handle the case of SCG failure followed by MCG failure. UE and network do not know whether MCG failure may occurs after SCG failure. For the first occurred SCG failure, it is actually 1b, 2b, 3b, 7b, 8b case which should trigger MRO at once when network receiving SCGFailureInformation message. If RLF report indicating MCG failure is received by network later, RLF report has to be enhanced to correlate with the previous SCG failure information in order to withdraw the MRO triggered by SCG failure because solution 2 only perform optimization on MCG failure.

Proposal 9: It is proposed for RAN3 to discuss how to define the failure type for MRO for case 1c, 2c, 3c, 7c, 8c e.g., based on the first failure or always MCG failure.

For case 9, PCell and PSCell execution conditions cannot be met simultaneously, it is a common case should be supported.

Most of time, it may cause too late handover failure type and execution condition should be optimized to be more easily met in order to trigger handover earlier. But for the following PCell and PSCell coverage illustrated in figure1, it is useless to solve too late handover failure type by just optimizing execution condition, because for any execution conditions, both PCell and PSCell execution conditions could not be satisfied at the same time due to no common coverage for PCell and PSCell.

Figure 1: not suitable to configure CHO with candidate SCG scenario

For the scenario in figure 1, the UE should first perform handover from PCell 1 to PCell 2 and then perform PSCell change from PSCell A to PSCell B. So, it is needed to optimize whether configure CHO with candidate SCG or not rather than CHO with candidate SCG configuration itself.

Proposal 10: For case 9, PCell and PSCell execution conditions cannot be met simultaneously. The cause may be:

- execution conditions is sub-optimized;

- Coverage issue, i.e. no common coverage to meet both PCell and PSCell execution conditions.

According to above understanding, before deciding the detection mechanism on CHO with candidate SCGs failure type, the text on scenario validation may be captured first in stage2 description. The following text may be captured:

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

Proposal 11: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

In the figure 1 scenario, CHO with candidate SCG may not be triggered. So, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG should be used instead. If only configure CHO with candidate SCG, RLF may occur because CHO with candidate SCG would not be executed.

Therefore, since we may not able to aware about the scenario ahead of the time, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG may be configured as a backup together with CHO with candidate SCG in order to avoid RLF.

Proposal 12: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

For the SCG failure after successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B, due to PCell 2/PSCell B taking as last serving node, Pell 2 may receive failure report from UE or third node, i.e. SCGFailureInformation message. After MRO analysis, PCell 1/PSCell A may be informed to perform optimization. Xn interface message may be enhanced to provide SCGFailureInformation message for the same UE from PCell 2 to PCell 1/PSCell A.

For example，SCGFailureInformation message is received by MN (PCell 2) shortly after a successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B. If PCell1/PSCell A as the initiating node should be optimized, a handover report message（or other messages）should be sent from PCell2 to PCell1 which includes SCGFailureInformation message.

Proposal 13: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE between source MN and target MN.

For information in RLF Report, the agreement and FFS is listed below:

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

Since candidate cell list for CHO with candidate SCGs would be recorded in RLF Report, there is an example in which the first fulfilled execution condition is recorded based on candidate cell list in RLF report.

| Index in candidate cell list | Candidate PCell ID | Candidate PSCell ID | The first fulfilled execution condition |
| --- | --- | --- | --- |
| 1 | PCell 1 | PSCell A | CHO |
| 2 | PCell 1 | PSCell B | CPAC |
| 3 | PCell 1 | PSCell C | NONE |
| 4 | PCell 2 | PSCell A | CHO |
| 5 | PCell 2 | PSCell B | CPAC |
| 6 | PCell 3 | PSCell A | NONE |

Since we have agreed to record the first fulfilled execution condition as above, according to the relationship with candidate cell list, the candidate PCell ID and PSCell ID which met the configured CHO execution conditions can be known.

Proposal 14: According to the relationship between candidate cell list and the first fulfilled execution condition list, the candidate PCell ID and PSCell ID which met the configured CHO execution can be known.

Take the case of index 1 as an example in above table, i.e. CHO is first met. If CHO with candidate SCGs is not initiated at last, we only know CHO is met, and do not know whether CPAC is met or not. There may be two cases:

- CPAC is met, but not at the same time with CHO;

- CPAC is not met all the time.

For the first case, CPAC configuration should be optimized to trigger earlier in order to trigger at the same time with CHO e.g., case 9.

For the second case, CPAC configuration may be optimized to trigger more easily e.g., case 3.

Therefore, whether the second execution condition is met is also needed in RLF Report.

Proposal 15: Besides including the first fulfilled execution condition list in RLF report, the information on whether the second execution condition is met should also be recorded in the list.

- 2.3 MRO for Subsequent CPAC

In last RAN3 meeting, stage 2 description on S-CPAC problem definition has been agreed.

As for the Detection mechanism, CPC detection mechanism could also be reuse for S-CPAC without modification.

Proposal 16: It is proposed to reuse CPC detection mechanism for S-CPAC without modification.

For legacy MRO for CPAC, SCGfailureInformation message is used to indicate SCG failure information from UE to NG-RAN. As for subsequent CPAC, SCGfailureInformation message can be reused to indicate SCG failure during subsequent CPAC. Some necessary enhancement for SCGfailureInformation message may be needed.

Proposal 17: It is proposed to reuse SCGfailureInformation message to indicate SCG failure for MRO for subsequent CPAC.

Here we discuss the forwarding mechanism which is also based on CPAC. But there are some differences due to the following principles applied to subsequent CPAC captured in TS37.340.

-	For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC).

-	For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC.

-	For both MN and SN initiated inter-SN subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

In case of SN initiated PSCell change, the initiating node may be not the source SN and target SN for current PSCell change when receiving SCGfailureInformation message from UE. For example, SN1 as source SN initiates subsequent CPC, UE perform PSCell change from SN1 to SN2, and then PSCell change from SN2 to SN3. If SCG failure occurs in SN3, SN2 and SN3 as current source SN and target SN may be involved to perform MRO. SN1 as the initiating node which provide the candidate cell list may be also optimized.

Observation 2: For MRO for S-CPAC, the initiating node may be not source SN and target SN for the current PSCell change in case of SN initiated CPC.

For optimization on candidate cell selection, PSCell candidate list is first selected by initiating node, and then selected by target SN.

Proposal 18: The initiating node and target SN is responsible for optimize candidate cell selection.

For optimization on execution conditions, for the initial execution of S-CPAC, MN or SN as the initiating node generate the execution conditions.

For the following execution of subsequent S-CPAC, in case of no intra-SN PSCell change, candidate SN generates the execution conditions. Here the candidate SN refers to the source SN of PSCell change.

In case of intra-SN PSCell change, the source SN (or last serving SN) generates the execution conditions.

Proposal 19: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

The forwarding mechanism would include two steps:

Step 1: optimization on execution conditions optimization

MN receives SCGfailureInformation message and then perform initial analysis. If it is not MN to optimize execution conditions, SCGfailureInformation message should be provided to the source SN (or last serving SN) to optimize execution conditions.

Step 2: optimization on PSCell candidate list

SCGfailureInformation message should be provided to initiating node and target SN to perform MRO on PSCell candidate selection.

Therefore, the forwarding mechanism is updated in following TP to include above optimization.

Proposal 20: It is proposed for RAN3 to discuss the updated forwarding mechanism in TP for TS37.340.

Network could save UE context in MN which may include the initiating node information. But MN may only know the initiating SN node ID and may not know the PSCell ID in the initiating node which is optional present in S-NODE CHANGE REQUIRED message. For the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

As we discussed before, the initiating node and initiating PSCell may be required to perform MRO, therefore, SCGfailureInformation may be enhanced to include source PSCell information in initiating node.

Proposal 21: It is proposed to include source PSCell information in initiating node in SCGfailureInformation message.

There is an issue raised in RAN3 meeting as below:

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: it is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. Moreover, most of the times MRO is performed based on the statistics, not one time failure. Therefore, it is not needed to perform MRO in a hurry or each time Ping-Pong issue occurs. Comparing with solution 3, solution 4 also has no specification impact. So, solution 4 is preferred.

Proposal 22: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Conclusions

MRO for LTM:

Proposal 1: It is proposed for RAN3 to discuss the above LTM detection mechanism.

Proposal 2: Fall back mechanism from RACH-less to RACH-based access is not supported and RAN2 has agreed to include access type in RLF Report, so, there is no more work on specification to assist RACH optimization.

Proposal 3: It is proposed to send RLF Report and LTM cell switch failure type from CU to DU.

Proposal 4: It is proposed for RAN3 to discuss how to tell frequent LTM handover from ping-pong issue and whether/how to update UHI during LTM.

Proposal 5: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

MRO for CHO with candidate SCGs:

Observation 1: MRO for CHO with SCG is not supported now, especially for both failure cases. But the specification impact may be same as MRO for CHO with candidate SCG

Proposal 6: It is proposed for RAN3 to check whether any additional specification impact in MRO for CHO with SCG after supporting MRO for CHO with candidate SCGs.

Proposal 7: It is proposed to support case 1c, 2c, 3c, 7c, 8c because MCG failure occurs after SCG failure or SCG failure occurs after MCG failure is a common case.

Proposal 8: It is proposed to use only one failure (MCG failure or SCG failure) to define MRO failure type for case 1c, 2c, 3c, 7c, 8c because detection on two failure type may lead to conflicting result.

Proposal 9: It is proposed for RAN3 to discuss how to define the failure type for MRO for case 1c, 2c, 3c, 7c, 8c e.g., based on the first failure or always MCG failure.

Proposal 10: For case 9, PCell and PSCell execution conditions cannot be met simultaneously. The cause may be:

- execution conditions is sub-optimized;

- Coverage issue, i.e. no common coverage to meet both PCell and PSCell execution conditions.

Proposal 11: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

Proposal 12: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

Proposal 13: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE between source MN and target MN.

Proposal 14: According to the relationship between candidate cell list and the first fulfilled execution condition list, the candidate PCell ID and PSCell ID which met the configured CHO execution can be known.

Proposal 15: Besides including the first fulfilled execution condition list in RLF report, the information on whether the second execution condition is met should also be recorded in the list.

MRO for Subsequent CPAC:

Proposal 16: It is proposed to reuse CPC detection mechanism for S-CPAC without modification.

Proposal 17: It is proposed to reuse SCGfailureInformation message to indicate SCG failure for MRO for subsequent CPAC.

Observation 2: For MRO for S-CPAC, the initiating node may be not source SN and target SN for the current PSCell change in case of SN initiated CPC.

Proposal 18: The initiating node and target SN is responsible for optimize candidate cell selection.

Proposal 19: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

Proposal 20: It is proposed for RAN3 to discuss the updated forwarding mechanism in TP for TS37.340.

Proposal 21: It is proposed to include source PSCell information in initiating node in SCGfailureInformation message.

Proposal 22: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact

Reference

[1]

TP for TS 38.300

- 15.5.2.2.2	Connection failure due to intra-system mobility

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/LTM recovery cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO/LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 37.340

- 10.18.x	CHO with candidate SCG(s).

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

### 10.18.3	Conditional PSCell addition or change failure

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

For CPA or MN initiated CPC or MN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report messageto the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation. In case of following S-CPAC, the MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report messageto the SN which initiating S-CPAC or CPC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report messageto the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation. In case of S-CPAC, the MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>