3GPP TSG-RAN WG3 #125bis                                                 R3-245186

Hefei, CN, 14th – 18th Oct, 2024

Agenda Item:	10.4

Source:	CATT

Title:	(TP for 37.340 and 38.413) SON enhancement for R18 leftover

Document for:	other

Introduction

According to the agreements achieved in last meeting, here we provide some analysis on MRO enhancement for MR-DC SCG failure.

Discussion

  - MRO for MR-DC SCG failure

In last RAN3 meeting, the following agreements have been achieved:

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

In last RAN2 meeting, SCGFailureInformationNR message is enhanced as below:

To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

Then RAN3 decides to include source PSCell ID, Failed PSCell ID and timeSCGFailure in the new SCG FAILURE INFORMATION REPORT message to assist SN to perform PSCell change failure analysis when MN receiving SCGFailureInformationNR message from UE.

Here we continue discussing the forwarding mechanism in EN-DC.

First, MN could detect whether intra-SN PSCell change has been triggered in the last serving SN from SCGFailureInformationNR message. If previousPSCellId is one of the cells in source SN, there is no intra-SN PSCell occurs. On the contrary, if previousPSCellId is one of the cells in target SN, there is intra-SN PSCell occurs. So, the procedure on MN checking intra-SN PSCell change with SN is not necessary.

Proposal 1: It is proposed to not introduce the procedure on MN checking intra-SN PSCell change with SN.

For MN initiate PSCell change, MN selects target SN and then target SN selects PSCell. So, for PSCell change to wrong PSCell failure type, both MN and target SN may optimize target PCell selection. And MN generates execution condition, so, MN needs to optimize execution condition.

Observation 1: In case of MN initiate PScell change, both MN and target SN may need to optimize PSCell selection for PSCell change to wrong PSCell failure type. And MN may need to optimize execution condition.

For SN initiate PScell change, source SN selects target SN and then target SN selects PSCell. So, for PSCell change to wrong PSCell failure type, both source SN and target SN may optimize target PCell selection. And source SN generates execution condition, so, source SN needs to optimize execution condition.

Observation 2: In case of SN initiate PScell change, both source SN and target SN may need to optimize PSCell selection for PSCell change to wrong PSCell failure type. And source SN needs to optimize execution condition.

For EN-DC and NGEN-DC case, the SCGFailureInformationNR message is defined in TS36.331 as below:

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]]

}

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

The field measResultSCG is encoded in NR format hence LTE MN cannot decode it. Although measResultFreqListNR-r15 can be decoded by MN, it is so limited because it includes only SSB measurement result, i.e. cell measurement based on CSI-RS is not supported. In our understanding, SSB RRM measurement result is a little coarse for mobility decision and CSI-RS can provide more precise RRM measurement result which is more important for improving handover successful rate.

In general, the measResultFreqListNR-r15 is used to select/optimize target SN, while the measResultSCG with more precise measurement information is used to select/optimize target PSCell.

Observation 3: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message but it shall be used for the next suitable PSCell selection.

Therefore, we propose NR SN takes the responsibility to select the next suitable PSCell based on measResultSCG in SCGFailureInformationNR message.

Proposal 2: NR SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message for both MN initiate PSCell change and SN initiate PSCell change.

Proposal 3: It is proposed to include the next suitable PSCell in SCG FAILURE TRANSFER message.

There is another issue on whether NR SN should provide PSCell change failure type to MN in case of MN initiated PScell change. If not providing PSCell change failure type to MN, MN should detect PSCell change failure type based on MN MRO configuration, for example, Tstore_UE_cntxt and threshold for detecting coverage hole. Manual work is needed to keep MRO configuration aligned between MN and SN. Otherwise, MN and SN may detect different PSCell failure type. For example, for different Tstore_UE_cntxt configured in MN and SN, SN may detect too late PSCell change failure type (SCG RLF occurs not shortly after successful PSCell change) while MN may detect to wrong cell PSCell change failure type(SCG RLF occurs shortly after successful PSCell change).

So, we slightly prefer NR SN providing PSCell change failure type from MN to SN.

Proposal 4: It is proposed for RAN3 to discuss whether including PSCell change failure type in SCG FAILURE TRANSFER message.

The MN performs the initial analysis when SCGFailureInformationNR is received from the UE. In the first step, MN forwards the SCG Failure Information Report message to this last serving SN. Last serving SN select the next suitable PSCell and send to MN via SCG Failure Transfer. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN which could also select the next suitable PSCell by its own. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

Proposal 5: It is proposed for RAN3 to discuss the above forwarding mechanism which is capture in TP for 37.340.

  - RACH optimization for SDT

According the RAN2#126 meeting agreement for SDT as below, RSRP/data volume related information at the time when the UE evaluates if it should perform SDT will be logged in RA report for successful case. Obviously, if SDT based RA procedure is failed, the RSRP and data volume is better to be reported to network to see if the parameters associated to these two values are configured reasonable.

- RAN2#126 meeting Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

Proposal 6: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in failed SDT based RA procedure.

According to legacy procedure, the parameters including DL RSRP and pending UL data volume size are also used by CG-SDT procedure, in which the UE specific UL grant is configured for initial CG-SDT data transmission. So, it is better to make network realize whether CG-SDT is not configured for a history RA procedure, then the network will consider the impact to CG-SDT when adjusting the thresholds. For example, if the indication of CG-SDT not configured is included in RA report for a previous RA procedure but the CG-SDT is configured by network now, then the network will adjust the parameters more carefully to consider the impact to CG resource.

Proposal 7: Whether the UE has performed the CG-SDT selection prior to RA-SDT needs to be logged in RA report.

  - MHI/UHI enhancement for SCG deactivation/activation

RAN2#127: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

For how to collect the time spent information, there are following two options:

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state.

Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

RAN2 kindly asks RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

The network can only collect the UHI when UE is in RRC connected status. For the UE in RRC idle and inactive, only UE can do MHI collection. If we want to have the full picture of UHI for DC optimization, the UE history information report by UE is necessary.

Proposal 8: Network based solution (UHI) is not sufficient for network optimization, a UE based solution (MHI) is needed.

For network based solution, the Time in SCG activated state can be added in Last Visited PSCell Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Time in SCG activated state | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in activated state in the cell in 1/10 seconds. If the UE stays in activated state in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | YES | ignore |

Proposal 9: Add Time in SCG activated state IE in Last Visited PSCell Information.

Conclusions

Proposal 1: It is proposed to not introduce the procedure on MN checking intra-SN PSCell change with SN.

Observation 1: In case of MN initiate PScell change, both MN and target SN may need to optimize PSCell selection for PSCell change to wrong PSCell failure type. And MN may need to optimize execution condition.

Observation 2: In case of SN initiate PScell change, both source SN and target SN may need to optimize PSCell selection for PSCell change to wrong PSCell failure type. And source SN needs to optimize execution condition.

Observation 3: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message but it shall be used for the next suitable PSCell selection.

Proposal 2: NR SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message for both MN initiate PSCell change and SN initiate PSCell change.

Proposal 3: It is proposed to include the next suitable PSCell in SCG FAILURE TRANSFER message.

Proposal 4: It is proposed for RAN3 to discuss whether including PSCell change failure type in SCG FAILURE TRANSFER message.

Proposal 5: It is proposed for RAN3 to discuss the above forwarding mechanism which is capture in TP for 37.340.

Proposal 6: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in failed SDT based RA procedure.

Proposal 7: Whether the UE has performed the CG-SDT selection prior to RA-SDT needs to be logged in RA report.

Proposal 8: Network based solution (UHI) is not sufficient for network optimization, a UE based solution (MHI) is needed.

Proposal 9: Add Time in SCG activated state IE in Last Visited PSCell Information.

Reference

[1] R3-245009, LS on MHI enhancement solution for SCG deactivation/activation, CMCC.

TP for TS 37.340

- 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

For NR-DC, MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

For EN-DC and NGEN-DC, The MN performs the initial analysis when SCGFailureInformationNR is received from the UE. In the first step, MN forwards the SCG Failure Information Report message to this last serving SN. Last serving SN select the next suitable PSCell and send to MN via SCG Failure Transfer. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN which could also select the next suitable PSCell by its own. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

TP for TS 38.413

- 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Time in SCG activated state | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in activated state in the cell in 1/10 seconds. If the UE stays in activated state in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | YES | ignore |

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Unchanged part is omitted

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-TimeInSCG-ActivatedState,

maxnoofAllowedAreas,

Unchanged part is omitted

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TimeInSCG-ActivatedState					CRITICALITY ignore	EXTENSION TimeInSCG-ActivatedState		PRESENCE optional},

...

}

Unchanged part is omitted

-- T

TimeInSCG-ActivatedState ::= INTEGER (0..40950)

- 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

Unchanged part is omitted

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-TimeInSCG-ActivatedState								ProtocolIE-ID ::= xxx