3GPP TSG-RAN WG3 Meeting #124	R3-245196

Hefei, China, 14th – 18th Oct 2024

Title: 	Discussion on AI/ML for split architecture use cases

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussion and Decision

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

Measured Energy Cost can be transferred from DU to CU.

And in RAN3 #125 meeting,

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3 #124.

And in RP-242385, the WI description is agreed that:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

The detailed objectives of the WI are listed as follows:

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: Split architecture support for Rel-18 use cases

Note: Coordination with RAN2, SA5 when needed if any.

So in this proposal, we will focus on split architecture support for load balance, mobility enhancement and Energy Saving.

.2	Discussion

In R18, RAN3 focused first on non-split architecture use cases and procedures, and split RAN architecture are not pursued. And in R19, we will continue the discussion on split architecture use cases and procedures.

In R18, three use cases: Load Balance, Mobility Enhancement and Energy Saving are considered and discussed:

- For Load balance, it has been agreed to transfer Measured UE performance from CU-UP to CU-CP.

- For Energy Saving use case, it has been agreed that Measured Energy Cost can be transferred from DU to CU.

From the LS R3-245021, SA5 provides the following information:

SA5 would like to response to the questions with the following:

RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558:

- TS 28.558 does not define M4 measurement.

- There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases:

- The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558.

From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552.

From SA5 perspective, the misalignment between TS28.558 and TS 28.552 has been corrected in TS28.558v18.1.0. The missing M4 measurement has be added into TS28.558 as attached.

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

And for the R18 use cases: Load Balance, Mobility Enhancement, the following shall be discussed:

2.1 Load Balance

- For load balance, the UE performance feedback may be requested through Xn after HO, which includes Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss. The Throughput DL and Throughput UL are measured in gNB-DU, Packet Loss DL is measured in gNB-DU, but Packet Loss UL is measured in gNB-CU-UP, while Packet Delay involve measurements from both gNB-DU and gNB-CU-UP.

Proposal 1: Except Transferring Measured UE performance from CU-UP to CU-CP, it is also beneficial to transfer UE performance data from gNB-DU to gNB-CU through F1 interface.

2.2 Mobility Enhancement

- It has been agreed that the prediction function of UE trajectory is located in gNB-CU-CP. The prediction of UE trajectory may need to collect current UE traffic metric locally.

- The gNB needs predicted UE traffic (data volume) as input for AI/ML Training and Inference. To predict UE traffic, gNB-CU-CP needs to collect current UE traffic metric(data volume) from gNB-CU-UP and gNB-DU.

For mobility enhancement, UE traffic metric shall be reported to gNB-CU. It has been agreed to use data volume as the start point for UE traffic metric, the current DL/UL PDCP SDU Data Volume are measured in gNB-CU-UP. The cell-level UE trajectory prediction function is agreed to be located in gNB-CU-CP.

Proposal 2: It is beneficial to transfer the UE traffic (data volume) from gNB-CU-UP to gNB CU-CP through E1 interface. gNB-CU-CP shall use the UE traffic (data volume) as input for UE traffic prediction and UE trajectory prediction.

2.3 Predication from CU to DU

In R18 WI phase, it has been agreed that the following predication functions are located in gNB-CU:

- The cell-level UE trajectory prediction function is located in gNB CU-CP. Which will later be transferred to target CU after Handover, via Xn interface.

- The resource status prediction function is located in gNB CU-CP.

There is no discussion and no conclusion on whether to transfer the resource status prediction from source gNB-CU to gNB-DU, and how whether to transfer the UE trajectory prediction from target gNB-CU to target gNB-DU.

And in R19 SI phase, it has been agreed that:

Predicted CCO Issue (including predicted affected cells/beams) from CU to DU, in the following two scenarios:

- CU1 predicts the CCO issue for DU1 and sends to DU1

- Based on the received future CCO state (including an associated coverage modification cause) from CU1, CU2 will deduce for DU2 the predicted CCO issue and affected cells/beams, and send it to DU2.

So it is clear that prediction need to be transferred from gNB-CU to gNB-DU. When and how to transferred the prediction from gNB-CU to gNB-DU shall be discussed in WI phase.

Proposal 3: It is suggested to introduce procedures from gNB-CU to gNB-DU via F1 interface, to transfer the predictions.

Proposal 4: What kind of prediction shall be transferred from gNB-CU to gNB-DU needs further discussed.

We think when the cell-level UE trajectory prediction is transferred from source CU to target CU after Handover, the target CU shall let target DU get the information, to assistant the radio resource allocation and reservation in corresponding cell.

And for the resource status prediction, it always helps the DU to reserve corresponding resource. So it is beneficial to transfer it from source CU to DU.

Proposal 5: It is proposed to transfer the cell-level UE trajectory prediction from target gNB-CU to target gNB-DU, and transfer the resource status prediction from source gNB-CU to gNB-DU.

2.4 Procedure Design over E1 and F1 interfaces

In RAN3#121-bis, the WA has been achieved as following:

WA: Defining new procedures to transfer the measured EC over F1 follows the design over Xn.

From above discussion, we can see not only Energy Cost, but also the other information needs to be transferred from gNB-DU, gNB-CU-UP to gNB-CU-CP. Currently, only resource status can be transferred in existing Resource Status Request Initiation and Reporting procedures over E1 and F1 interfaces.

The procedure over F1 and E1 for AI/ML data collection is very similar as the procedures over Xn, also the data collected over F1 and E1 is related to the ones over Xn. So it is straightforward to introduce new procedures over F1 and E1 for Data Collection, and follow the current design over Xn.

To follow the design over Xn interface, the new introduced procedures over E1 and F1 interfaces shall include the Request, Response/Failure and Update messages. Different measurement shall be supported using different code points in the report characteristics request, but they can share the same Request and Update message. Both one-time and periodic collections shall be supported.

Proposal 6: It is proposed to turn the WA into agreement, to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB-CU and gNB-DU over F1 interface, to transfer UE performance, energy consumption data. Similar as the procedures designed over Xn interfaces.

Proposal 7: It is proposed to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB-CU-UP and gNB-CU-CP over E1 interface, to transfer UE performance and UE traffic (data volume). Similar as the procedures designed over Xn interfaces.

For the transfer of prediction from gNB-CU to gNB-DU, currently no legacy procedures in F1 interface can be used. And this procedure is totally different from the DATA COLLECTION procedures. So a new procedure need to be discussed and introduced in F1 interface to transfer predictions.

Proposal 8: It is proposed to introduce a new procedure to transfer the predication information from gNB-CU to gNB-DU, through F1 interface.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN split architecture use cases, following observations and proposals are made:

Observcation1: It is clear from this LS reply that Average delay DL needs to involve measurement collected from gNB-DU.

Proposal 1: Except Transferring Measured UE performance from CU-UP to CU-CP, it is also beneficial to transfer UE performance data from gNB-DU to gNB-CU through F1 interface.

Proposal 2: It is beneficial to transfer the UE traffic (data volume) from gNB-CU-UP to gNB CU-CP through E1 interface. gNB-CU-CP shall use the UE traffic (data volume) as input for UE traffic prediction and UE trajectory prediction.

Proposal 3: It is suggested to introduce procedures from gNB-CU to gNB-DU via F1 interface, to transfer the predictions.

Proposal 4: What kind of prediction shall be transferred from gNB-CU to gNB-DU needs further discussed.

Proposal 5: It is proposed to transfer the cell-level UE trajectory prediction from target gNB-CU to target gNB-DU, and transfer the resource status prediction from source gNB-CU to gNB-DU.

Proposal 6: It is proposed to turn the WA into agreement, to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB-CU and gNB-DU over F1 interface, to transfer UE performance, energy consumption data. Similar as the procedures designed over Xn interfaces.

Proposal 7: It is proposed to introduce a new Data Collection Reporting Initiation and Reporting procedures between gNB-CU-UP and gNB-CU-CP over E1 interface, to transfer UE performance and UE traffic (data volume). Similar as the procedures designed over Xn interfaces.

Proposal 8: It is proposed to introduce a new procedure to transfer the predication information from gNB-CU to gNB-DU, through F1 interface.

4. Reference

- 3GPP RAN3#125 Chairman’s notes (RAN3_125_agenda_20240823EOM.doc)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- DRAFT R3-244848 TR38.743 of AIML for NG-RAN_v1 (Release 19)

- R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics (Ericsson), 3GPP RAN3#125-bis