3GPP TSG-RAN WG3 Meeting #125-bis	R3- 245197

Hefei, China, 14th – 18th Oct 2024

Title: 	Discussion on continuous MDT

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussions & Approval

- Introduction

In R18, the following consensus are gotten on MDT enhancement for AI/ML:

Potential MDT enhancement related issues as follows, need more time to discuss the details and potential standard impacts, coordination with RAN2/SA5 if needed:

- enhance the mdt procedure to solve the issue how to support the consecutive ai/ml data collection for the certain time-series ai/ml model.

- how the source ng-ran node obtains logged ue trajectory information when ue enters rrc connected state and reports to the new ng-ran node.

- how to enable a more granular selection of ues based on enhanced mdt configuration information in management based MDT

- how to map ai/ml feedback information to ai/ml actions and report them over MDT

Study the scenarios, issues and solutions to support the continuous data collection within a period for AI/ML via MDT.

WA: There are the benefits in enabling UEs to continuously collect MDT measurement across RRC states and to provide to the network such continuous time series of data for the purpose of AI/ML Training/Retraining. The evaluation of gains vs impacts related to the use cases in R18 coming from potential solutions needs to be further discussed.

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

It is proposed that solutions to address the above shortcomings are not pursued in Rel.18.

In #125, it is agreed as common sense that: Keep the current use case description for Continuous MDT and add "Potential solutions and standard impacts are left for further discussion during the normative phase.

In study phase, TR 38.743 was finalized to capture the following use cases description on Continuous MDT:

5.4	Continuous MDT collection targeting the same UE across RRC states

5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

5.4.2	Potential Standard impacts

Potential solutions addressing the problems above can be discussed during normative phase.

And in RP-242385, the WI description is agreed that:

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

The detailed objectives of the WI are listed as follows:

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]: Continuous MDT collection targeting the same UE across RRC states

Note: Coordination with RAN2, SA5 when needed if any.

So in this proposal, how to resolve the two problems to enhance Continuous MDT will be discussed.

.2	Discussion

In R18, it has been agreed to use the existing MDT framework as a baseline for data collection from UE. But enhance it to support the continuous collection of MDT trace for AI/ML training in OAM.

The continuous MDT collection from UE can be categorized into two parts: management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Currently in R18, when NG-RAN node can provision logged MDT or immediate MDT to UE using different trace sessions. And when UE transit to RRC_Idle or RRC_Inactive state, it can start to collect logged MDT, and when it reconnects to the gNB, it will report the logged MDT to the new serving NG-RAN node.

However, the trace session ID, and UE ID are different for the immediate MDT record and the logged MDT record. TCE can’t associated them together as continuous MDT for a given UE.

Another problem is when UE connects to a new NG-RAN node, the new NG-RAN node can’t know it should provision immediate MDT for this UE, so the MDT collection can’t be continuous.

So these two problems needed to be addressed to guarantee the continuous MDT collection and association for a given UE:

- How to enable immediate MDT after UE transit from RRC_Idle or RRC_Inactive to RRC_Connected state.

- How to correlate different MDT records for a specific UE in TCE.

Proposal 1: It is proposed to discuss two issues in WI phase: 1. How to enable the continuous collection of MDT data for the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive); 2. How to correlate different MDT reports for a given UE in TCE.

After some discussion in previous meetings, there are different solutions to address these problems. RAN3 shall work out a solution which can minimize the impact to current procedures for UE and network.

  - Problem A (measurement continuity)

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

There are mainly two kinds of solution on this issue: 1. Store the “continuous MDT” information in UE; 2. Store the “continuous MDT” information in 5GC.

Some companies think the enhancement solution should be UE agnostic. However, MDT is only supported in about 10% UE, and if the solution is only adding an indicator or job type in the MDT provisioning information, it won’t change the UE mechanism. So we think the UE minor impact solution is acceptable.

Meanwhile, for the solution that stores the “continuous MDT” flag or job type or session ID in UE context within 5GC, the NG-RAN nodes may still not get the information for the inter-AMF switch over or cross AMF scenario. Also for the 5GC solution, both 5GC and NG-RAN procedures need to be updated.

It is proposed to introduce a new indicator, and follow the procedures below to enable Continuous MDT after UE transits the RRC state:

- If OAM want to collect “Continuous MDT” from a UE, it shall provision both Management-based logged MDT and Immediate MDT tracing sessions, with a “Continuous MDT” tag.

- When NG-RAN received the signals from OAM, it shall configure the logged MDT into UE correspondingly, with “Continous MDT” tag.

- When UE is in RRC_Connected state, gNB actives the immediate MDT tracing on UE, and then UE report to NG-RAN node1.

- NG-RAN node1 reports the immediate MDT in tracing session1 to TCE.

- Then when UE transits from RRC-Connected to RRC_Idle state, as NG-RAN node1 has already configured logged MDT tracing session2 in UE, the UE shall begin to collect the logged MDT in tracing session2. Here the logged MDT shall carry the “Continuous MDT” tag which was configured by NG-RAN node1 in the record.

- When UE transits again to RRC_Connected state, after it sets up RRC with NG-RAN node2, it shall report the logged MDT record to NG-RAN node2, carrying the “Continuous MDT” tag.

- When NG-RAN node2 receive the logged MDT report, it is notified by the “Continuous MDT” tag, so NG-RAN node2 configure both Management-based logged MDT and Immediate MDT tracing sessions, with a “Continuous MDT” tag, into the UE. The new serving NG-RAN node shall configure new area scope for the MDT.

- And UE can start immediate MDT collection now to make the MDT continuous.

In the above solution, provisioning the “Continuous MDT” tag to UE, and carrying the tag in logged MDT report, it can guarantee later MDT collections are continuous. It may still a little gap between the time when UE report logged MDT and the time when UE enables immediate MDT after NG-RAN node2 configure it. But we think this interval is ignorable for the data collected for AI/ML.

And after UE transit from RRC_Idle to RRC_Connected, MDT context may be deleted, the area scope for this MDT may also change. So it is important for the new serving NG-RAN node to re-configure the MDT into UE, with updated area scope.

This solution won’t change the signaling procedures between NG-RAN, 5GC and UE, only introduces a new indicator in MDT configuration procedures. We think this solution can minimize the impacts to network and UE.

Proposal 2: It is proposed to add the “Continuous MDT” requirement information in the MDT configuration. NG-RAN node shall provision this information in UE to let UE aware of the Continuous MDT requirement.

Proposal 3: When UE transits from RRC_Connected to RRC_Idle or RRC_Inactive state, it can automatically begin to collect logged MDT measurements according to the logged MDT configuration, under the indication of “Continuous MDT” information.

Proposal 4: When UE transits from RRC_Idle to RRC_Connected state, it shall report the logged MDT report to NG-RAN node, with the “Continuous MDT” tag in the report.

Proposal 5: When NG-RAN node receives the logged MDT report with “Continuous MDT” tag, it shall provision both immediate and logged MDT trace sessions to the UE again, to guarantee the MDT measurement can be collected continuously.

  - Problem B (trace correlation)

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

According to the LS reply from SA5 in R3-245021, it is said that: SA5 has agreed to add a clarification in TS28.558 that the measurement bullet g) is only applicable for management-based activation of 5GC UE level measurement collection, and the bullet g) in NG-RAN UE level measurements are set to “N/A” instead of “S-TMSI”, please see the agreed CRs attached.

From our understanding, it means the MDT session activated from NG-RAN has no Identifier of “5G-S-TMSI”. So we can’t use “5G-S-TMSI” to correlate all the gNB activated MDT records for a given UE.

Currently, none of the UE identifiers (such as IMEI-TAC, NR-CGI) can keep unchanged after gNB switch over or turn from RRC_Connect to RRC_Idle and then active again. And the Trace session ID (TR/TRSR) has to be global unique  for different MDT trace sessions even for the same UE.

To avoid the impact to 5GC and UE, we think to let OAM configure the same tag for the different MDT traces for a given UE. When OAM configure the Continuous MDT for a UE, it will configure both immediate MDT and logged MDT with the same Continuous MDT tag. The tag can be Octets, and unique for the UE in the OAM area. The tag shall only be provisioned for management-based Continuous MDT.

Then when gNB active the immediate MDT trace session, the “Continuous MDT tag” will be configured and carried in the MDT report. When the UE Handover to a new gNB, this  “Continuous MDT tag” shall also be transferred as UE context to the new gNB. So when the new gNB actives immediate MDT for the same UE again, the  “Continuous MDT tag” remains the same.

When gNB configure the UE for logged MDT, the “Continuous MDT tag” will be stored in UE. And when UE sends gNB back this logged MDT report, the tag will be carried in this report.

At last, TCE will use this tag to correlate all the MDT reports with the same tag, as the Continuous MDT for a given UE.

Proposal 6: It is proposed to introduce “Continuous MDT tag” from OAM into gNB. And carry the information in MDT configuration when gNB activate the MDT session.

Proposal 7：For management based continuous MDT, it is proposed to use “Continuous MDT tag” as the unique identifier in both the immediate MDT an logged MDT reports to associated them for a given UE in TCE.

Proposal 8：When UE handover from one gNB to another, the “Continuous MDT tag” will be transferred and keep unchanged as UE context.

3	Summary

In this contribution, we provide additional input considerations on the remaining issues on AI/ML for NG-RAN Continuous MDT enhancement, following proposals are made:

Proposal 1: It is proposed to discuss two issues in WI phase: 1. How to enable the continuous collection of MDT data for the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive); 2. How to correlate different MDT reports for a given UE in TCE.

Proposal 2: It is proposed to add the “Continuous MDT” requirement information in the MDT configuration. NG-RAN node shall provision this information in UE to let UE aware of the Continuous MDT requirement.

Proposal 3: When UE transits from RRC_Connected to RRC_Idle or RRC_Inactive state, it can automatically begin to collect logged MDT measurements according to the logged MDT configuration, with “Continuous MDT” information.

Proposal 4: When UE transits from RRC_Idle to RRC_Connected state, it shall report the logged MDT report to NG-RAN node, with the “Continuous MDT” tag in the report.

Proposal 5: When NG-RAN node receives the logged MDT report with “Continuous MDT” tag, it shall provision both immediate and logged MDT trace sessions to the UE again, to guarantee the MDT measurement can be collected continuously.

Proposal 6: It is proposed to introduce “Continuous MDT tag” from OAM into gNB. And carry the information in MDT configuration when gNB activate the MDT session.

Proposal 7：For management based continuous MDT, it is proposed to use “Continuous MDT tag” as the unique identifier in both the immediate MDT an logged MDT reports to associated them for a given UE in TCE.

Proposal 8：When UE handover from one gNB to another, the “Continuous MDT tag” will be transferred and keep unchanged as UE context.

4. Reference

- 3GPP RAN3#125 Chairman’s notes (RAN3_125_agenda_20240823EOM.doc)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- DRAFT R3-244848 TR38.743 of AIML for NG-RAN_v1 (Release 19)