3GPP TSG-RAN WG3 Meeting #125-bis	R3-245202

Hefei, CN, 14 - 18 Oct, 2024

Title:	Discussion on Inter-CU LTM procedure

Agenda Item:	13.2

Source:	Huawei

Document for:	Discussion

1	Introduction

In previous RAN3 meetings, RAN3 discussed Rel-19 inter-CU LTM and made some agreements.

RAN3 #123bis: Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution RAN3 #124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. RAN3 #125: Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure

In this contribution, we continue to discuss the basic procedures of inter-CU LTM, including the standalone case and the NR-DC case.

2	Discussion

2.1	Inter-CU LTM in standalone case

2.1.1 LTM Preparation

General

In RAN3 #123bis and RAN3 #124 meetings, RAN3 agreed to re-use existing Xn Handover Request and Handover Request ACK for inter-CU LTM preparation, and it is the source gNB-CU that initiates the inter-CU LTM preparation procedure.

RAN3 #123bis Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. RAN3 #124 Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

The existing Xn Handover Request procedure is initiated per candidate cell, instead of per UE. In RAN3 #125 meeting, RAN3 discussed whether the Xn Handover Request for LTM candidate cell preparation is for one single candidate cell or for multiple candidate cells, but there was no consensus.

One main obstacle to introduce a list of candidate cell in Xn handover request is the huge impact on the current specification. For example, in order to support different admission control results for different candidate cells, the Handover Request Acknowledgment message needs to be enhanced to carry a list of PDU Session Resources Admitted List. This issue was already acknowledged during the discussion for intra-CU LTM in Rel-18.

Therefore, we propose,

Proposal 1: RAN3 confirms that the handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

Stage 3 design

a) L1 measurement and report configuration

In Rel-18 intra-CU LTM, L1 measurement and report is based on SSB. The CU utilizes the SSB configuration obtained in F1 SETUP to produce the CSI resource configuration and send it to the source DU to generate the CSI report configuration.

In Rel-19 inter-CU LTM, to support SSB based L1 measurement and reporting, similarly, the source gNB is able to obtain the SSB configuration of candidate cells from neighbour gNBs via Xn setup. It can then utilize the obtained SSB configuration to produce the CSI resource configuration and send it to the source gNB-DU to generate the CSI report configuration. The processing on F1 can re-use the UE Context Modification Request procedure.

Proposal 2: The source gNB-CU utilizes the candidate cells’ SSB configuration obtained from neighbour gNBs during Xn setup to produce the CSI resource configuration.

In case of subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, so the other candidate gNB-CUs can request the corresponding candidate gNB-DU to generate the CSI report configurations. Then the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Proposal 3: To support subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The SSB configuration exchanged in Xn Setup does not include the SSB transmit power information, which is needed in the UE LTM configuration. Therefore, in Rel-18, the SSB Time/Frequency Configuration is included in the UE Context Setup Response message from the candidate DU to the CU.

In Rel-19, similarly, it’s better to let the candidate gNB-CU to respond the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 4: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

b) Early RACH configuration

In order to support PDCCH ordered early RACH, candidate DU first needs to allocate early RACH resources in the preparation step. To avoid RACH resource conflict, the early RACH resources of one candidate cell should be different for different source gNB-DUs. This is the reason why RAN3 introduced the source gNB-DUs List in Early UL Sync Configuration in Rel-18. The gNB-DUs List ensures that the candidate gNB-DUs can allocate non-overlapping RACH resources for different source gNB-DUs.

In Rel-19 inter-CU LTM, two gNB-DUs of difference gNB-CUs may have the same gNB-DU ID. Therefore, to avoid early RACH resource conflict, the pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then the candidate gNB-DUs during handover preparation.

Similar with L1 measurement and report configuration, the early RACH configuration can be allocated by Handover Request and Handover Request ACK procedure.

Proposal 5: The source gNB-CU sends the pair of (gNB ID, gNB-DU ID) to the candidate gNB-CU via the Handover Request message.

Proposal 6: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

c) Reference configuration

RAN2 agreed to support one single reference configuration in Rel-19 inter-CU LTM, as shown below:

RAN2 #127 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Basically, it means that the source gNB-CU can generate a reference configuration and sends the reference configuration to all candidate gNBs in Handover Request message.

Proposal 7: The source gNB-CU sends the reference configuration to all candidate gNBs in Handover Request message.

2.1.2 Early Synchronization

In Rel-19 inter-CU LTM, in PDCCH ordered early RACH procedure, in order to transfer the TA information between the candidate gNB and the source gNB, RAN3 agreed to introduce a new procedure on Xn.

RAN3 #124 Introduce a new procedure on Xn to transfer the TA information.

In RAN3 #125 meeting, there was no consensus on whether the TA information transfer message is UE associated message or non-UE associated message.

According to the RAN2 progress on PDCCH ordered early RACH, there is no intention in RAN2 to further enhance the Rel-18 mechanism. In this case, when a candidate cell receives a preamble for early TA acquisition, it does not know the UE identity because the early RACH resources are shared by multiple UEs in the source cell. Therefore, the TA information transfer procedure should be a new non-UE associated class 2 procedure also on Xn interface.

Proposal 8: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

After the candidate gNB-DU receives a preamble for early TA acquisition, it transfers the TA value to the source gNB-DU. In Rel-18, on F1 interface, messages (e.g. DU-CU/CU-DU TA Information Transfer) were introduced to transfer the TA information from candidate gNB-DU to source gNB-DU. They can be re-used between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 9: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, to let the candidate gNB-CU know where to forward the TA information, a source gNB ID is need in the corresponding DU-CU TA Information Transfer message.

Proposal 10: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

2.1.3 Candidate gNB-initiated LTM Cancel

In RAN3 #125 meeting, RAN3 had the following agreement to support candidate gNB-initiated LTM cancellation:

RAN3 #125 Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure

We think that option 1 is not suitable, since the message name includes “CHO”, which may cause confusion when it is used for LTM. We propose to adopt option 3, to introduce a new procedure for LTM cancel.

Proposal 11: To introduce a new class 2 procedure for candidate gNB-initiated LTM cancellation.

One additional issue we need to study for subsequent LTM is: the serving gNB of the UE will change, how the candidate gNB knows which candidate gNB is the current serving gNB and then it can send the “cancel” message to UE’s current serving gNB.

The straightforward thinking is that the candidate gNB always contacts the initial source gNB, then the initial source gNB forwards the information of candidate cell to be cancelled to the current serving gNB of the UE. This makes the initial source gNB behave like an anchor gNB of the UE and requires the initial source gNB should be always informed by the new serving gNB after each LTM cell switch.

Another option could be that after each LTM cell switch, the target gNB (the new serving gNB) notifies all candidate gNBs about the serving gNB change. E.g., the target gNB sending a Xn Handover Success message to all candidate gNBs. In this procedure, the UE associated XnAP connections between this new serving gNB and other candidate gNBs are established, so when a candidate gNB decides to cancel candidate cells, it can directly contact with this new serving gNB.

Proposal 12: RAN3 to discuss how to support candidate gNB initiated LTM cancel with the above two options

2.1.4 LTM configuration sync up among candidate gNBs

In addition, to support subsequent inter-CU LTM, some LTM configurations, e.g., early sync configuration, configuration ID, and data forwarding addresses, etc may need to sync up among candidate gNBs.

Taking the TCI states as an example, after the source gNB-CU collects the TCI state configuration of candidate cells in Handover Request Ack message, it should send the TCI state configuration to other candidate gNB-CUs.

In general, there are three options to sync up the TCI state configuration among candidate gNB-CUs. Option 1 is to sync up just after the initial handover preparation. Option 2 is that the source gNB sends the configuration to the target gNB once the LTM is initiated. Option 3 is when receiving the successful access notification from the target gNB. The three options are illustrated in Figure 1.

Figure 1: TCI state configuration transfer for subsequent inter-CU LTM

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Basically, the three options above can be mapped to the three cases of NOTE 2.

In RAN3 #125 meeting, RAN3 agreed to introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM (LTM CONFIGURATION UPDATE). This new procedure can be used for this purpose in above three cases.

Proposal 13: To reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs in case of subsequent LTM. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

2.1.5 UE XnAP ID allocation

During LTM, the LTM Cell Switch Notification from the source gNB-CU to the target gNB-CU can be successfully transmitted for the first time cell switch, because the source gNB-CU and the target gNB-CU already have the NG-RAN node UE XnAP ID of each other.

However, in subsequent inter-CU LTM, it is unclear how the new source gNB sends the LTM Cell Switch Notification to the target gNB and how the target gNB identify the correct UE, as there is no any Xn signalling connection setup procedure between these two gNBs beforehand.

One possible solution is that each candidate gNB allocates UE XnAP ID and send to other candidate gNBs in the preparation step.

Another solution is that after each cell switch, the new serving gNB allocates the source node UE XnAP ID and send it to other candidate gNB-CUs, and then the candidate gNB-CUs reply with the target node UE XnAP ID. This UE XnAP ID negotiation can be done for example, during the handover success notification procedure.

Proposal 14: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

2.2	Inter-CU LTM in NR-DC

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

RAN3 #123bis: Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. RAN2 #126 An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In order to make sure that the inter-CU LTM is not configured at the same time in MN and SN, inter-node interaction is needed. When the MN decides to configure inter-MN LTM candidates, the MN should send a notification to the SN to disable inter-SN LTM. When the SN decides to configure inter-SN LTM candidates, the SN should send a request to the MN. The MN may either agree or reject the request from the SN.

Proposal 15: When the MN decides to initiate inter-CU MCG LTM for a UE, it sends a notification to the SN.

Proposal 16: When the SN decides to initiate inter-CU SCG LTM for a UE, it sends a request to the MN, which may be accepted or rejected.

3	Conclusion

This document proposes:

LTM configuration:

Stage 2:

Proposal 1: RAN3 confirms that the handover preparation procedure is per candidate cell for inter-gNB-CU LTM.

Stage 3:

Proposal 2: The source gNB-CU utilizes the candidate cells’ SSB configuration obtained from neighbour gNBs during Xn setup to produce the CSI resource configuration.

Proposal 3: To support subsequent LTM, the source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Proposal 4: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 5: The source gNB-CU sends the pair of (gNB ID, gNB-DU ID) to the candidate gNB-CU via the Handover Request message.

Proposal 6: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

Proposal 7: The source gNB-CU sends the reference configuration to all candidate gNBs in Handover Request message.

Early Synchronization:

Proposal 8: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 9: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 10: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

Candidate gNB-initiated LTM Cancel:

Proposal 11: To introduce a new class 2 procedure for candidate gNB-initiated LTM cancellation.

Proposal 12: RAN3 to discuss how to support candidate gNB initiated LTM cancel with the above two options

LTM configuration sync up:

Proposal 13: To reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs in case of subsequent LTM. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

UE XnAP ID allocation:

Proposal 14: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

Inter-CU LTM in NR-DC:

Proposal 15: When the MN decides to initiate inter-CU MCG LTM for a UE, it sends a notification to the SN.

Proposal 16: When the SN decides to initiate inter-CU SCG LTM for a UE, it sends a request to the MN, which may be accepted or rejected.

The stage 2 and stage 3 TPs are provided in [1] and [2].

4	Reference

- R3-245203	(TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure

- R3-245204	(TP for LTM BLCR for TS 38.473): F1 impact for Inter-CU LTM