3GPP TSG-RAN WG3 Meeting #125bis	R3-245216

Hefei, China, 14 - 18 October, 2024

Agenda item:	19.2

Source: 	Huawei

Title: 	Discussion on gNB-gNB CLI mitigation in SBFD scenario

Document for:	Discussion

1. Introduction

There was an LS from RAN1 about CLI mitigation, the content is listed as follow:

The WID objective for CLI handling related to information exchange among gNBs is as follows: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - … - Note: Without dedicated optimization for dynamic/flexible TDD. - … - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact While the WID objective lists only RAN3 as responsible working group, RAN1 has discussed the sub-bullets of the objective and would like to clarify several PHY/L1-related aspects that are not typically discussed in RAN3. RAN1 would like to inform RAN3 (RAN2 is cc’d) of the following RAN1 agreement. Agreement Send an LS to RAN3 with the following information (cc: RAN2) From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the confi guration is provided to a gNB. Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB. Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

In this contribution, we will discuss the RAN3 impacts to support CLI mitigation, including SBFD configuration exchange, measurement procedure, and CLI mitigation procedure among gNBs.

2. Discussion

2.1	General discussion

First of all, we want to clarify the scenario of gNB-gNB CLI in case of cell configured with SBFD resource. As the following figures show, in case of the gNB-gNB co-channel CLI, one gNB sends downlink signal would cause interference towards other gNBs that are receiving signals.

According to the WID [1] and LS from RAN1[2], RAN3 shall start to work on the support of CLI mitigation in case of sub-band full duplex scenario. For enhancements for CLI handling, the following aspects shall be considered:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Information exchange of strongest DL beam information

- Information exchange of CLI-mitigation request

To exchange the CLI related information between gNBs, the XnAP specification needs to be enhanced, the gNBs could exchange the SBFD configuration and measurement information directly. In addition, for the case of dis-aggregated gNB, the CLI related information should also be exchanged over F1 interface, considering that the gNB-DU is responsible for radio resource management, the gNB-DU of peer node is the entity of originator and terminator of the CLI related information.

Proposal 1: It is proposed RAN3 agree to exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration

- Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Strongest DL beam information

- CLI-mitigation request

2.2	Exchange of SBFD configuration

In the previous meeting, RAN1 has achieved the following agreements about SBFD time and frequency configuration:

- SBFD subband time-domain location indication Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot SBFD subband frequency-domain location indication Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

We think the gNB shall provide the SBFD time and frequency location configuration to other gNB in Xn Setup procedure with XnAP XN SETUP REQUEST message and XnAP XN SETUP RESPONSE message, and NG-RAN node Configuration Update procedure with XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message. Considering the SBFD time and frequency location configuration is cell-specific, similar to the exchange of current TDD UL-DL configuration and cell-specific frequency configuration, the SBFD time and frequency location configuration should also be provided at cell granularity.

Proposal 2: It is proposed RAN3 to agree that the gNB shall exchange the the SBFD time and frequency location configuration with cell granularity in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Currently, the TDD UL-DL Configuration Common NR IE and the Carrier List IE are included in Served Cell Information NR IE to indicate the time and frequency configuration of the cell.

| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |

In which, the TDD UL-DL Configuration Common NR IE is directly referred to the field tdd-UL-DL-ConfigurationCommon in TS 38.331, and the Carrier List IE is explicitly defined with relevant values. For the SBFD time configuration, RAN2 has not decided the signalling to indicate the time location of SBFD symbols, if the SBFD time configuration is extended in tdd-UL-DL-ConfigurationCommon IE in TS 38.331, the current XnAP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed need to be introduced to Served Cell Information NR IE. for frequency configuration, new filed need to be introduced to Served Cell Information NR IE. Thus, how to enable the exchange of SBFD time and frequency configuration among gNBs depends on RAN2 work.

One more thing is, since RAN2 is in cc in the LS from RAN1, maybe there is a need for RAN3 to remind RAN2 to work on the IE design of SBFD time and frequency configuration, and from RAN3 perspective, RAN1 should also provide the detailed parameters for the SBFD timing and frequency info.

Proposal 2bis: It is proposed RAN3 to agree that the local gNB provide SBFD time and frequency configuration in Served Cell Information NR IE to neighbour gNBs, the detailed IE design is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Besides, the F1AP specification should also support to provide the SBFD time and frequency configuration of the served cell and obtain the SBFD time and frequency configuration of other cells. In the current F1AP specification, the TDD UL-DL Configuration Common NR IE and Carrier List IE are provided from gNB-DU to gNB-CU. Similarly, the SBFD time and frequency location configuration should also be provided from gNB-DU to gNB-CU in Served Cell Information IE, RAN3 can simply use the way of enhancement for Xn interface. And also, to make gNB-DU know the SBFD configuration of the cell of neighbour gNBs, the F1AP should also be enhanced to provide the time and frequency configuration of other cells from gNB-CU to gNB-DU.

Proposal 3: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Proposal 3bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration received over Xn interface from gNB-CU to gNB-DU.

2.3	Exchange of measurement configuration and measurement result

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Information exchange of strongest DL beam information

Firstly, RAN3 should discuss whether to introduce the new procedure or enhance the existing procedure to support gNB-gNB CLI measurement info exchange. For this issue, there are three options may be used:

- Option 1: To reuse interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement configuration and result

Option 1 and option 2 are intend to reuse the existing procedure. among these two options, we think option2 is preferred to support gNB-gNB CLI measurement, since the existing framework of Resource Status Reporting procedures can support controlling CLI measurements with smaller changes, the gNB1 can trigger the gNB2 to perform measurement for the indicated cell and control the report periodicity. For option3, we think the similar procedures as option2 can be introduced.

Proposal 4: It is proposed RAN3 discuss which procedure to support the gNB-gNB CLI measurement info exchange, with the following options:

- Option 1: To reuse interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: To reuse resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement configuration and result

In addition, whatever the solution option above is to be chosen, we need to further consider the concrete IE design. As we could see that from the IE “Served Cell Information NR”, in which there is sub-IE “Additional Measurement Timing Configuration List” which further includes IE “CSI-RS Index”, similar case also could be seen for “SSB Index” in IE “Radio Resource Status” or “NG-RAN NODE CONFIGURATION UPDATE”. Technically, it is difficult to reuse the existing IE, since in general the configuration for “CSI-RS” or “SSB” in a cell supporting SBFD may be the sub-set configuration of the whole serving cell. With this understanding, we think there is a need to introduce a standalone IE, as the SBFD configuration for measurement configuration, i.e. NZP CSI-RS resources, and measurement result, i.e. strongest DL beam information in the context of CRS-RS or SSB. Obviously, such IE should be per-cell basis.

Proposal 4bis: It is proposed RAN3 agree to introduce standalone IEs as the SBFD gNB-gNB CLI measurement info, at least including NZP CSI-RS resources as measurement configuration, and strongest DL beam information in the context of CRS-RS or SSB as measurement results, which are per-cell basis.

2.4	Exchange of CLI-mitigation request

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of CLI-mitigation request

And in the WID, the following requirement is listed:

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

Based on these, we think RAN3 only needs to introduce the signalling of CLI-mitigation request while no need to discuss the node behavior upon reception of request, which basically includes two aspects:

1)	How to indicate this request

2)	Where to put this request.

For the first point, we think it is quite straightforward, just an indication IE, such as “CLI-mitigation requested”; while for where to put the CLI-mitigation request, we think there are the following options:

- Option 1: Reuse NG-RAN node Configuration Update procedure

- Option 2: Introduce New procedure to provide CLI-mitigation request

Proposal 5: It is proposed RAN3 to agree to introduce an indication IE such as “CLI-mitigation requested”

Proposal 5bis: It is proposed RAN3 discuss which procedure to be used to support CLI-mitigation request, in the following options:

- Option 1: Reuse NG-RAN node Configuration Update procedure

- Option 2: Introduce new procedure to provide CLI-mitigation request

Here the option 2 is preferred, and two stage 2 TPs to 38.420 and 38.470 and a draft LS are attached in the Annex part, stage3 TP to TS 38.423 in [3].

3. Conclusion

In this contribution, we analyzed the specification impacts of gNB-gNB mitigation. The following proposals were provided:

Proposal 1: It is proposed RAN3 agree to exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration

- Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Strongest DL beam information

- CLI-mitigation request

Proposal 2: It is proposed RAN3 to agree that the gNB shall exchange the the SBFD time and frequency location configuration with cell granularity in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 2bis: It is proposed RAN3 to agree that the local gNB provide SBFD time and frequency configuration in Served Cell Information NR IE to neighbour gNBs, the detailed IE design is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Proposal 3: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Proposal 3bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration received over Xn interface from gNB-CU to gNB-DU.

Proposal 4: It is proposed RAN3 discuss which procedure to support the gNB-gNB CLI measurement info exchange, with the following options:

- Option 1: To reuse interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: To reuse resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement configuration and result

Proposal 4bis: It is proposed RAN3 agree to introduce standalone IEs as the SBFD gNB-gNB CLI measurement info, at least including NZP CSI-RS resources as measurement configuration, and strongest DL beam information in the context of CRS-RS or SSB as measurement results, which are per-cell basis.Proposal 5: It is proposed RAN3 to agree to introduce an indication IE such as “CLI-mitigation requested”

Proposal 5: It is proposed RAN3 to agree to introduce an indication IE such as “CLI-mitigation requested”

Proposal 5bis: It is proposed RAN3 discuss which procedure to be used to support CLI-mitigation request, in the following options:

- Option 1: Reuse NG-RAN node Configuration Update procedure

- Option 2: Introduce new procedure to provide CLI-mitigation request

4. Reference

[1]. RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2]. R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation

[3]. R3-245217	BL CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

Annex A: TP to TS 38.420

## 5.2	Functions of Xn-C

### 5.2.1	Xn-C interface management and error handling functions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.2.X	CLI management function

This function allows exchanging CLI measurement configuration and result, and CLI-mitigation request between NG-RAN nodes, such that the NG-RAN node can manage CLI appropriately.

# 6	Xn interface procedures

## 6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

## 6.2	Control plane protocol procedures

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 6.2.X	CLI management procedures

The CLI management procedures are used by NG-RAN nodes to indicate CLI measurement configuration and result, and CLI-mitigation request to each other.

-	CLI measurement Initiation

-	CLI measurement Reporting

-	CLI mitigation request

Annex B: TP to TS 38.470

## 5.2	F1-C functions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.2.X	CLI management function

This function allows exchanging CLI measurement configuration and result, and CLI-mitigation request between the gNB-CU and the gNB-DU.

## 6.1	Control plane procedures

### 6.1.1	Interface Management procedures

The F1 Interface management procedures are listed below:

-	Reset procedure;

-	Error Indication procedure;

-	F1 Setup procedure;

-	gNB-DU Configuration Update procedure;

-	gNB-CU Configuration Update procedure;

-	gNB-DU Resource Coordination procedure;

-	gNB-DU Status Indication procedure;

-	F1 Removal procedure;

-	Network Access Rate Reduction procedure;

-	DU-CU TA Information Transfer procedure;

-	CU-DU TA Information Transfer procedure.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 6.1.X	CLI management procedures

The CLI management procedures are listed below:

-	CLI measurement Initiation

-	CLI measurement Reporting

-	CLI mitigation request

Annex C: Draft LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Title:	LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Release:	Release 19

Work Item:	NR_duplex_evo

Source:	TSG RAN WG3

To:	TSG RAN WG2, TSG RAN WG1

Cc:

Contact person:	Yang Xudong

yangxudong@huawei.com

# 1	Overall description

RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1.

RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration info, which should be based on parameters provided by RAN1.

With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN1 to work on the detailed parameters, and ask RAN2 to work on the IE design of SBFD time and frequency configuration info.

# 2	Actions

To TSG RAN1

ACTION: 	RAN3 respectfully asks RAN1 to provide the detailed parameters for SBFD time and frequency configuration info.

To TSG RAN2

ACTION: 	RAN3 respectfully asks RAN2 to work on the IE design of SBFD time and frequency configuration info.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#126	18th – 22nd November 2024	 	Orlando, US

RAN3#127	17th – 21st Feburary 2025	 	Athens, GR