3GPP TSG-RAN WG3 Meeting #125-bis	R3-245219

Hefei, CN, 14 - 18 Oct, 2024

Title: 	(TP for TS 38.473) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei

Agenda item:	17.3

Document Type:	Other

# Introduction

The last RAN#105 meeting updated and agreed the WID in [RP-242354], indicating only case 2 for on-demand SIB1 for UEs in idle/inactive mode.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

The following agreements have been made in the last RAN3-125 meeting:

Whether On-Demand SIB1 activation/deactivation is needed？ Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

# Discussion

As observed from the WID, for Case 2, the UE obtains the UL WUS configuration from Cell A, transmits UL WUS to NES Cell, and receives on-demand SIB1 from NES Cell.

The overall flowchart of Case 2 is exemplified in Figure 1 from our perspective.

- Figure 1. The On-demand SIB1 procedure

## 2.1 WUS broadcast feedback

Previous meetings reached many general agreements. Especially, upon reception of the UL WUS configuration, the Cell A can provide feedback to NES Cell on whether the WUS configuration transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

Then when the NES Cell CU receives feedback (e.g., confirm, reject), it should forward the feedback to the NES Cell DU. After that, the NES Cell DU will know that Cell A is acknowledged with the WUS configuration and can decide initiate on-demand SIB1 operation.

The NES Cell CU can notify the NES Cell DU of the reception status of the UL WUS configuration transmission (e.g., confirm/reject from Cell A).

## 2.2 Which node (NES Cell CU vs. NES Cell DU) decides whether the SIB1 should be broadcast or not

- Then, the next issue is which node determines to start/stop on-demand SIB1 broadcast. Basically, there are two options.

- Option 1: The NES Cell CU takes full decision and signals the decision to the NES Cell DU each time the NES Cell SIB1 broadcast status is changed (e.g., begin or stop SIB1 broadcast). It is FFS whether the NES Cell DU can reject.

- Option 2: The NES DU itself decides when to stop or begin the SIB1 broadcast with assistance from NES CU (i.e., allowed cell list).

As analysed above, when the NES Cell DU receives the UL WUS signal request (i.e., RACH) from the UE, according to RAN2 agreements below, it is the NES Cell DU to send to the RAR (e.g., accept, or reject). Hence it is the NES Cell DU to decide whether to begin the SIB1 broadcast. And it makes sense the NES Cell DU decides when to stop the SIB1 broadcast.

RAR monitoring for MSG1 based OD-SIB1 REQ 13. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

- Observation: The NES Cell DU receives the WUS signal to request the on-demand SIB1 broadcast and provide the RAR for accepting the request.

On the other hand, considering that the CU has general system level information, e.g., the neighbour Cell A deployment, system overload etc, it would be beneficial for the CU to provide some assistance information. Hence the CU could provide a cell list to indicate which cells are allowed to enable on-demand SIB1 operation. After that, the DU could determine itself whether to enter the on-demand SIB1 operation to save the network energy based on the load etc in time.

The NES Cell CU can provide the NES Cell DU with a list of allowed cells, and the NES Cell DU itself can determine when to start or stop the SIB1 broadcast.

## 2.3 The NES Cell OD SIB1 Status Indicator

In last meetings, it was discussed whether On-Demand SIB1 activation/deactivation is needed. As indicated in Figure 1, in addition to the UL WUS configuration, the NES Cell OD SIB1 Status Indicator should also be considered. The NES Cell OD SIB1 Status Indicator is to request Cell A to broadcast or stop broadcast the WUS configuration.

Specifically, when the NES Cell receives an uplink WUS signal (i.e., RACH) from the UE and begins broadcasting SIB1, it should notify Cell A to stop broadcasting the uplink WUS configuration. Conversely, when the NES Cell decides to stop its SIB1 broadcast for energy saving purpose, it should inform Cell A to resume broadcasting the uplink WUS configuration. The OD SIB1 Status Indicator (e.g., on, off) would reflect the current SIB1 broadcast status of NES Cell and help determine whether Cell A should start or stop broadcasting the NES Cell's WUS configuration.

In the split structure, the NES Cell DU that holds the SIB1 should send the OD SIB1 Status Indicator (e.g.,  on, off) to the NES Cell CU via the F1 interface.

The NES Cell can send an SIB1 Status Indicator to Cell A via the Xn interface, indicating whether to broadcast the WUS configuration based on the current OD SIB1 status. For example, if the status is “on”, it indicates that the uplink WUS configuration should be broadcasted.

The NES Cell DU can send an SIB1 Status Indicator to NES Cell CU via the F1 interface, indicating whether to broadcast the WUS configuration based on the current OD SIB1 status.

In addition, under some cases e.g., cell overload, energy cost etc., Cell A CU may determine it can no longer broadcast the WUS configuration for NES cell, it can send a Stop WUS Configuration Broadcast Request to the NES Cell CU via the Xn interface. In this case, the NES Cell CU should always accept the request.

In the split structure, Cell A DU can send a Stop WUS Configuration Broadcast Request to Cell A CU, which will then accept the request and forward it to the NES Cell.

The Cell A CU can send a Stop WUS Configuration Broadcast Request to the NES Cell CU via the Xn interface, upon which the NES Cell CU should always accept the request.

The Cell A DU can send a Stop WUS Configuration Broadcast Request to the Cell A CU over F1 interface. RAN3 can further discuss whether the Cell A CU should always accept the request.

## Other aspects

- Previous RAN3-125 meeting also had an FFS regarding Cell A’s broadcast timing of WUS Configuration:

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

- It is our understanding that this FFS is more like an implementation issue, and other WGs should first discuss this matter due to several reasons:

  - It involves both the physical layer aspects, managed by RAN1, such as timing, resource allocation, and synchronization of the WUS broadcast.

  - The system information broadcast falls under the scope of RAN2, including the configuration and coordination of signalling messages between cells.

Remove the FFS regarding Cell A broadcasts right way or not, which can be left to the node implementation or other WGs.

## New procedure vs. legacy procedure

There were proposals at previous meeting discussing whether to define new procedures or reuse the legacy procedures to enable the inter-node on-demand SIB1 coordination. Both options have their pros and cons.

A new procedure can be used solely for on-demand SIB1 purpose and potentially support NES features. However, this raises the question of why the legacy procedure cannot be reused, especially considering the Rel-18 NES is reusing the legacy procedures. If legacy procedure is reused, we can easily introduce new IEs under some existing parent IEs, but this approach may lead to readability issue.

RAN3 can discuss further whether to enhance the existing interface management messages or define new procedure(s) for On-demand SIB1, considering the pros and cons of each option.

RAN3 to discuss whether to enhance the existing interface management messages or define new procedure(s) for On-demand SIB1.

The Annex provides these two options for TP for TS 38.473, and a flow chart of Option 2 implementation is provided below.

- Figure 2. The On-demand SIB1 procedure (Option 2)

# 3. Conclusion

- The NES Cell CU can notify the NES Cell DU of the reception status of the UL WUS configuration transmission (e.g., confirm/reject from Cell A).

- Observation: The NES Cell DU receives the WUS signal to request the on-demand SIB1 broadcast and provide the RAR for accepting the request.

- The NES Cell CU can provide the NES Cell DU with a list of allowed cells, and the NES Cell DU itself can determine when to start or stop the SIB1 broadcast.

- The NES Cell can send an SIB1 Status Indicator to Cell A via the Xn interface, indicating whether to broadcast the WUS configuration based on the current OD SIB1 status. For example, if the status is “on”, it indicates that the uplink WUS configuration should be broadcasted.

- The NES Cell DU can send an SIB1 Status Indicator to NES Cell CU via the F1 interface, indicating whether to broadcast the WUS configuration based on the current OD SIB1 status.

- The Cell A CU can send a Stop WUS Configuration Broadcast Request to the NES Cell CU via the Xn interface, upon which the NES Cell CU should always accept the request.

- The Cell A DU can send a Stop WUS Configuration Broadcast Request to the Cell A CU over F1 interface. RAN3 can further discuss whether the Cell A CU should always accept the request.

- Remove the FFS regarding Cell A broadcasts right way or not, which can be left to the node implementation or other WGs.

- RAN3 to discuss whether to enhance the existing interface management messages or define new procedure(s) for On-demand SIB1.

The Annex provides these two options for TP for TS 38.473.

# 4. Reference

- RAN3_125_agenda_20240823EOM

# 5. Option 2: TP for TS 38.473 – new procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |
| Multicast Context Notification | MULTICAST CONTEXT NOTIFICATION INDICATION | MULTICAST CONTEXT NOTIFICATION CONFIRM | MULTICAST CONTEXT NOTIFICATION REFUSE |
| Multicast Common Configuration | MULTICAST COMMON CONFIGURATION REQUEST | MULTICAST COMMON CONFIGURATION RESPONSE | MULTICAST COMMON CONFIGURATION REFUSE |
| NES gNB-DU On-demand SIB1 | NES GNB-DU ON-DEMAND SIB1 REQUEST | NES GNB-DU ON-DEMAND SIB1 RESPONSE | NES GNB-DU ON-DEMAND SIB1 FAILURE |
| NES gNB-CU On-demand SIB1 | NES GNB-CU ON-DEMAND SIB1 REQUEST | NES GNB-CU ON-DEMAND SIB1 RESPONSE | NES GNB-CU ON-DEMAND SIB1 FAILURE |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.xx	NES On-demand SIB1 procedures

### 8.xx.1	NES gNB-DU On-demand SIB1

#### 8.xx.1.1	General

The purpose of the NES gNB-DU On-demand SIB1 procedure is to enable the gNB-DU to request the gNB-CU to enable the On-demand SIB1 related operations. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful Operation

Figure 8.2.16.3-1: NES gNB-DU On-Demand SIB1 procedure: Successful Operation

The gNB-DU initiates the on-demand SIB1 procedure by sending a NES GNB-DU ON-DEMAND SIB1 REQUEST message to the gNB-CU.

If the NES Cell To Request IE is contained in the NES GNB-DU ON-DEMAND SIB1 REQUEST message, the gNB-CU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6], and return the result in the NES GNB-DU ON-DEMAND SIB1 REQUEST RESPONSE message including the NES Cell To Response List IE

If the Cell A To Request List IE is contained in the NES GNB-DU ON-DEMAND SIB1 REQUEST message, the gNB-CU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6].

Editor’s note: The above procedure texts need to be reformulated based on further progress.

#### 8.xx.1.3	Unsuccessful Operation

Figure 8.2.16.3-1: NES gNB-DU On-Demand SIB1 procedure: Unsuccessful Operation

If the gNB-CU is not able to accept the on-demand SIB1 request, it shall respond with a NES GNB-DU ON-DEMAND SIB1 FAILURE message and appropriate cause value.

#### 8.xx.1.4	Abnormal Conditions

Not applicable.

### 8.xx.2	NES gNB-CU On-demand SIB1

#### 8.xx.2.1	General

The purpose of the NES gNB-CU On-demand SIB1 procedure is to enable the gNB-CU to request the gNB-DU to  enable the On-demand SIB1 related operations. The procedure uses non-UE associated signalling.

#### 8.xx.2.2	Successful Operation

Figure 8.2.17.2-1: NES Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a NES GNB-CU ON-DEMAND SIB1 REQUEST message to the gNB-DU.

If the Cell A To Request IE is contained in the NES GNB-CU ON-DEMAND SIB1 REQUEST message, the gNB-DU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6], and return the result in the NES GNB-CU ON-DEMAND SIB1 REQUEST RESPONSE message including the Cell A To Response List IE.

If the NES Cell To Request List IE is contained in the NES GNB-CU ON-DEMAND SIB1 REQUEST message, the gNB-CU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6].

If the On-Demand SIB1 Allowed List Received IE is included in the NES GNB-CU ON-DEMAND SIB1 REQUEST message, the gNB-DU shall, if supported, consider the cells within the indicated list allowed to enable the on-demand SIB1 broadcast.

Editor’s note: The above procedure texts need to be reformulated based on further progress.

#### 8.xx.2.3	Unsuccessful Operation

Figure 8.2.17.3-1: NES Configuration Update procedure: Unsuccessful Operation

If the gNB-DU is not able to accept the on-demand SIB1 request, it shall respond with a NES GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

#### 8.xx.2.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.aa	Network Energy Saving On-demand SIB1 Messages

#### 9.2.aa.1	NES GNB-DU ON-DEMAND SIB1 REQUEST

This message is sent by the gNB-DU to request the NES on-demand SIB1 related operations.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NES Cell To Request List |  | 0..1 |  | List of NES cells served by the gNB-DU | YES | ignore |
| >NES Cell To Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>UL WUS Configuration Information [FFS] | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | - |  |
| >>NES Cell SIB1 Status Indicator | O |  | ENUMERATED (on, off, ...) |  | - |  |
| Cell A To Request List |  | 0..1 |  | List of Cell A(s) served by the gNB-DU | YES | ignore |
| >Cell A To Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Cell A Stop Broadcast Indicator | O |  | ENUMERATED (true, ...) |  | - |  |

#### 9.2.aa.2	NES GNB-DU ON-DEMAND SIB1 RESPONSE

This message is sent by a gNB-DU to a gNB-CU to acknowledge to the NES on-demand SIB1 related operations.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NES Cell To Response List |  | 0..1 |  | List of NES cells served by the gNB-DU | YES | ignore |
| >NES Cell To Response List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NES Cell SIB1 Status Indication Response | O |  | ENUMERATED (true, ...) |  | - |  |
| >>Cell A Broadcast Indication Response | O |  | ENUMERATED (accept, reject...) |  | EACH | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.aa.3	NES GNB-DU ON-DEMAND SIB1 FAILURE

This message is sent by the gNB-DU to indicate gNB-CU the NES on-demand SIB1 failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.aa.4	NES GNB-CU ON-DEMAND SIB1 REQUEST

This message is sent by the gNB-CU to request the NES on-demand SIB1related operations.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cell A To Request List |  | 0..1 |  | List of Cell A served by the gNB-DU. | YES | ignore |
| >Cell A To Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>UL WUS Configuration Information [FFS] | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | - |  |
| >>NES Cell SIB1 Status Indicator | O |  | ENUMERATED (on, off, ...) |  | - |  |
| NES Cell To Request List |  | 0..1 |  | List of NES cell served by the gNB-DU. | YES | Ignore |
| >NES Cell to Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Cell A Stop Broadcast Indicator | O |  | ENUMERATED (true, ...) |  | - |  |
| On-Demand SIB1 Allowed List |  | 0 ..1 |  | List of allowed NES Cells | YES | ignore |
| >On-Demand SIB1 Allowed List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.aa.5	NES GNB-CU ON-DEMAND SIB1 RESPONSE

This message is sent by a gNB-DU to a gNB-CU to acknowledge the NES on-demand SIB1 related operations.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cell A To Response List |  | 0..1 |  | List of Cell A served by the gNB-DU. | YES | ignore |
| >Cell A To Response Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell SIB1 Status indicator | O |  | ENUMERATED (true, ...) |  | - |  |
| >>Cell A Broadcast Indication Response | O |  | ENUMERATED (accept, reject...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.aa.6	NES GNB-CU ON-DEMAND SIB1 FAILURE

This message is sent by the gNB-CU to indicate gNB-DU the NES on-demand SIB1 failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s note: The details of the above procedures and details (e.g., IE name, IE format) need to be further reformulated.

# 5. option 1: TP for TS 38.473 – enhancing the legacy procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NES Cell SIB1 Status Indicator IE is included in the Served Cell To Modify IE in the gNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to determine the broadcast status of the UL WUS configuration, and return the result in the gNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message including the NES Cell SIB1 Status Indication Response List IE.

If the Cell A Stop Broadcast Indicator IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the Cell A requests to stop the uplink WUS configuration broadcast.

Editor’s note: The above procedure texts need to be refined based on further progress.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Cell A To Request List IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6], and return the result in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message including the Cell A To Response List IE.

If the NES Cell To Request List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information for on-demand SIB1 broadcast as specified in TS 38.300 [6].

If the On-Demand SIB1 Allowed List IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cells within the indicated list allowed to enable the on-demand SIB1 broadcast.

Editor’s note: The above procedure texts need to be refined based on further progress.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| >>NES Cell SIB1 Status Indicator | O |  | ENUMERATED (on, off, ...) |  | YES | ignore |
| >>Cell A Stop Broadcast Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| NES Cell SIB1 Status Indication Response List |  | 0..1 |  | Only used between the NES Cell gNB-DU and NES gNB-CU | YES | reject |
| >NES Cell SIB1 Status Indication Response List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >NES Cell SIB1 Status Indication Response | O |  | ENUMERATED (true, ...) |  | - |  |
| >Cell A Broadcast Indication Response | O |  | ENUMERATED (accept, reject...) |  | - |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |
| eRedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselection-eRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which eRedCap UEs are allowed access, according to the setting of the barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit=half-duplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | Ignore |
| UL WUS Configuration Information [FFS] | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

Editor’s note: The details of the UL WUS Configuration Information need to be defined, and are pending to other groups.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cell A To Request List |  | 0..1 |  | List of Cell A served by the gNB-DU. | YES | ignore |
| >Cell A To Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>UL WUS Configuration Information [FFS] | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | - |  |
| >>NES Cell SIB1 Status Indicator | O |  | ENUMERATED (on, off, ...) |  | - |  |
| NES Cell To Request List |  | 0..1 |  | List of NES cell served by the gNB-DU. | YES | Ignore |
| >NES Cell to Request Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Cell A Stop Broadcast Indicator | O |  | ENUMERATED (true, ...) |  | - |  |
| On-Demand SIB1 Allowed List |  | 0 ..1 |  | List of allowed NES Cells | YES | ignore |
| >On-Demand SIB1 Allowed List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells with SSBs Activated List |  | 0..1 |  |  | YES | ignore |
| >Cells with SSBs Activated List Item |  | 1.. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>SSBs activated List |  | 1 .. < maxnoofSSBAreas > |  |  | - |  |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam activated. | - |  |
| Cell A To Response List |  | 0..1 |  | List of Cell A served by the gNB-DU. | YES | ignore |
| >Cell A To Response Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell SIB1 Status Indication Response | O |  | ENUMERATED (true, ...) |  | - |  |
| >>Cell A Broadcast Indication Response | O |  | ENUMERATED (accept, reject...) |  | - |  |

Editor’s note: The details of the IE names and contents need to be further reformulated.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>