3GPP TSG-RAN WG3 Meeting #125-bis	R3-245220

Hefei, CN, 14 - 18 Oct, 2024

Title: 	(TP for TS 38.413 and TS 38.423) Discussion on Low-power wake-up signal and receiver for NR

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# 1. Introduction

In RAN#105 meeting, the WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was further refined in [1]. And at the previous RAN3-125 meeting, it was agreed that:

New IEs should be introduced to support LP-WUS on NG, Xn and F1.

In this contribution, we continue to discuss the RAN3 related issues on the procedure and configuration for RRC_IDLE/INACTIVE UEs.

# 2. Discussion

## 2.1 General introduction of LP-WUS/WUR

The study item on the LP-WUS/WUR was discussed and concluded in the TR 38.869 [2]. Instead of periodically monitoring the PDCCH in each DRX cycle, the UE power consumption may be significantly decreased if the UE can only wake up when they are triggered, e.g., by paging. This can be achieved by using a wake-up signal (i.e., LP-WUS) to trigger the main radio (MR) and a separate receiver (i.e., LP-WUR) which has the ability to monitor LP-WUS with ultra-low power consumption.

As indicated in Figure.1, the LP-WUR is responsible for detecting and processing LP-WUS along with a newly-designed reference signal (i.e. LP-SS). The MR works for legacy signalling and data transmission and reception such as PDCCH/PDSCH monitoring and PSS/SSS reception, and can be turned off or set to “ultra-deep sleep” state when LP-WUR is on. The power saving gain of the LP-WUS/WUR mainly comes from the new sleep state of MR as well as the low-power design of the wake-up signal and hardware module of the wake-up receiver.

Figure 1: general framework of LP WUS/WUR

In RRC_IDLE/INACTIVE state, the impact of LP-WUS on the legacy procedures should be considered, which specifies how UEs shall monitor paging, keep system information up to date, receive ETWS/CMAS information, and perform RRM measurements and corresponding cell selection/re-selection to ensure that UE is camping on the suitable cell. During the previous meetings, RAN1 and RAN2 made many agreements on the LP-WUS/WUR in RRC_IDLE/INACTIVE state, which is given in the following sections.

## 2.2 LP-WUS CN-based Subgrouping and UE-ID based Subgrouping

As observed in the WID, the main objective for LP-WUS in RRC_IDLE/INACTIVE mode is to trigger the paging monitoring.  RAN1 made the following related agreements in RAN1#116 [2]:

RAN1#116 Agreements - For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. - It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO - Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Observation 1: The UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

Observation 2: For a UE supporting PEI, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Meanwhile, RAN2 discussed that two types of subgrouping method can be reused from Rel-17 PEI, i.e., CN controlled and UE_ID based subgrouping and the following agreement were made at previous two RAN2 meetings [3] [4].

Agreement at RAN2-125bis The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. Agreement at RAN2-126 RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

Observation 3: For LP-WUS, CN assigned and UE_ID based subgrouping are supported, and it is up to CN to assign the LP-WUS subgroup ID to the UE.

As agreed at previous RAN3-125 meeting [5], new IEs should be introduced to support the LP-WUS over NG, Xn and F1. Specifically, the following changes are needed.

- For RRC-idle UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the NGAP paging message for the UE capable of CN controlled subgrouping.

- For RRC-inactive UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, and in the XnAP Paging message.

- Over the F1 interface, the LP-WUS assistance information can be introduced in the paging message. And RAN3 can continue discuss whether to introduce a LP-WUS Subgrouping support indication, based on other WG progress.

Observation 4: The detailed RAN3 specification change includes:

- For RRC-idle UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the NGAP paging message for the UE capable of CN controlled subgrouping.

- For RRC-inactive UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, and in the XnAP Paging message.

- Over the F1 interface, the LP-WUS assistance information can be introduced in the paging message. And RAN3 can continue discuss whether to introduce a LP-WUS Subgrouping support indication, based on other WG progress.

RAN3 to agree the TPs to cover the above RAN3 specification changes including the LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID.

- From RAN1’s latest agreement [6], the maximum number of subgroups for LP-WUS will be down-selected between several candidate values ranging from 8 to 256.

RAN1#118 Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256

First, it can be observed that the maximum number of subgroups are not finalized yet. It could be 8, the same as PEI CN-assigned subgroup ID, or even larger. RAN3 can set the maximum number of LP-WUS CN-based subgroup ID as FFS in the TPs to be agreed

RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as FFS in the TPs to be agreed.

Second, the gNB will be provided by the core network or other gNB with an IE of Extended UE Identity Index Value carried in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, or in the XnAP RAN Paging message.

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

The Extended UE Identity Index Value IE contains 16 least significant bits of 5G-S-TMSI (i.e., UE ID), in which up to 12 bits are to determine the PF and PO (if configured with eDRX) and up to 15 bits (if configured with eDRX) are used for PEI subgrouping as specified in TS 38.304 [7].

If RAN2 agrees to allocate PEI and LP-WUS subgroups independently and separately, the design of LP-WUS subgrouping should ensure the bits in UE ID used for LP-WUS will never overlap with those used for PEI. In this case, the Extended UE Identity Index Value IE with 16 bits may not be sufficient if both LP-WUS and PEI are supported by UE and gNB. Therefore, RAN3 should wait for RAN1 to determine the maximum number of LP-WUS subgroups as well as RAN2 to design the UE_ID based subgrouping, then to discuss whether/how to extend the size of Extended UE Identity Index Value IE, e.g., to introduce a new IE with larger size.

Observation 5: The current size of Extended UE Identity Index Value IE may not be sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 is pending on the extending the size of Extended UE Identity Index Value IE, based on RAN1/2 progress on the total LP-WUS subgroup number and the UE_ID based subgrouping.

## 2.3 LP-WUS capability indication in the F1 paging message

In the RAN1 liaison in [8], it is described that the UE may not support LP-WUS reception on all the bands supported by the UE as follows.

# 1 Overall description In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. # 2 Actions To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

This means that the LP-WUS capability would be band-specific capability for the UE.  In the F1 Paging message, the CU can indicate the LP-WUS capability to the DU within the Paging Cell List, so that the DU can page the UE in the indicated cells using the LP-WUS, while not in the other indicated cells.

Introduce the cell-level LP-WUS capability in the F1 Paging message, marked as FFS to be finally confirmed by other WGs.

## 2.4 LP-WUS/LP-SS Configuration over F1 interface

RAN2 already concluded that the LP-WUS configuration for RRC_IDLE/INACTIVE UEs should be carried in system information, as given below [4].

RAN2#126 Agreement The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

In the CU/DU split case, RAN3 can discuss whether the LP-SS/LP-WUS configuration is generated by the DU or the CU. This may be dependent on which SIB would carry the configuration, which is to be decided by RAN2.

- Option 1: in SIB1.

- Option 2: in a new SIB.

If the LP-WUS related configuration is provided in SIB1, naturally it is up to gNB-DU configuration. It is our opinion that the LP-SS and LP-WUS configuration are more related to lower-layer resource configuration, then the gNB-DU can easily decide both parameters.

For RRC idle/inactive UEs, RAN3 is pending to discuss whether the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU (dependent on the SIB agreements in other WGs).

## 2.5 Entry/exit condition for LP-WUS monitoring

Regarding the entry/exit condition, RAN2’s latest meetings agreed the following as baseline [4][9]:

RAN2#126 Agreement Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2#127 Agreement Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. RAN2#127 Working Assumption Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

It can be observed that the LP-WUS entry/exit condition is based on the pre-configured RRC measurement threshold. RAN3 can discuss whether the gNB-CU or gNB-DU can generate the Entry/exit condition for LP-WUS monitoring. And as indicated in the WID, the network need not be aware of whether the idle/inactive UE is monitoring LP-WUS or not. Hence RAN3 can conclude that no further impacts when the idle/inactive UE enters or leave the LP-WUS monitoring state.

For RRC idle/inactive UEs, RAN3 is pending to discuss whether Entry/exit condition is generated at the gNB-DU or the gNB-CU (dependent on the SIB agreement in other WGs). And RAN3 considers no further impacts when the idle/inactive UE enters or leave the LP-WUS monitoring state.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: The UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

Observation 2: For a UE supporting PEI, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Observation 3: For LP-WUS, CN assigned and UE_ID based subgrouping are supported, and it is up to CN to assign the LP-WUS subgroup ID to the UE.

Observation 4: The detailed RAN3 specification change includes:

- For RRC-idle UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the NGAP paging message for the UE capable of CN controlled subgrouping.

- For RRC-inactive UE, a new LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID can be included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP, and in the XnAP Paging message.

- Over the F1 interface, the LP-WUS assistance information can be introduced in the paging message. And RAN3 can continue discuss whether to introduce a LP-WUS Subgrouping support indication, based on other WG progress.

- RAN3 to agree the TPs to cover the above RAN3 specification changes including the LP-WUS assistance information carrying the LP-WUS CN-based subgroup ID.

RAN3 to set the maximum number of LP-WUS CN-based subgroup ID as FFS in the TPs to be agreed.

Observation 5: The current size of Extended UE Identity Index Value IE may not be sufficient if both PEI and LP-WUS are supported by UE and gNB.

RAN3 is pending on the extending the size of Extended UE Identity Index Value IE, based on RAN1/2 progress on the total LP-WUS subgroup number and the UE_ID based subgrouping.

Introduce the cell-level LP-WUS capability in the F1 Paging message, marked as FFS to be finally confirmed by other WGs.

For RRC idle/inactive UEs, RAN3 is pending to discuss whether the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU (dependent on the SIB agreements in other WGs).

For RRC idle/inactive UEs, RAN3 is pending to discuss whether Entry/exit condition is generated at the gNB-DU or the gNB-CU (dependent on the SIB agreement in other WGs). And RAN3 considers no further impacts when the idle/inactive UE enters or leave the LP-WUS monitoring state.

The TPs to NGAP and XnAP are provided in the Annex to capture the above agreements. And the F1AP CR is provided in [10].

# 4. Reference

- RP‑241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- 3GPP TR 38.869 V18.0.0, Study on low-power wake-up signal and receiver for NR;

- RAN2#125bis chairman notes;

- RAN2#126 chairman notes;

- RAN3#125 chairman notes;

- RAN1#118 chairman notes;

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode

- RAN2#127 chairman notes;

- R3-245221	Introduction of low-power wake-up signal and receiver for NR	Huawei

# 5. TP to TS 38.413

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

LP-WUS	Low Power Wake UP Signal

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.3	UE Context Management Procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, replace the previously provided Core Network Assistance Information for RRC INACTIVE and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.5.1	Paging

#### 8.5.1.1	General

The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Paging

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Paging Cause IE is included in the PAGING message, the NG-RAN node shall, if supported, transfer it to the UE according to TS 38.331 [18] and TS 36.331 [21].

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

If the LP-WUS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for LP-WUS paging subgrouping of the UE, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.4	Paging Messages

#### 9.2.4.1	PAGING

This message is sent by the AMF and is used to page a UE in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Paging Identity | M |  | 9.3.3.18 |  | YES | ignore |
| Paging DRX | O |  | 9.3.1.90 |  | YES | ignore |
| TAI List for Paging |  | 1 |  |  | YES | ignore |
| >TAI List for Paging Item |  | 1..<maxnoofTAIforPaging> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Paging Priority | O |  | 9.3.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Paging Origin | O |  | 9.3.3.22 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.3.1.69 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.3.1.138 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.3.1.139 | If this IE is present, the Paging DRX IE is ignored. | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| WUS Assistance Information | O |  | 9.3.1.143 |  | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates whether the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.3.62 |  | YES | ignore |
| CN MT Communication Handling | O |  | ENUMERATED (supported, …) | This IE indicates the CN support of CN based MT communication handling. | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.3.1.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aaa	LP-WUS Assistance Information

This IE provides the LP-WUS information related to CN assigned subgrouping for a particular UE, as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..7, …) [FFS] |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<ASN.1 to be added when the above is stable>

# 6. TP to TS 38.423

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LBT	Listen-Before-Talk

LP-WUS	Low Power Wake UP Signal

MBS	Multicast/Broadcast Service

MCG	Master Cell Group

ML	Machine Learning

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE Specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.2.3.144a |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.172 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.2.3.195 |  | YES | ignore |
| LP-WUS Assistance Information | O |  | 9.3.3.aaa |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.aaa	LP-WUS Assistance Information

This IE provides the LP-WUS information related to CN assigned subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..7, …)[FFS] |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<ASN.1 is to be added when the above is stable>