3GPP TSG-RAN WG3 Meeting #125-bis	R3-245232

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP for BLCR for SON for 38.413, 36.423) Rel-18 SON and MDT leftovers

Agenda Item:	10.4

Source:	Huawei

Document for:	other

# 1. Introduction

The agreements in previous meetings are as follows:

RAN3#123b:

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

LS to RAN2 on the feasibility to support MRO for SCG failure in R19 in R3-242195

RACH optimization for SDT:

Work on scenario of RACH optimization for SDT first.

RAN3#124:

RACH optimization for SDT:

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

MRO for MR-DC SCG failure:

To be continued based on the reply LS from RAN2.

Check RAN2 progress

RAN3#125:

MRO for MR-DC SCG failure:

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce stage 2 descriptions of PSCell change failure in EN-DC in TS36.300. Check/agree TP in R3-244461.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

Include IEs in SCGFailureInformationNR message in SCG Failure Information Report message. Check/agree TP in R3-244766.

RACH optimization for SDT

The information that stored and reported in the RA report for SDT:

- a. RSRP when SDT fails

- b. Remaining data volume after SDT transmission

- c. the time from the start of RA-SDT to the reporting of RA report

- d. CG-SDT related information if the UE is configured with CG-SDT but fails to initiate the CG-SDT

- e. indication regarding that the UE failed to initiate RA-SDT or CG-SDT

# 2. Discussion

## 2.1	RACH optimization for SDT [WFF]

From our point of view, at least the following information are helping the network to analyze the issues causing SDT failure and optimize the related condition parameters to maximize the performance of SDT:

- Data volume at the time of initiating the SDT or normal RA

- DL RSRP at the time of initiating the SDT or normal RA

- Configured RSRP and data volume via SIB1

The first two seems to be agreed in RAN2 and for the 3rd one we have the previous agreement to not report this from the UE. In turn, the RAN2#123 meeting has confirmed the agreent achieved by RAN3 that there is no need to add logging of sdt-RSRP-Threshold. For enabling the network to retrieve the 3rd, the last RAN3 meeting has decided to report the time between the SDT and the SDT report.  We propose that, RAN3 inform RAN2 of the RAN3’s decision.

Send our agreement to RAN2 regarding logging the time from the start of RA-SDT to the reporting of RA report.

## 2.3 UHI and MHI enhancement for SCG deactivation/activation [HO]

The potential anhancement of MHI/UHI was put on hold in RAN3 while waiting for RAN2 progress. This was discussed in Last RAN2 meeting with the following conclusions

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

The hierarchical structure of the UHI is a list of 16 PCells where each PCell can include a list of 16 PSCells. This IE is defined as below in NGAP:

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |

As can be seen above, the IE already contains the stay time. It is also worth noting that the stay time is capped at 4096 seconds. Hence, the most straightforward solution would be to add an optional IE rrepresenting the ratio of time the SCG is in activated state, i.e. as indicated below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

## 2.2	MRO for MR-DC SCG failure [WFF]

We agreed some TPs in last meeting. Howerver, we notice that the en-gNB shall be restricted to en-gNB acting as Secondary Node in EN-DC, which is missing in the current BLCR. Besides, there is a mistake about the direction of the SCG failure transfer procedure from MeNB to en-gNB as SN in the illustrated figure 8.7.y.w-1. The description text is correct, showing that it originates from en-gNB to MeNB.

Agree to update the TP as included in the annex.

# 3	Conclusion

This document proposes:

Proposal 1:	Send our agreement to RAN2 regarding logging the time from the start of RA-SDT to the reporting of RA report.

Proposal 2:	Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

Proposal 3:	Agree to update the TP as included in the annex.

# Annex	LS to RAN2 on SDT

Title:	[DRAFT] LS on SDT enhancements for SON

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed SDT enhancements for SON. RAN3 noted that RAN2 already made some progress and agreed that in addition to agreements in RAN2, RAN3 sees benefit in including the time between the RA event and the report in the RA report. This could be used by the network to determine the configuration that was used at the time the RA event occurred.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

# Annex	TP for UHI for 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-RelativeTimeInSCGActivatedState,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

[snip]

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RelativeTimeInSCGActivatedState	CRITICALITY ignore	EXTENSION RelativeTimeInSCGActivatedState		PRESENCE optional

...

}

[snip]

RelativeAMFCapacity ::= INTEGER (0..255)

RelativeTimeInSCGActivatedState ::= INTEGER (0..100)

ReportArea ::= ENUMERATED {

cell,

...

}

# Annex	TP for MR-DC SCG failure for 36.423

### 8.7.x	SCG Failure Information Report

#### 8.7.x.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to an en-gNB.

The procedure uses UE-associated signalling.

#### 8.7.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB acting as Secondary Node in EN-DC. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

#### 8.7.x.3	Unsuccessful Operation

Not applicable.

#### 8.7.x.4	Abnormal Conditions

Void.

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB acting as Secondary Node in EN-DC initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.x	SCG FAILURE INFORMATION REPORT

This message is sent by the MeNB to the en-gNB acting as Secondary Node in EN-DC to report a PSCell change failure event.

Direction: MeNB  the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Source PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9] | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) |  | YES | ignore |

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB acting as Secondary Node in EN-DC to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |