3GPP TSG-RAN WG3 Meeting #125bis	R3-245236

Hefei, China, 14 – 18 October 2024

Agenda item:	31.1

Source:	Nokia

Title:	Correction of MBS PDCP packets discard

Document for:	Discussion and Decision

# 1	Discussion

At the last RAN3#125 the unresolved issue of PDCP discard of broadcast packets related to MBS broadcast resource sharing release 18 feature was discussed again without concluding.

The issue arises for both MOCN case and multiple cell ID case when there is a switch of MBS shared tunnel.

At RAN3#125, the CR in [1] proposed solutions which were not agreed.

The proposed solutions were:

- For MOCN case: when the gNB-CU switches the NG-U tunnel from PLMN1 to PLMN2, the gNB-CU ensures PDCP SN continuity i.e. it sets the PDCP SN of the first packet received over the new NG-U 2 and to be delivered over the F1-U tunnel with the PDCP SN of the next packet to be delivered to the UE (PDCP SN continuity), so that the UE will not discard the packets but instead re-initialize its PDCP SN with the received value.

NOTE 1: It is expected that the gNB CU-UP supports PDCP SN continuity after establishment of shared NG-U resources with 5GC2.

- For multiple-cell ID case, it was proposed that the DU may use different MRB configuration in order to trigger a release/add MRB action for the UEs which would re-establish the PDCP.

NOTE 2: The gNB-DU1/2 may use different MRB configuration after the shared NG-U and F1-U path change, to enable the UEs to release and add related MRB to avoid unexpected packets discard.

It was commented that the note 2 does not work because it relies on a wrong UE assumption and therefore the CR in [1] was noted altogether without discussion on note 1.

Actually, the proposal in note 1 is very different than note 2. Because the MBS broadcast does not require the same synchronization over the 5GC as multicast, the PDCP SN set by the gNB-CU is not controlled by the QoS flow SN of the MB-UPF and the gNB-CU can freely change the PDCP SN at the path switch into the SN value of the next packet to be delivered.

Therefore, the proposal on Note 1 can work.

Moreover, similar solution could be applied to multiple cell ID case i.e. if the gNB-CU of PLMN2 is made aware of the SN of next packet to be delivered by the gNB-DU there is no reason why the gNB-CU of PLMN2 could not initialize or re-initialize as well the PDCP SN to this value over the new F1-U tunnel of PLMN2.

Proposal 1: agree the PDCP SN continuity to solve the PDCP discard issue for both MOCN and Multiple Cell ID case and the corresponding CRs in [2] presented in annex A and [3] presented in annex B.

# 2	Conclusion and Proposals

This paper has continued the discussion on PDCP discard issue initiated by tdoc [1] and makes the following proposal to solve it:

Proposal 1: agree the PDCP SN continuity to solve the PDCP discard issue for both MOCN and Multiple Cell ID case and the corresponding CRs in [2], [3], presented in in annex A and annex B.

# 3	References

- R3-244097, Correction on PDCP packets discard in case of MBS broadcast path update

- R3-24xxxx, Correction on PDCP packets discard in case of MBS broadcast path update, TS 38.401

- R3-24xxxx, Correction on PDCP packets discard in case of MBS broadcast path update, TS 38.473

# 4	Annex A: CR for TS 38.401

3GPP TSG-RAN WG3 Meeting #125bis	R3-24xxxx

Hefei, China, 14 - 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0xxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCP packets discard in case of MBS broadcast path update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Broadcast RAN sharing discussion, it was discussed that for an ongoing broadcast service, if the PDCP COUNT of the packets received by the UE is outside of the UE’s receiving window, the UEs will drop these packets and it is not preferred. In case of MOCN, if the gNB-CU-CP decides (e.g., in case the 5GC1 triggers Broadcast Session Release) to establish shared NG-U resources with the 5GC2 with which the shared NG-U was not established, after shared NG-U path change, the gNB-CU-UP should keep the PDCP COUNT continuity after the shared NG-U change to avoid the above mentioned packet discard by the UE. Similarly, in case of RAN sharing with multiple cell-ID broadcast, if the gNB-DU1/2 decides (e.g., upon receiving the Broadcast Context Release Command from gNB-CU-CP1 in case the 5GC1 triggers Broadcast Session Release) to establish the shared F1-U tunnel for PLMN2 which was not established, the gNB-DU1/2 may send the PDCP SN of next packet to be delivered to the gNB-CU UP2 which should keep the PDCP SN continuity over the new F1-U tunnel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add related NOTEs on gNB-CU-UP and gNB-DU1/2 handling in case of RAN sharing with MOCN or with multiple cell-ID broadcast to keep the PDCP SN continuity. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it describes the handling and gNB-CU-UP and gNB-DU to avoid packet discard by the UE after shared NG-U (and shared F1-U) path change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case the network enables Broadcast resource efficiency for RAN sharing, in case of shared NG-U (or shared F1-U) path change, packet discard occurs at the UE (before T-reordering expires, up to 2750ms) due to PDCP COUNT outside of receiving window. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.1a, 8.15.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.15.1.1a	Broadcast MBS Session Setup for resource efficiency for MOCN

Figure 8.15.1.1a-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Broadcast MBS Session Setup with the support of resource efficiency for MOCN.

Figure 8.15.1.1a-1: Broadcast MBS Session Setup for resource efficiency for MOCN

1.	The 5GC1 triggers the Broadcast MBS Session Setup procedure for TMGI 1 as defined in section 8.15.1.1, and the Associated Session ID x is provided together with TMGI 1 in step 1, 2 and 5 of 8.15.1.1. The broadcast MBS media stream received from 5GC1 is provided to the UEs of PLMN1.

2.	The 5GC2 triggers the Broadcast MBS Session Setup procedure for TMGI 2 with the Associated Session ID x.

3.	Upon receiving the same Associated Session ID x, the gNB-CU-CP may determine to not establish shared NG-U tunnel for PLMN2 with 5GC2, in case of multicast transport, the gNB-CU-CP stores the multicast transport address received from the 5GC2.

4.	The gNB-CU-CP triggers the F1AP Broadcast Context Setup Request for TMGI 2 with the Associated Session ID x towards the gNB-DU.

5.	Upon receiving the same Associated Session ID x, the gNB-DU shall determine to not establish shared F1-U tunnel for PLMN2.

6.	The gNB-DU triggers the F1AP Broadcast Context Setup Response for TMGI 2 to the gNB-CU, and indicates that the shared F1-U tunnel is not established.

7.	The gNB-DU provides configuration of both TMGI 1 and TMGI 2 by means of MCCH. The broadcast MBS media stream received from 5GC1 is provided to both UEs of PLMN1 and UEs of PLMN2.

8.	The gNB-CU-CP sends NGAP Broadcast Session Setup Response towards 5GC2.

Afterwards, if the gNB-CU-CP decides (e.g., in case the 5GC1 triggers Broadcast Session Release) to establish shared NG-U resources with the 5GC2 with which the shared NG-U was not established:

-	in case of multicast transport, the gNB-CU-CP triggers BC Bearer Context Modification Request towards the gNB-CU-UP providing the stored multicast address received from the 5GC2. Then the gNB-CU-UP joins the NG-U multicast group of 5GC2 accordingly;

-	in case of unicast transport, the gNB-CU-CP triggers Broadcast Session Transport Request towards the core network providing the unicast transport address received from the gNB-CU-UP.

NOTE: It is expected that the gNB CU-UP supports PDCP SN continuity after establishment of shared NG-U resources with 5GC2.

#### 8.15.1.1b	Broadcast MBS Session Setup for resource efficiency for RAN sharing with multiple cell-ID broadcast

Figure 8.15.1.1b-1 illustrates an exemplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Broadcast MBS Session Setup with the support of resource efficiency for RAN sharing with multiple cell-ID broadcast.

The gNB-DU1/2 entity shown in Figure 8.15.1b-1 is a simplified representation of the gNB-DU1 of PLMN 1, the gNB DU2 of PLMN 2 and respective radio resources of the shared cell.

Figure 8.15.1.1b-1: Broadcast MBS Session Setup for RAN sharing with multiple cell-ID broadcast

1.	The 5GC1 triggers the Broadcast MBS Session Setup procedure towards gNB-CU-CP1 for TMGI 1 as defined in section 8.15.1.1, and the Associated Session ID x is provided together with TMGI 1 in step 1, 2 and 5 of 8.15.1.1. The broadcast MBS media stream received from 5GC1 is provided to the UEs of PLMN1 via the gNB-CU-UP1 and the gNB-DU1.

2.	The 5GC2 triggers the Broadcast MBS Session Setup procedure for TMGI 2 with the Associated Session ID x towards gNB-CU-CP2.

3/4.	The gNB-CU-CP2 sets up the broadcast bearer context with gNB-CU-UP2 including the Associated Session ID x. The gNB-CU UP uses the received information to correlate the MBS sessions and include the shared NG-U termination endpoint if applicable.

5.	The gNB-CU-CP2 triggers the F1AP Broadcast Context Setup Request for TMGI 2 with the Associated Session ID x towards the gNB-DU2.

6.	Upon receiving the same Associated Session ID x, the gNB-DU1/2 may determine to not establish shared F1-U tunnel for PLMN2.

7.	The gNB-DU2 triggers the F1AP Broadcast Context Setup Response for TMGI2 to the gNB-CU-CP2, and indicates that the shared F1-U tunnel is not established.

8.	The gNB-CU-CP2 may determine to not establish shared NG-U tunnel for PLMN2. In case of multicast transport, the gNB-CU-CP2 stores the NG-U multicast transport address received from 5GC2 in step 2. In case of unicast transport, the gNB-CU-CP2 stores the NG-U unicast transport address received from the gNB-CU-UP2 in step 4.

9/10.	The gNB-CU-CP2 triggers E1AP BC Bearer Context Modification procedure towards the gNB-CU-UP2, and indicates that the shared F1-U tunnel is not established.

11.	The gNB-DU1/2 provides configuration of both TMGI 1 and TMGI 2 by means of MCCH. The broadcast MBS media stream received from 5GC1 is provided to both UEs of PLMN1 and UEs of PLMN2.

12.	The gNB-CU-CP2 sends NGAP Broadcast Session Setup Response towards 5GC2.

Afterwards, if the gNB-DU1/2 decides (e.g., upon receiving the Broadcast Context Release Command from gNB-CU-CP1 in case the 5GC1 triggers Broadcast Session Release) to establish the shared F1-U tunnel for PLMN2 which was not established:

- 	the gNB-DU2 triggers Broadcast Transport Resource Request towards the gNB-CU-CP2 and providing the F1-U TNL at DU and the PDCP SN of next packet to be delivered;

-	and then, the gNB-CU-CP2 triggers the BC Bearer Context Modification Request towards the gNB-CU-UP2 providing the F1-U TNL at DU and the received PDCP SN of next packet to be delivered which gNB-CU UP may use to support PDCP SN continuity,

-	in case of multicast transport, the gNB-CU-CP2 also provides the stored NG-U multicast transport address received from 5GC2. Then the gNB-CU-UP2 joins the NG-U multicast group of 5GC2 accordingly;

-	in case of unicast transport, the gNB-CU-CP2 triggers the Broadcast Session Transport procedure towards the 5GC2 providing the stored NG-U unicast transport address received from gNB-CU-UP2.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 5	Annex B: CR for TS 38.473

3GPP TSG-RAN WG3 Meeting # 125bis	R3-24xxxx

# Hefei, China,  14 – 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS PDCP packets discard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of MBS RAN resource sharing with multiple cell ID, in case the F1-U tunnel with gNB-CU UP1 of PLMN1 fails or receiving a broadcast sesion release over PLMN1 the gNB-DU may need to setup a new F1-U transport resource towards PLMN2 gNB-CU UP2 and switch the delivery of data over the new F1-U tunnel. However if the first packets received over the new tunnel have PDCP SN outside the current PDCP receiving window of the UE, all these packets will be discarded by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The last PDCP SN delivered to the UE is included in the broadcast transport resource request message to enable the CU to configure the PDCP SN of the first packets delivered over the new F1-U tunnel to ensure PDCP SN continuity. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR has an impact on the MBS Broadcast function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Many packets discarded by the UE and lost in case the F1-U tunnel fails or a broadcast sesion release from current PLMN is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.14.2, 9.2.13.9, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.14.14.2	Successful Operation

Figure 8.14.14.2-1: Broadcast Transport Resource Request procedure. Successful operation

The gNB-DU initiates the procedure by sending the BROADCAST TRANSPORT RESOURCE REQUEST message to the gNB-CU.

If the MRB Progress Information IE is received in the BROADCAST TRANSPORT RESOURCE REQUEST message, the gNB-CU shall, if supported, use it when configuring the requested F1-U transport bearer.

Interaction with the Broadcast Context Modification procedure:

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.13.9	BROADCAST TRANSPORT RESOURCE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to establish the F1-U resources for the broadcast Session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Transport Request List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Transport Request Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| >>MRB Progress Information | O |  | 9.3.2.12 | PDCP SN of last delivered packet to the UE. | YES | ignore |
| F1-U Path Failure | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-MRB-ProgressInformation,

maxNRARFCN,

maxnoofErrors,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

BurstArrivalTimeWindow-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-MRBs-Transport-Request-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { {Broadcast-MRBs-Transport-Request-Item-ExtIEs} } OPTIONAL,

...

}

Broadcast-MRBs-Transport-Request-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MRB-ProgressInformation		CRITICALITY ignore	EXTENSION MRB-ProgressInformation		PRESENCE optional	},

...

}

#### -- C

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-MRB-ProgressInformation							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>