3GPP TSG-RAN3 Meeting #125bis	R3-245238

# Hefei, China, 14th – 18th Oct 2024

Agenda Item:	8.1

Source:	CATT

Title:	Discussion support L3 based LTM

Document for:	Decision

- Introduction

In R3-245008, RAN2 inform RAN3 about supported LTM based on L3 measurement, and request RAN3 corresponding enhancement in F1AP. In this paper, we give analyse about the received LS and the potential impact on RAN3.

R3-245008 1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

2. Discussion

Track the history of the LS, the intention of the enhancement is from the perspective of UE capability, decoupling UE L1 measurement capability and LTM capability, and enable UE without L1 measurement capability to execute LTM. So, this enhancement is only an enhancement for special UE supporting LTM, and RAN3 shall try to support this function with minimize standard impact and try not impact legacy L3 HO.

Observation 1: The intention of support L3 measurement based LTM is for enable UE without L1 measurement capability to execute LTM.

Proposal 1: Try to support L3 measurement based LTM with minimize standard impact and not impact legacy L3 HO.

2.2 Support L3 measurement based LTM

There are two possible ways to realize DU triggered LTM execution by L3 measurement:

- Alt 1: CU sends L3 measurement report to the DU, DU decides to trigger LTM execution according to L3 measurement

- Alt 2: CU sends necessary information to DU, and DU trigger the LTM execution

For the alt1, it required DU able to interpret L3 measurement result, and decided to trigger LTM execution based on the L3 measurement. But from our understanding, it is not necessary as CU and DU shall have the same interpretation for the same L3 measurement result. Otherwise, it may have heavy impact on L3 HO. For example, if the DU is more sensitive about the L3 measurement, the LTM HO may occurred earlier than L3 HO. As L3 HO has worked for many years, and the optimization mechanism is already sufficient, we don’t think DU can make better HO decision than CU based on the same L3 measurements.

For the alt2, it request CU to sending necessary information to DU, based on the assistance information, DU can send MAC CE to the UE to trigger LTM. The necessary information may include the configuration ID of the target cell and the corresponding TCI State ID.

Proposal 2: It is propose to support Alt 2 for support L3 based LTM, i.e., CU send necessary information to DU for combine MAC CE triggering LTM.

Proposal 3：Agree to introduce TCI state and target configuration ID from CU to source DU for triggering LTM, other necessary UE can be further discussed.

In the legacy L3 HO, inter-CU HO is realized by CU sending UE CONTEXT MODIFICATION REQUEST message to the source DU. Considering the message designation for transferring the assistance information, we think UE CONTEXT MODIFICATION REQUEST still can be the suitable one, which can be reused in both intra-DU and inter-DU LTM.

Proposal 4: Reuse UE CONTEXT MODIFICATION message to transfer identified information from CU to DU.

Proposal 5: Discuss and agree the CR[1] for F1AP.

2.3 impact on CSI-resource

In Rel-18 discussion, we have a tricky IEs, CSI-RESOURCE, which leads to complex interaction in RAN3, i.e., how to collect CSI-RESOURCE and update it due to cell rejection. What’s more, it request candidate to reserved CSI-REPORT resource. In case of UE not support L1 measurement, CSI-RESOURCE is not needed and the corresponding inter-action can be avoided.

Observation 2: For the UE not support L1 measurement, CSI-RESOURCE and CSI-REPORT is not necessary to provide by the candidate node.

Proposal 6: Clarification the usage of CSI-RESOURCE in stage 2, discuss the CR in [2].

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: The intention of support L3 measurement based LTM is for enable UE without L1 measurement capability to execute LTM.

Proposal 1: Try to support L3 measurement based LTM with minimize standard impact and not impact legacy L3 HO.

Proposal 2: It is propose to support Alt 2 for support L3 based LTM, i.e., CU send necessary information to DU for combine MAC CE triggering LTM.

Proposal 3：Agree to introduce TCI state and target configuration ID from CU to source DU for triggering LTM, other necessary UE can be further discussed.

Proposal 4: Reuse UE CONTEXT MODIFICATION message to transfer identified information from CU to DU.

Proposal 5: Discuss and agree the CR[1] for F1AP.

Observation 2: For the UE not support L1 measurement, CSI-RESOURCE and CSI-REPORT is not necessary to provide by the candidate node.

Proposal 6: Clarification the usage of CSI-RESOURCE in stage 2, discuss the CR in [2].

4. Reference

[1] R3-245238	CR for support L3 based LTM(stage 3)

[2] R3-245239	CR for support L3 based LTM(stage 2)