3GPP TSG-RAN3 Meeting #125bis	R3-245241

# Hefei, China, 14 – 18 Oct 2024

Agenda Item:	9.1

Source:	CATT

Title:	Discussion on UE Location Information for NB-IoT NTN with draft LS

Document for:	Agreement

1. Introduction

RAN2 thanks SA2 for the update on the NAS-based coarse UE location reporting procedure. SA2 question: SA2 asks RAN2 and RAN3 whether they think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to MME For the above question, from RAN2 perspective, even if not essential it may be useful for the eNB if the MME further signals the coarse location information received from the UE in NAS back to eNB.

In this contribution, we will analyse the issue, and provide our proposals accordingly.

2. Discussion

During a long time discussion, Inmarsat clarified 3 cases of interest of providing coarse location from to the MME to the eNB.

- Intra-beam carrier allocation

- Intra-beam load balancing

- Inter-beam load balancing

Based on those case, RAN3 identify the coarse location information may be useful at eNB side for the purpose of load balance. However, for whether the eNB shall resend the updated ULI back to the MME, RAN3 not identify reasonable case. So, we propose only support MME sends the coarse location back to the eNB, and how to use it is due to eNB implementation.

Proposal 1: Only support MME sends the coarse location back to the eNB, and how to use it is due to eNB implementation

For reducing the signalling overload, it is request to designing suitable principle for sending the location information from MME to gNB, e.g., periodicity, one shot at the initial access, or event trigger. However, as this is MME’s action and have impact on the NG interface, we think is better to let OAM define the principle for location reporting, and sending an LS for SA5 about this requirements.

Proposal 2: Enable OAM to design suitable rules for send the location information from MME to eNB, and send LS to SA5 about this requirement.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Only support MME sends the coarse location back to the eNB, and how to use it is due to eNB implementation

Proposal 2: To avoiding the signalling overload, sending LS to SA5 for designing the principle of sending coarse location to eNB.

4. Reference

[1]	R3-241523, Reply LS on UE Location Information for NB-IoT NTN

[2]	R3-243009, Reply LS on UE Location Information for NB-IoT NTN

5. Draft LS

Title:	[DRAFT] LS on UE Location Information for NB-IoT NTN

Response to:

Release:	Rel-18

Work Item:	IoT_NTN_enh

Source:	CATT [to become RAN3]

To:	SA5,

Cc:

Contact Person:	Jiaying Sun

sunjiaying@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB. To avoiding signalling overload, RAN3 think it is necessary to define the principle for location reporting by OAM.

2. Actions:

To SA2 group:

ACTION: 	RAN3 asks SA5 to design suitable report principle for sending coarse location information received from the UE in NAS back to eNB.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #126	18-22 Nov 2024			Orlando, US