3GPP TSG-RAN3 Meeting #125bis	R3-245244

# Hefei, China, 14th – 18th Oct 2024

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

2. Discussion

2.1 Single vs. Parallel messages

Based on previous discussion, two options are still on table,

- a single Handover preparation that allows LTM preparation of multiple cells, or

- a single Handover preparation that allows LTM preparation of (only) one cell

Companies concern about the UE association, latency, and signalling load for above the two options.

For the UE association perspective, there is no difference between option A and option B. In TS 38.423, it is state that for a certain UE among one protocol peer, only one XnAP procedure is used unless explicitly indication. Thus, no matter option A and option B, source gNB will assign the same Source NG-RAN node UE XnAP ID IE, and candidate/target cell shall assign the same Target NG-RAN node UE XnAP ID IE.

## 5.2 Parallel transactions Unless explicitly indicated in the procedure specification, at any instance in time one protocol peer shall have a maximum of one ongoing XnAP procedure related to a certain UE.

Observation 1: For a certain UE among one protocol peer, only one XnAP procedure is used unless explicitly indication.

Considering about the latency, source/target node treat the request message in a parallel way, even the request is sending one by one, the target node can treat them once it received, instead of waiting feedback from the correspond Node. What’s more, the latency essential in LTM is in the execution phase, candidate cells preparation is by pre-configured basis, which has no impact on the delay requirement of LTM. For this parallel request, companies may concern about the signaling load is too high, from our perspective, candidate cell preparation always limited by the maximum of candidate cells and it’s still limited to 8 at defined in RAN2, the signaling load is still acceptable.

Proposal 1: Source gNB requests candidate cell in a parallel way, e.g., B)	a single Handover preparation that allows LTM preparation of (only) one cell.

Considering the existence IEs in HANDOVER REQUEST message, the Target Cell Global ID IE can be reused for indicate LTM candidate cell. Compared with initial LTM request in F1AP, seems we not need to introduce an explicated indicator as we may have some LTM-dedicated IE, e.g., Request for RACH configuration, which will always appear in the initial request and indicate this request is for LTM procedure.

Proposal 2: Reuse the Target Cell Global ID IE to request LTM candidate cell.

Proposal 3: Further consider whether need to introduce an explicit indicator for initial LTM candidate cell request.

2.2 Early sync

For the early sync information request, for avoiding early sync confliction, candidate DU shall provide RACH configuration in per source DU in Rel-18. Considering the same logic, in inter-CU case, besides DU ID, CU ID should also be transferred in early sync information request. However, as a gNB can not explore its own architecture to neighbour gNB, it is reasonable to let candidate gNB provides the early sync resource per source gNB, and source gNB-CU further split the resource into per-gNB-DU granularity, if needed.

Proposal 4: Candidate gNB provide RACH configuration per source gNB per candidate cell.

2.3 L1 measurement and report configuration

In Rel-18 LTM, L1 measurement and report is only based on SSB. CU obtains partial SSB information (without SSB Transmit power) by F1 SETUP procedure and obtains complete SSB information via UE CONTEXT SETUP RESPONSE message. SSB also be used in Rel-19. Similar, if the CU able to get partial SSB information via Xn SETUP REQUEST message, CU can construct CSI-RESOURCE and sending to other candidate cells.

Proposal 5: In Rel-19, s-gNB may construct CSI-RESOURCE in advance by SSB information obtained in Xn SETUP procedure.

Except SSB, Rel-19 LTM also agreed to support CSI-RS at least for L1-RSRP, accordingly, RAN3 shall also design the signalling for CSI-RS. Considering CSI-RS is a UE-specific RS, it is not possible for CU to obtain it in advance anymore.

Observation 2: CSI-RS is a UE-specific RS, the CSI-resource including CSI-RS is not able to obtain at Xn setup phase.

Thus, source node shall request the candidate node to provide CSI-RS for specified UE in the first round candidate cell request, e.g., HO REQUEST message for XnAP. And the candidate node shall contain the CSI-RS of the requested cell by HO REQUEST ACK message.

Proposal 6: The source CU request candidate gNB to provide CSI-RS for specified UE by HO REQUEST message, and candidate CU shall contain the CSI-RS of the requested cell by HO REQUEST ACK.

2.4 TA validity

TA validity is discussed in Rel.18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

In Rel.18, though not specific explicitly, one possible way to maintain TA validity is to start a timer at source side, the range of the timer usually as the same level with TAT due to they are for the same purpose. As defined in TS38.331, TAT is defined as ENUMERATED and the minimum value is 500ms. Usually, latency between the Xn interfaces can be seen as 10ms. It is easy to see the latency of Xn interface is much less than the TAT, i.e., TA validity time. Thus, the validity of TA is still possible to be decided at source DU, and let it to implementation as REL.18.

Proposal 7: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validity.

To avoid unnecessary early sync, in Rel-18, we enable source to transfer the obtained TA values in Cell Switch Notification message. In intra-DU scenario, the movement range of UE is limited, TA still meaningful in target cell in a high possibility. However, extend to inter-CU case, UE movement ranger lager than before, it is hardly to say TA obtained at source is still workable in target side, no matter in source or in target. Using an unsuitable TA may lead to HO failure, the drawback of transferring TA among different CU outweigh the potential advantages.

Proposal 8: Due to TA is relative with UE location, transferring TA values among different gNB is not support in inter-CU LTM.

2.5 Synchronization among candidate cells

To support subsequent inter-CU LTM, at least the below information should below information should be synchronize among candidate cells.

- TCI state information list

- Configuration ID list

- Early sync information

- The data forwarding information

This procedure can be done by the second round interaction between source CU and candidate CUs, also can be done the after UE switch to the new DU. Whatever, this information synchronize should be finished no later than the UE access to the new serving cells.

Proposal 9: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

2.6 Path switch

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC. And usually, it should be finished as soon as UE access to the target gNB. In LTM, when UE access to the candidate DU, the candidate DU shall send a access success message to the CU among F1AP, which means UE had access to the target gNB. It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 10: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

2.7 LTM Cancel

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

For the open issue about candidate cell initial cancellation, to make the procedure clear, it is better  to introduce a new message. we prefer to have a more generally name for this message, that is, not only limited to LTM, to avoid the same discussion will happen in further discussion for candidate node triggered cancellation.

Proposal 11: Introduce a new message for candidate cell initial cancel for LTM, the name of the message shall be generally name instead of only limited to LTM.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: For a certain UE among one protocol peer, only one XnAP procedure is used unless explicitly indication.

Proposal 1: Source gNB requests candidate cell in a parallel way, e.g., B)	a single Handover preparation that allows LTM preparation of (only) one cell.

Proposal 2: Reuse the Target Cell Global ID IE to request LTM candidate cell.

Proposal 3: Further consider whether need to introduce an explicit indicator for initial LTM candidate cell request.

Proposal 4: Candidate gNB provide RACH configuration per source gNB per candidate cell.

Proposal 5: In Rel-19, s-gNB may construct CSI-RESOURCE in advance by SSB information obtained in Xn SETUP procedure.

Observation 2: CSI-RS is a UE-specific RS, the CSI-resource including CSI-RS is not able to obtain at Xn setup phase.

Proposal 6: The source CU request candidate gNB to provide CSI-RS for specified UE by HO REQUEST message, and candidate CU shall contain the CSI-RS of the requested cell by HO REQUEST ACK.

Proposal 7: TA validity assurance is up to network implementation in inter-CU LTM, Source DU is in charge of checking the TA validit

Proposal 8: Due to TA is relative with UE location, transferring TA values among different gNB is not support in inter-CU LTM.

Proposal 9: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

Proposal 10: It is propose target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 11: Introduce a new message for candidate cell initial cancel for LTM, the name of the message shall be generally name instead of only limited to LTM.

4. Reference

[1]	 R3-244272 Discussion for Inter-CU LTM