3GPP TSG-RAN3 Meeting #125bis	R3-245245

# Hefei, China, 14th – 18th Oct 2024

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM in DC

Document for:	Decision

- Introduction

Based on the previous meeting, RAN2 had made some conclusion about DC scenario, and extracted as below:

RAN2#126

An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

RAN2#127

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

We start RAN3 discussion based on the above agreements.

2. Discussion

2.1 Simultaneous MCG LTM and SCG LTM Configuration

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

In the previous discussion, it is confirm that for inter-CU LTM is not configured in both MCG and SCG at the same time. The first issue is whether the MCG LTM and SCG LTM have different priority. There are three possible options:

- Option 1: MCG LTM take the high priority

- Option 2: SCG LTM take the high priority

- Option 3: Flexible priority, the LTM configuration triggered first take the high priority

For the option 1, MCG LTM have a high priority, thus, once the MN decided to trigger inter-CU MCG LTM, the MN shall inform UE and related SN to release the prepared configuration for inter-CU SCG LTM.

For the option 2, SCG LTM have a high priority, thus, once source SN decided to trigger inter-CU LTM(the only case confirmed by RAN2), it shall inform the MN, the MN shall release the prepared configuration for inter-CU MCG LTM.

For the option 3, it pend on which configuration is triggered earlier. If Inter-CU SCG LTM triggered earlier than inter-CU MCG LTM (source SN send corresponding request to MN), the Inter-CU SCG LTM has higher priority than Inter-CU MCG LTM. If Inter-CU MCG triggered earlier than inter-CU SCG LTM (source MN send corresponding request to candidate MN), Inter-CU MCG LTM has higher priority than Inter-CU SCG LTM.

We slightly prefer option 3. If the MN received an inter-CU SCG LTM request before it trigger inter-CU MCG LTM, the MN shall not trigger inter-CU MCG as it already know the SCG LTM is prepared. If the MN received an inter-CU SCG LTM request after it trigger inter-CU MCG LTM, the MN shall response a reject message with a suitable cause.

Proposal 1: Consider the priority of MCG LTM and SCG LTM, slight prefer option3, e.g., Flexible priority, the LTM configuration triggered first take the high priority.

Inter-CU SCG LTM triggered earlier than inter-CU MCG LTM (source SN send corresponding request to MN), Inter-CU SCG LTM has higher priority than Inter-CU MCG LTM.

Inter-CU MCG triggered earlier than inter-CU SCG LTM (source MN send corresponding request to candidate MN), Inter-CU MCG LTM has higher priority than Inter-CU SCG LTM.

Proposal 2: Using a suitable cause value for MN informing SN inter-CU MCG LTM has already triggered, MCG LTM triggered may be reused.

2.2 Inter-CU SCG LTM

For the inter-CU SCG LTM, RAN2 had down selection the scope and only focus on SN initiate SCG LTM. The source SN shall trigger the request of LTM candidate cells to MN, and then the MN request candidate SN to allocate resource for the UE.

The SN CHANGE REQUIRED message can be reused for request LTM candidate cell(s) form source SN to MN. And the SN ADDITION REQUEST message can be reused for request LTM candidate cell(s) from MN to candidate SN.

Proposal 3: Enhance SN CHANGE REQUIRED message for SN initiate SCG LTM candidate cell preparation from source SN to MN.

Proposal 4: Enhance SN ADDITION REQUEST for SN initiate SCG LTM candidate cell preparation from MN to candidate SN.

There are two possible ways for requesting candidate cell preparation:

- Single message: Similar with S-CPAC, source SN sends recommended candidate Pscells and the upper limit for the number of PSCells that can be prepared by each candidate SN to the MN. MN requests each candidate SN(s) to allocate resources for UE, and the candidate SN decides the list of PSCell(s) to prepared.

- Parallel messages: Similar with LTM, source SN sends the request candidate PScell to the MN, and MN requests candidate SN to allocate resources for UE, candidate SN decided whether accept or reject the request.

The main different is whether enable source request multiple candidate cells by one message, similar with what we discussed in SA inter-CU LTM. But we think this discussion is not fully relay on the conclusion on SA inter-CU scenario. For the DC scenario, it is possible to introduce more flexible way.

Proposal 5：Discussion how to request candidate cell in DC scenario, e.g., Single vs. Parallel messages, it is propose to decoupling this discussion with SA scenario.

2.3 Inter-CU SCG LTM

For Inter-MN PCell change, the issue need to be discussed handle of SCG.e.g,	 SCG release ot SCG kept.

For our understanding, if the candidate MN cannot keep the previous SCG, it shall not configuration any other SCGs, i.e., the previous SCG shall be released.

The similar procedure is discussed at CHO with SCG.

1. The source MN starts the conditional handover procedure by initiating the X2 Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN may include the (source) SN UE X2AP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message. NOTE 3: In case of the CHO with/without SN change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1. 2. If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SgNB Addition Request message to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SgNB Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SgNB Addition Request message to the candidate SN including neither the SN UE X2AP ID nor the UE context in the source SN that was established by the source MN.

We suggest taking this procedure as baseline for the discussion of LTM candidate SCG kept/release. When considering inter-CU MCG LTM, the source MN can indicate the SCG information to candidate MN, and it depend on the candidate MN to decide whether kept the previous SCG and send feedback the source MN.

Proposal 6: Take SCG kept procedure in CHO with SCG as baseline for the discussion of LTM candidate SCG kept/release.

Proposal 7: It is propose that the candidate MN decide whether kept the current SCG and send feedback the source MN

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Consider the priority of MCG LTM and SCG LTM, slight prefer option3, e.g., Flexible priority, the LTM configuration triggered first take the high priority.

Inter-CU SCG LTM triggered earlier than inter-CU MCG LTM (source SN send corresponding request to MN), Inter-CU SCG LTM has higher priority than Inter-CU MCG LTM.

Inter-CU MCG triggered earlier than inter-CU SCG LTM (source MN send corresponding request to candidate MN), Inter-CU MCG LTM has higher priority than Inter-CU SCG LTM.

Proposal 2: Using a suitable cause value for MN informing SN inter-CU MCG LTM has already triggered, MCG LTM triggered may be reused.

Proposal 3: Enhance SN CHANGE REQUIRED message for SN initiate SCG LTM candidate cell preparation from source SN to MN.

Proposal 4: Enhance SN ADDITION REQUEST for SN initiate SCG LTM candidate cell preparation from MN to candidate S

Proposal 5：Discussion how to request candidate cell in DC scenario, e.g., Single vs. Parallel messages, it is propose to decoupling this discussion with SA scenario.

Proposal 6: Take SCG kept procedure in CHO with SCG as baseline for the discussion of LTM candidate SCG kept/release.

Proposal 7: It is propose that the candidate MN decide whether kept the current SCG and send feedback the source MN

4. Reference

[1]	RP-240299 Revised WID: NR mobility enhancements Phase 4