3GPP TSG-RAN WG3 Meeting #125bis                                                             R3-245249

Hefei, China, October 14 - 18, 2024

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SIB1 for NES enhancements

Document for:		Discussion

# 1	Introduction

In the last TSG RAN meeting, the WID for enhancements of network energy savings for NR was updated [1]. The updated WID includes the following objectives for OD SIB1:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN3 agreed in the last meetings on OD SIB1 [2]:

Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

This contribution aims to make progress for the Case-2 OD-SIB1 scenario.

# 2	Discussion

## 2.1	UL WUS configuration transfer via F1

RAN3 agreed that the UL WUS configuration should be sent from NES-cell DU to NES-cell CU via F1. The question arises whether an existing procedure can be used for this purpose, or whether a new procedure should be introduced.

Since the UL WUS configuration is part of the NES-cell DU configuration, it makes sense to have the NES-cell DU include this configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message. The NES-cell CU confirms the reception of these messages via the corresponding legacy F1 SETUP RESPONSE and F1 gNB-DU CONFIGURATION UPDATE ACK messages.

Using a new procedure to convey the UL WUS configuration would not add any benefit. In fact, using a separate procedure from F1 SETUP would imply that OD SIB1 operation could be “turned on” at a later point in time after F1 SETUP, and/or that it could also be potentially “turned off”. In the last meeting, RAN3 discussed the potential support for the switching between OD SIB1 operation mode and legacy broadcast SIB1 mode, and RAN3 decided to not further pursue such dynamic switching. In absence of such switching, OD SIB1 operation needs to be considered as a semi-static feature of the NES-cell DU. This implies that the UL WUS configuration should already be disclosed during F1 SETUP.

Proposal 1-1: Since OD SIB1 represents a semi-static (and not dynamic) feature of the NES-cell DU, the NES-cell DU should include the UL WUS configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message.

The NES-cell DU relies on the NES-cell CU to pass the UL WUS configuration to Cell-A. The NES-cell CU must therefore be able to interpret the meaning of the IE containing this message. In case the NES-cell CU is not able to interpret and/or pass on this configuration, the NES-cell CU should reject the message with an appropriate cause value.

Proposal 1-2: Since the NES-cell DU relies on the NES-cell CU’s ability to pass the UL WUS configuration to Cell-A, the NES-cell CU should reject the message containing the UL WUS configuration in case it cannot interpret the UL WUS configuration IE.

In case the NES-cell CU cannot pass the UL WUS configuration to Cell-A, or in case Cell-A rejects the OTA transmission of the UL WUS configuration, the NES Cell CU should not activate (or should deactivate) the affected NES-cells.

Proposal 1-3: In case the NES-cell CU cannot pass the UL WUS configuration to Cell A or Cell A rejects the OTA transmission of the UL WUS configuration, the NES-cell CU should not activate (or should deactivate) the affected NES-cells.

The NES-cell DU may support multiple cells, where some cells apply OD-SIB1 while others apply legacy broadcast SIB1. The UL WUS configuration could further differ among the cells supporting OD-SIB1. For these reasons, the UL WUS configuration should be reported with served-cell granularity.

Proposal 1-4: Since the UL WUS configuration may be different for each cell, it should be signalled to the NES-cell CU with served-cell granularity. Optimizations are not precluded.

## 2.2	UL WUS configuration transfer via Xn

RAN3 agreed that the UL WUS configuration should be sent from the NES-cell CU to Cell-A via Xn. RAN3 further agreed that upon reception, Cell-A should provide feedback on whether it intends to broadcast the UL WUS configuration received.

For Xn, the same question arises as for F1 on whether an existing procedure can be used for this purpose, or whether a new procedure needs to be introduced. However, for Xn the situation is different for than for F1. In case of F1, the sending NES-cell DU solely informs the NES-cell CU about its OD-SIB1 functionality and the associated UL WUS configuration. In case of Xn, the NES-cell CU does not only report an UL WUS configuration, but it also files the request for an OTA transmission of this UL WUS configuration. Further, the NES-cell CU may send this request for multiple NES-cell DUs, and in this case, Cell-A may be inclined to accept the request only for a subset of UL WUS configurations received, e.g., for those whose NES-cells have coverage overlap with the Cell-A. This implies that Cell-A must have means to provide differentiated feedback on the request with respect to which UL WUS configuration it is inclined to transmit OTA vs. which ones it is not.

Observation 1: The sending of an UL WUS configuration to Cell-A is not just an information report, but it contains an implicit request to Cell-A for OTA transmission.

While the legacy Xn SETUP REQUEST and NG-RAN-node CONFIGURATION UPDATE messages would allow including the NES-cells’ UL WUS configurations, the intend of these messages primarily serves for information purposes rather than for requesting specific actions, such as OTA transmissions.

Observation 2: The legacy Xn SETUP REQUEST and NG-RAN-node CONFIGURATION UPDATE messages primarily serve to convey information rather than to request OTA transmissions.

Further, the legacy Xn SETUP RESPONSE and NG-RAN-node CONFIGURATION UPDATE ACK messages have not been designed do provide differentiated accept/reject feedback on cell-specific requests.

Observation 3: The legacy Xn SETUP RESPONSE and NG-RAN-node CONFIGURATION UPDATE ACK messages have not been designed to provide differentiated accept/reject feedback on cell-specific requests.

For these reasons, it is proposed to introduce a new class-1 Xn procedure, which is initiated by the NES-cell toward the A-cell to request transmission of UL WUS information. The reply message can include the list of UL-WUS configurations accepted vs. rejected.

Proposal 2-1: Based on the above observations, introduce a new class-1 Xn procedure with the following functionality:

- Request from NES-cell to Cell-A to broadcast a list of UL WUS configurations contained in the same message.

- Response from Cell-A to NES-Cell on whether each of these UL WUS Configurations will be OTA transmitted.

This new procedure should include for each UL WUS configuration the corresponding cell ID. This allows Cell-A to perform an educated analysis on whether OTA transmission is justified, e.g., based on overlap with Cell-A’s coverage overlap.

Proposal 2-2: The Xn request for OTA transmission of a UL WUS configuration to include the corresponding cell ID so that Cell-A can make an educated decision on whether to accept vs. reject the request, e.g., based on Cell-A’s coverage overlap.

# 3	Conclusion

This contribution aimed to make further progress for the Case-2 OD-SIB1 scenario. The following observations and proposals have been made:

UL WUS configuration transfer via F1

Proposal 1-1: Since OD SIB1 represents a semi-static (and not dynamic) feature of the NES-cell DU, the NES-cell DU should include the UL WUS configuration in the F1 SETUP REQUEST message and send updates via the F1 gNB-DU CONFIGURATION UPDATE message.

Proposal 1-2: Since the NES-cell DU relies on the NES-cell CU’s ability to pass the UL WUS configuration to Cell-A, the NES-cell CU should reject the message containing the UL WUS configuration in case it cannot interpret the UL WUS configuration IE.

Proposal 1-3: In case the NES-cell CU cannot pass the UL WUS configuration to Cell A or Cell A rejects the OTA transmission of the UL WUS configuration, the NES-cell CU should not activate (or should deactivate) the affected NES-cells.

Proposal 1-4: Since the UL WUS configuration may be different for each cell, it should be signalled to the NES-cell CU with served-cell granularity. Optimizations are not precluded.

UL WUS configuration transfer via Xn

Observation 1: The sending of an UL WUS configuration to Cell-A is not just an information report, but it contains an implicit request to Cell-A for OTA transmission.

Observation 2: The legacy Xn SETUP REQUEST and NG-RAN-node CONFIGURATION UPDATE messages primarily serve to convey information rather than to request OTA transmissions.

Observation 3: The legacy Xn SETUP RESPONSE and NG-RAN-node CONFIGURATION UPDATE ACK messages have not been designed to provide differentiated accept/reject feedback on cell-specific requests.

Proposal 2-1: Based on the above observations, introduce a new class-1 Xn procedure with the following functionality:

- Request from NES-cell to Cell-A to broadcast a list of UL WUS configurations contained in the same message.

- Response from Cell-A to NES-cell on whether each of these UL WUS Configurations will be OTA transmitted.

Proposal 2-2: The Xn request for OTA transmission of a UL WUS configuration to include the corresponding cell ID so that Cell-A can make an educated decision on whether to accept vs. reject the request, e.g., based on Cell-A’s coverage overlap.

# Reference

[1]  RP-242354, TSG RAN meeting #105, Melbourne, Australia, September 2024

[1]  Chair notes, RAN WG3 meeting #125, Maastricht, The Netherlands, August 2024