3GPP TSG-RAN3 Meeting #125bis	R3-245250

Hefei, CN, Oct 14th – Oct 18th 2024

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE

Document for:	Discussion/Decision

# Background

WID of Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (RP-240801) has the below objective for RAN3:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In this contribution, we discuss the details on supporting LP-WUS subgrouping to indicate paging via LP-WUS for a UE in RRC_IDLE/RRC_INACTIVE states.

# Discussion

## 2.1 LP-WUS subgrouping methods

Below are some notes from RAN2#126 on LP-WUS subgrouping [1]

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

Below is an agreement and notes from RAN3#125 on LP-WUS subgrouping [2]

For UE-ID-based subgrouping, wait until RAN1/2 makes further progress

[Agreement] New IEs should be introduced to support LP-WUS on NG, Xn and F1.

These notes and agreements indicate that subgrouping is supported for LP-WUS, specifically CN-assigned subgrouping, and is independent of PEI subgrouping

Observation 1: RAN2 and RAN3 agreed to support LP-WUS CN-assigned subgrouping independent of PEI.

Observation 2: RAN3 agreed to introduce new IEs for LP-WUS on NG, Xn and F1.

Similar to PEI CN-assigned subgrouping, support of LP-WUS CN-assigned subgrouping requires AMF to assign the CN-assigned subgrouping identifier and include the identifier in the messages sent to RAN on the NG interface. RAN uses the received information for supporting LP-WUS CN-assigned subgrouping to the UE. Thus, RAN3 should inform SA2 on the RAN3 agreement and that AMF is required to support LP-WUS CN-assigned subgrouping assistance information.

Proposal 1: RAN3 to liaise SA2 (cc: CT1) on RAN3’s agreement to support CN-assigned subgrouping for LP-WUS.

Proposal 2: Support receiving LP-WUS Assistance Information, including LP-WUS CN-assigned subgroup ID, in Paging message and Core Network Assistance Information for RRC INACTIVE UEs from AMF similar to PEI CN-assigned subgrouping.

LP-WUS assistance information to support CN-assigned subgrouping would be very similar to PEI and thus similar IEs have to be introduced over NG, Xn and F1. RAN3 should agree that for this purpose, the  PEI IEs be replicated for LP WUS in the relevant messages exchanged over NG, Xn and F1.

Proposal 3: Support new IEs similar to PEI IEs for LP-WUS on NG, Xn and F1 for supporting LP-WUS subgrouping

## 2.2 LP-WUS configuration

In RAN3#127, chair has noted that RAN3 should discuss whether the LP-SS and LP-WUS configuration, entry/exit condition is generated at the gNB-DU or the gNB-CU. Below are some notes from RAN2#126 [1] and RAN2#127 [3] on LP-SS/LP-WUS configuration parameters

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Do not support dedicated LP-WUS configuration

Noted

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Observation 3: RAN2 assumes to send LP-WUS configuration via cell-specific system information.

Observation 4: RAN2 assumes to use serving cell quality measured by MR/LR, i.e. (LP)RSRP or (LP)RSRQ, for entry/exit conditions determination of LR monitoring.

From RAN2 notes, it is clear that the LP-SS and LP-WUS configuration is included in cell specific System Information. Hence, it is suitable for gNB-DU to generate the LP-SS and LP-WUS configuration and send to gNB-CU.

Proposal 4: For RRC idle/inactive UEs, the LP-SS and LP-WUS configuration to be generated at the gNB-DU as the LP-SS and LP-WUS configuration information (e.g., bitmap, codepoint) has been agreed to be included in cell-specific system information.

In RAN1/2 study report [4], it is clarified that there are scenarios where LP-SS link performance is not better than legacy physical channels (e.g., PDCCH or PUSCH), and there could be partial LP-SS coverage in the cell. For the partial coverage case, UE may need to keep track of whether it moves in and out of LP-SS coverage in the cell, so that UE is able to fall back to monitoring paging using the main receiver when the coverage of LP-WUS is not sufficient.

RAN2 agreed that the UE that supports LP-WUS, monitors both LP-SS and SSBs of the serving cells to determine whether the LP entry/exit conditions are met. The UE should use MR-based thresholds like RSRP/RSRQ and LR-based thresholds like LP-RSRQ/LP-RSRP to determine whether it is in LP-SS/LP-WUS coverage or not. The UE keeps the MR in ultra-deep sleep power state if the UE is in the LP-SS coverage (i.e. entry condition is met) in the serving cell, and it receives page indications via LP-WUS. When the UE moves out of LP-SS coverage, i.e. exit condition is met, the UE will start up the MR to monitor the serving cell PEI and/or paging messages using MR. The LP-SS measurements are only used by the UE for switching between MR and LR in the serving cell and not for cell selection/reselection. Therefore, MR/LR entry/exit criteria are not affected by inter-DU relations, and consequently, these criteria can be determined by the gNB-DU rather than the gNB-CU.

Proposal 5: For RRC idle/inactive UEs, the entry/exit condition is generated by the gNB-DU since RAN2 agreed to use these conditions only for MR/LR switch within the serving cell but not for inter-cell reselection.

## 2.3 LP-WUS coexistence with eDRX

Observation 5: According to the RAN1/RAN2 TR [4] introduction section, LP-WUS is introduced to provide better power savings than long eDRX. Thus, UE will wake up MR only when paged and monitor per the long eDRX.

As per performance analysis in this TR, there seems to be no special handling for LP-WUS when eDRX is enabled. Further, enabling both, LP-WUS and eDRX, may provide better power savings. Thus, we believe that RAN3 should agree to support simultaneous operation of LP-WUS and (long)eDRX,and inform RAN2/RAN2 about this decision.

Proposal 6: From RAN3 perspective, simultaneous operation of eDRX and LP-WUS should be supported to provide better power savings for IOT devices.

# Summary

Below are the observations and proposals discussed in this document.

Observation 1: RAN2 and RAN3 agreed to support LP-WUS CN-assigned subgrouping independent of PEI.

Observation 2: RAN3 agreed to introduce new IEs for LP-WUS on NG, Xn and F1.

Observation 3: RAN2 assumes to send LP-WUS configuration via cell-specific system information.

Observation 4: RAN2 assumes to use serving cell quality measured by MR/LR, i.e. (LP)RSRP or (LP)RSRQ, for entry/exit conditions determination of LR monitoring.

Observation 5: According to the RAN1/RAN2 TR [4] introduction section, LP-WUS is introduced to provide better power savings than long eDRX. Thus, UE will wake up MR only when paged and monitor per the long eDRX.

Proposal 1: RAN3 to liaise SA2 (cc: CT1) on RAN3’s agreement to support CN-assigned subgrouping for LP-WUS.

Proposal 2: Support receiving LP-WUS Assistance Information, including LP-WUS CN-assigned subgroup ID, in Paging message and Core Network Assistance Information for RRC INACTIVE UEs from AMF similar to PEI CN-assigned subgrouping.

Proposal 3: Support new IEs similar to PEI IEs for LP-WUS on NG, Xn and F1 for supporting LP-WUS subgrouping

Proposal 4: For RRC idle/inactive UEs, the LP-SS and LP-WUS configuration to be generated at the gNB-DU as the LP-SS and LP-WUS configuration information (e.g., bitmap, codepoint) has been agreed to be included in cell-specific system information.

Proposal 5: For RRC idle/inactive UEs, the entry/exit condition is generated by the gNB-DU since RAN2 agreed to use these conditions only for MR/LR switch within the serving cell but not for inter-cell reselection.

Proposal 6: From RAN3 perspective, simultaneous operation of eDRX and LP-WUS should be supported to provide better power savings for IOT devices.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] (draft) RAN3 #124 Meeting Report, Maastricht, the Netherlands

[3] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

[4] TR 38.869 - Study on low-power wake-up signal and receiver for NR