3GPP TSG-RAN WG3 Meeting #125bis                                                             R3-245254

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on 5G Femto in stage-2

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In the last meeting, the study of additional topological enhancements for NR was finished, and the conclusion and recommendation for 5G Femto is produced [1].

The objectives[2] of the 5G Femto normative work are as follows:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, we will discuss the impact on stage-2 for 5G Femto.

2 Discussion

2.1 Architecture

According to the WID [2] in RAN#105 meeting, the NR Femto architecture with optional NR Femto GW for NG interface is to be supported. Option 1 and option 2 in the TR [1] match the objective. As option 1 is a special case of option 2, it is proposed option 2 of NR Femto architecture for NG interface in the TR should be captured in the architecture part of stage-2.

Besides the architecture of option 2 in the TR, following figure should also be captured in stage-2 spec:

Figure 1: NR Femto Architecture

Proposal 1: The architecture part of stage-2 spec captures option 2 of NR Femto architecture for NG interface in TR 38.799 and above figure.

2.2 Protocol stack

The protocol stack examples of NG Control plane and User plane with and without the NR Femto GW are as below:

Figure 2: Protocol stack example of NG User plane for NR Femto without NR Femto GW

Figure 3: Protocol stack example of NG User plane for NR Femto with NR Femto GW

Figure 4: Protocol stack example of NG Control plane for NR Femto without NR Femto GW

Figure 5: Protocol stack example of NG Control plane for NR Femto with NR Femto GW

Proposal 2: The protocol stack part of stage-2 spec uses the figures above.

2.3 Function

A NR Femto node hosts the same functions as a gNB, with the following additional specifics in case of connection to the NR Femto GW:

- A NR Femto node shall only connect to a single NR Femto GW at one time. The NR Femto node should not simultaneously connect to another NR Femto GW, or other AMF/UPF.

- The TAC and PLMN ID used by the NR Femto node shall also be supported by NR Femto GW.

- Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the 5G-S-TMSI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message.

- NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

If the NR Femto GW is deployed, we propose that the NR Femto GW hosts the following functions in stage-2:

- Relaying UE-associated NG application part messages between AMF serving the UE and the NR Femto node serving the UE, for example INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message, HANDOVER REQUEST ACKNOWLEDGE message, PATH SWITCH REQUEST message, PATH SWITCH REQUEST ACKNOWLEDGE message. But except the UE CONTEXT RELEASE REQUEST message, it received from the NR Femto node with an explicit GW Context Release Indication. In that case, the NR Femto GW terminates the UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by a NR Femto node attached to it.

- Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF, for example NG SETUP REQUEST message, PWS RESTART INDICATION message, PWS FAILURE INDICATION message, OVERLOAD START message, OVERLOAD STOP message.

Proposal 3: The function of NR Femto node and NR Femto GW of stage-2 spec captures the above functions.

2.4 Access control

In the TR [1], for access control, NR Femto reuses the existing CAG mechanism and no stage-3 impact is expected. The the description in the TR for the three access modes for NR Femto is generally ok, the text would need to be simplified. For example, following definition for 3 access modes for NR Femto can be captured in stage-2 spec:

| Cell | Definitions |
| --- | --- |
| Open access mode | The cell is accessible as a normal cell by UEs without access control of CAG. |
| Hybrid access mode | The cell is accessible as a NPN cell by UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell and as a normal cell by all other UEs. |
| Closed access mode | The cell is NPN-only and accessible by UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell. |

Proposal 4: Stage-2 spec capture above definition for three access modes of NR Femto.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: The architecture part of stage-2 spec captures option 2 of NR Femto architecture for NG interface in TR 38.799 and above figure.

Proposal 2: The protocol stack part of stage-2 spec uses the figures above.

Proposal 3: The function of NR Femto node and NR Femto GW of stage-2 spec captures the above functions.

Proposal 4: Stage-2 spec capture above definition for three access modes of NR Femto.

4 Reference

- R3-244849, 3GPP TR 38.799,  Study on additional topological enhancements for NR, v1.2.0 (2024-08)

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105